3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205744r08

Elbonia, August 17 – 26, 2022	(revision of S2-2205744)

Source:	Huawei, HiSilicon

Title:	Conclusion on the solutions of KI#4

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN/ Rel-18

1.	Discussion

In SA2#151e, several solutions has been proposed to address KI#4 for communication of PIN. This document is proposed to capture the conclusion for key issue 4. The details of relevant solutions in KI #4 can be referred to the proposed evaluation document.

It is proposed to use the following principles for normative work on communication of PIN.

- Solution #16 for QoS differentiation for different PINEs

- The mapping between a Uu QoS parameters (including 5QI, GFBR/MFBR, ARP) value and N3GPP QoS parameters is performed by PEGC.

- The N3GPP QoS parameters and mapping procedure performed by PEGC are not specified by 3GPP.

- How to enforce QoS based on the Non-3GPP QoS assistance information in the non-3GPP network is not specified by 3GPP.

- PDU session modifications can be performed either by the PEGC or by the AF.

- When the PEGC detects new traffic from a device in the PIN, it may decide to map the traffic to an existing QoS flow or to ask for a new QoS to transfer the traffic within the PDU session. The criteria for taking the decision can be based on URSP matching, if available, or UE specific. Note that this procedure is the same used when the application generating the traffic resides directly on the UE.

- The AF may ask, via NEF, for a modification of the QoS. The mechanism and criteria used by the AF to determine the need for a QoS modification are not defined by 3GPP,

NOTE: The AF may rely on PIN signalling between the PINE/PEGC/PEMC and the AF which transferred via UP transparently to the 5G system to determine the need for a QoS modification.

- The simultaneous presence of multiple PEGCs in a PIN is not supported in this release

2.	Text proposal

It is proposed to capture the following changes vs. TR 23.700-88:

# * * * * First change * * * *

# merge conclusion part of S2-2205744, S2-2206249, S2-2206484, S2-2206016, at the start of the meeting

8.x	Conclusion for Key Issue #4: Communication of PIN

The normative work is based on the following principles

2) 5G QoS parameters (including QoS characteristics5QI, GFBR/MFBR, ARP, Periodicity) may be sent to PEGC to assist the deriving of N3GPP QoS parameters.

a) How PEGC performs the deriving of N3GPP QoS parameters and mapping procedure is not specified by 3GPP.

b) How to enforce QoS based on the Non-3GPP QoS assistance information in the non-3GPP network is not specified by 3GPP.

3) PDU session modifications can be initiated either by the PEGC or by the SMF based on request received from the AF via the PCF.

a) When the PEGC detects new traffic from a device in the PIN, it may map the traffic to an existing PDU session or establish a new PDU session. The criteria for taking the decision can be based on URSP matching to traffic, if available, or UE specific.

NOTE 1:	The procedure is the same used when application generating the traffic resides directly on the UE.

b) The AF may request PCF, directly or via NEF, for a modification of the QoS. The mechanism and criteria used by the AF to determine the need for a QoS modification are outside 3GPP scope,

NOTE 2: 	The AF relies on PIN signalling between the PINE/PEGC/PEMC and the PIN AF, which is transferred via UP transparently to the 5G system, to determine the need for a QoS modification.

4) The procedure for supporting one PINE connected to multiple PEGCs in the same PIN and PINE to move between PEGCs is outside the 3GPP scope.

NOTE 3: 	Since the association between the PINE and PEGC is managed over UP by interaction with PIN AF, whether one or more PEGCs are associated with a PINE and PINE moving between PEGCs are not specified by SA2.

5)	PIN direct communication is not specified since outside 3GPP scope.

5a) PIN indirect communication via PEGC is managed within the PIN, which may be supported by 5GS.

6)	a PEGC may establish a Single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PIN and in this case, there is at least one PDU session per PIN. .

7) IPv6 Prefix Delegation may be applied for IP address allocation of PINEs connected to PEGC.

8)		AF may provide necessary information to 5GC for PIN communication.

9)	AF may provide necessary PIN specific parameters in URSP guidance for PDU Session selection by the PEGC.

NOTE 4: The specific information for PIN communication needs to be determined in conclusion of KI#6.

10)		UDR is enhanced to support the storage and retrieval of PIN related policy and QoS parameters.

12) 5GC may take into count the delay budget between PINE and PEGC to guarantee the end to end delay for PINE traffic.

13)	The 5G system support for anchoring PDU Sessions of PEGCs and PEMCs at same SMF based on a combination of DNN, S-NSSAI as well as based on the procedure described in clause 4.3.6.2, 4.3.6.3, and 4.3.6.4 of TS 23.502 [3] and clause 5.6.7 of TS 23,501 [2].

# * * * * End of changes * * * *