3GPP TSG-WG SA2 Meeting #152E	S2-2205769r01

# E-meeting, August 17 - 26, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3677 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing UDSF timer service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 7.2.13 describes Nudsf_UnstructuredDataManagement as the only UDSF service. However, TS 29.598 also defines a UDSF timer service, which is currently missing from stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 7.2.13: introduces the UDSF timer service. Updating description of UDSF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 description of the UDSF timer service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.13; 6.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR3515 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.2.13	UDSF Services

The following NF services are specified for UDSF:

Table 7.2.13-1: NF Services provided by UDSF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nudsf_UnstructuredDataManagement | Allows NF consumers to retrieve, create, update, and delete data stored in the UDSF. | 5.2.14.2 |
| Nudsf_Timer | Allows NF consumers to start, stop, update, and search timers in UDSF. NF consumers may be notified about timer expiry | 5.2.14.3 |

# * * * * second change * * * *

6.2.12	UDSF

The UDSF is an optional function that supports the following functionality:

-	Storage and retrieval of information as unstructured data by any NF;

-	Timer service to any NF

NOTE 1:	Structured data in this specification refers to data for which the structure is defined in 3GPP specifications. Unstructured data refers to data for which the structure is not defined in 3GPP specifications.

NOTE 2:	Deployments can choose to collocate UDSF with UDR.

# * * * * End of changes * * * *