3GPP TSG SA2 Meeting #152 (e-meeting) 	S2-2205790r02

17 – 26 August, 2022, Elbonia

# Source:	Lenovo, Broadcom, Interdigital

# Title:	New WID: URSP rule enhancements for advanced non-3GPP access selection (TEI18_URSP_n3A)

# Document for:	Approval

Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	URSP rule enhancements for advanced non-3GPP access selection

Acronym:	TEI18_URSP_n3A

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

## 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820018 |  |  |
| 900009 |  |  |

# 3	Justification

A UE may be provisioned with WLAN Selection Policy (WLANSP) from a PLMN that indicates to the UE how to select/reselect a WLAN access. Each WLANSP rule contains a prioritized list of selection criteria groups and each group identifies certain capabilities or properties that a WLAN access network must support in order to match this selection criteria group. A WLANSP rule may also contain validity conditions, such as a day of time and/or a location in which the rule is valid.

The UE constructs a prioritized list of the available WLANs by discovering the available WLANs and selects the WLAN access that meets the criteria of the higher priority WLANSP rule.

If the UE has one or more URSP rule for different application traffic indicating a non-3GPP access as preferred access then the UE will route both application traffic via the selected WLAN access.

It is useful to support an enhanced selection of WLAN access based on more advanced conditions. Examples are:

- Select a WLAN access network based on the traffic that should be offloaded to this WLAN access network.

  - Example: If a WLAN access network is needed to directly offload the traffic of App-1, then select a WLAN access network using a first set of selection criteria. However, if a WLAN access network is needed to directly offload the traffic of App-2, then select a WLAN access network using a second set of selection criteria.

- Select a WLAN access network based on the network slice to which connectivity should be obtained.

  - Example: If a WLAN access network is needed to establish connectivity to network slice X of PLMN-1, then select a WLAN access network using a first set of selection criteria. However, if a WLAN access network is needed to establish connectivity to network slice Y of PLMN-1, then select a WLAN access network using a second set of selection criteria.

- Select a WLAN access network based on the DNN to which connectivity should be obtained.

  - Example: If a WLAN access network is needed to establish connectivity to DNN-x, then select a WLAN access network using a first set of selection criteria. However, if a WLAN access network is needed to establish connectivity to DNN-y, then select a WLAN access network using a second set of selection criteria.

- Select a WLAN access network based on the requested Connection Capabilities.

  - Example: If a WLAN access network is needed to support IMS connectivity (Connection Capabilities = IMS), then select a WLAN access network using a first set of selection criteria. However, if a WLAN access network is needed to support Internet connectivity (Connection Capabilities = Internet), then select a WLAN access network using a second set of selection criteria.

# 4	Objective

The objective of this work item is to:

-	Enhance URSP rules allowing the network operator to define a policy where a matched URSP rule maps to a specific WLAN selection policy

-	Define UE behaviour when a UE applies a matched URSP rule that links to a specific WLAN selection policy

-	The solution must ensure that a UE is connected to a single WLAN access at a time. The UE shall not connect to multiple WLAN accesses simultaneously.

-	The solution must ensure that UE does not frequently change WLAN accesses, e.g., when multiple URSP rules refer to different WLAN selection policies.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 | Add description and procedures related to support enhanced URSP rules for WLAN selection. | SA#99 (Mar. 2023) |  |

# 6	Work item Rapporteur(s)

Dimitrios Karampatsis (Lenovo), dkarampatsis@lenovo.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Lenovo

Broadcom

InterDigital