SA WG2 Meeting #S2-152E	S2-2205798r01

17 - 26 Aug, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #4: Evaluation of Key Issue #4

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an evaluation of Key Issue #4.

Discussion

There are 19 solutions in TR 23.700-08 0.3.0 to address key issue #4.

This key issue aims at addressing the following aspects:

-	Investigate which type of information needs to be exchanged between hosting network and a localized services provider so that a UE can perform discovery, selection and connection of the hosting network and access the localized services provided via the hosting network.

NOTE 1:	The hosting network can also act as a service provider for localized services.

-	What is the provisioning mechanism and the information needed for the UE to discover, select and access suitable hosting network for localized services with possible validity conditions for accessing the hosting network offering the localized services, because of the nature of the localized service and hosting network (e.g. time and location constrains). This includes information enabling the UE to be aware of services that can be accessed via hosting NPN.

-	Discovery and selection procedures of hosting network and localized services provided via the hosting network for UE to obtain localized services. Both automatic and end user manual selection apply.

-	How a UE already registered in a network (PLMN or NPN) can discover a suitable hosting network and the localized services provided via the hosting network when the hosting network and/or localized services become available.

-	How to ensure the localized services are accessed by UE according to the conditions when and where the localized services are allowed to be accessed by the UE.

-	How the UE is provisioned with credentials (if required) to access the selected localised services provided via the hosting network.

-	Mechanisms to authorize UE to access the hosting network.

NOTE 2:	Security aspects (such as authentication of the UE and security aspects of provisioning) are addressed in SA WG3. The authentication architecture is addressed in both SA WG2 and SA WG3.

Proposal

Add the following evaluation to TR 23.700-08.

*** BEGIN CHANGES ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.4	Key Issue #4 Enabling UE to discover, select and access NPN as hosting network and receive localized services

7.4.1	General

The evaluation for KI #4 solutions is made for each component listed below:

-	The content of the localized service information that is used by UE to perform discovery/selection/access to the hosting NW for localized services.

-	From where and how UE obtains such information.

-	How the localized service information is used by UE.

7.4.2	Evaluation for the content of the localized service information

Construct the information on localized service basis (Sol #11, #12, #13, #15, #24, #27, #31, #32, #34).

-	The information includes an element to distinguish different localized service (e.g. identifier, name of the localized service), validity conditions (time/location), associated hosting networks' information (e.g. hosting network identifier, temporary credential or credential type for accessing hosting network, CAG IDs, QoS, cost, etc).

NOTE:	Credential provisioning mechanisms need to be evaluated by WG SA3.

-	The benefit of this principle is to provide UE/end user with relationship between localized services and hosting networks, so that it enables the possibility for UE/end user to make request on specific localized service and for network to inform UE what are the available localized services and over which hosting networks these localized services are provided.

Construct the information based on priority list for network selection (Sol #10, #14, #18, #25, #29, #30, #33, #36).

-	The information is a new priority list for network selection. Each entry on the list includes hosting network identifiers, validity conditions (time/location), etc.

-	This principle requires other solution components (e.g. Sol #12, #13, #24) to complement, since the information does not include reference to the localized services available over the different hosting networks included in the priority list and therefore it assumes that either UE or the network providing this information knows beforehand which localized service end user intends to access or what are the localized services supported by the hosting network on the priority list.

The localized service information in Sol #22 provided by hosting network to UE, is not used for selecting hosting networks, but rather for UE to know how to access a service in the hosting network which can be addressed by existing functionality, such as URSP rules.

7.4.3	Evaluation for from where and how UE obtains the localized service information

UE can obtain the localized service information from application server, serving network, home network, or hosting network.

UE fetches the information from an application server (Sol #14, #28, #31).

-	When a full set of new subscription/credential is provided to UE via application server, this principle has less system impact on 5GS. But on the other hand, home network subscription/credential is  utilized and the UE needs to handle co-existence of multiple subscriptions/credentials on device.

-	When the information includes a list of hosting networks and an indication that home network credential is to be used, there will be impact on network selection. Application server obtaining such information via network exposure is not necessary, since it is covered as part of the SLA between localized service provider and hosting network operator.

UE fetches the information from home network (Sol #10, #13, #14, #15, #18, #23, #24, #29, #30, #33, #36).

-	This principle assumes that home network is capable of managing the localized service information data.

-	The impact is about how home network is provisioned with the localized service information data, and how UE is provisioned by the home network.

-	Most of the solutions following this principle propose that the home network provides the localized service information to UE via SoR procedure in combination with the principle that the localized service information is constructed based on a priority list for (hosting) network selection as described in clause 7.4.2.

-	Some solutions assumed that the SoR-AF within the home network should be involved as the data source and the mechanism regarding how SoR-AF obtains such data should be out of 3GPP scope.

-	Since SoR procedure is mainly used for network discovery and selection, it is not suitable to be extended to provision UE with data more than required for the UE’s actual network discovery and selection.

-	Sol #24 proposes another variant for UE to fetch the information from home network, that is via home network UDR and new UE policy. UE policy has the potential to include necessary data of the localized service, and possibility to be extended in the future. The new UE policy can be delivered to UE by PCF in home network, or PCF in serving network using N24.

UE fetches the information from serving network (Sol #11, #12, #24, #27, #32).

-	This principle assumes serving network (either home network or another network which UE is accessing using home network credential) is capable of storing the localized service information data.

-	The impact is about how serving network is provisioned with the localized service information data, and how UE is provisioned by the serving network.

-	Some solutions following this principle proposes to use AMF/SMF in serving network to store the localized service information data. In this case, the provisioning to serving network is based on configuration which has less impact, but it requires the UE is served by the correct AMF/SMF configured with such information (e.g. UE is located in the overlapping area where the hosting network is available) or requires all the AMF/SMF in the serving network are configured with such information. The data is provisioned to UE via MM NAS or SM NAS message (it is TBD which procedures/messages are impacted).

-	Other solutions following this principle, proposes to use UDR in the serving network to store the localized service information data as application data for any UE registered to the serving network. This enables the flexibility that if the serving network is not the home network, UE can obtain localized service information data from either the serving network, or from the home network, or from both. The impact is to extend service specific parameter provisioning for the serving network to receive the data (see TS 23.502 [4] clause 4.15.6.7) and new UE policy to provide the data to the UE.

UE fetches the information from hosting network (Sol #22, #26, #34).

-	This principle assumes a hosting network is selected prior to a localized service is selected, since the localized services information arrives later from the hosting network. It can happen that the fetched information is not interesting for the end user.

-	The impact is to extend broadcast with capability for localized service, new on-demand SIB, or code for URL. Another impact is to deliver localized service information via registration reject/accept message.

NOTE:	Privacy aspect needs to be evaluated by WG SA3.

-	This principle implies that in some scenarios the UE needs to suspend the connection with serving network when getting information from hosting network, which can cause service interruption.

There are different triggers to provision the information to UE:

-	UE request as trigger (Sol #11, #13, #15, #24, #27, #32, #34).

-	UE location as trigger (Sol #11, #12, #27, #32).

-	UE subscription data change via external parameter provisioning as trigger (Sol #14, #23).

-	UE Steering of Roaming information update (SoR based solutions)

UE request as trigger assumes UE/end user is aware of the desired localized services. UE location as trigger requires the serving network is aware of which hosting network is available in which serving network tracking areas, and the information sent to UE is unsolicited. UE subscription data change and SoR based solutions restrict the information source to home network only.

7.4.4	Evaluation for how the localized service information is used by UE

New network selection mode is proposed (Sol #10, #25, #30, #33), where UE selects only hosting network when it intends for localized services and bypasses or temporary stops existing network selection procedure. This is needed in case UE is using home network credential to access hosting network, since home network and hosting network could be collocated in the same area and UE will always prioritize home network based on existing network selection mechanism. The new network selection mode can be requested by UE and authorized by home network (Sol #25).

The time and location validity condition along with other localized service information provided to UE can be used as a criteria for the UE to determine when and where to enter the new network selection mode.

Except the validity conditions, other methods for UE to determine whether to enter hosting network selection mode are also proposed, e.g. serving network to indicate to UE the presence of hosting network via either NAS or broadcast (Sol #12, #29), the availability of hosting network detected by UE regular scan (Sol #10), which requires extra system impacts.

*** END CHANGES ***