3GPP TSG-SA2 Meeting # 152E (e-meeting)	S2-2205808r01

# Elbonia, Aug 17 - 26, 2022																		 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0107 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CP and UP-based security procedures for 5G ProSe UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #2 “The 5G ProSe UE-to-Network Relay shall only include in discovery messages the RSCs with the Control Plane Security Indicator set, as specified in clause 5.1.4.1, when the requested slice(s) corresponds to Control Plane based security procedure is(are) accepted”, In our view, the RSC(s) without Control Plane Security Indicator (CPSI) set should not be ruled out, otherwise Remote UE(s) supporting only RSC(s) without CPSI may not be able to discover a Relay. When RSCs w/wo CPSI are supported, the Relay UE can select the RSCw/wo CPSI. In addition, if the Control Plane based security procedure is not supported by the UE and/or not supported by the network in the requested S-NSSAI, the Relay-enabled UE shall not include RSC with CPSI in Discovery message. #3 It is the Relay UE instead of the Remote UE that performs the CP-based security procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.1.4.3.1, clarify the coexistence of the CP and UP-based security procedure. Clause 5.1.4.3.2 Clarify that Relay UE support RSC does not rule out RSCs without CPSI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Function not working as intended, risk of no Relay service |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.3.1, 5.1.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Next Changes * * *

#### 5.1.4.3	Network controlled security procedures for 5G ProSe UE-to-Network Relay

##### 5.1.4.3.1	General

Security procedures over Control Plane and User Plane are specified for 5G ProSe UE-to-Network relaying in TS 33.503 [29].

When security procedures are supported over both the Control Plane and the User Plane,  the decision whether to use Control Plane or User Plane based security procedure is up to the UE implementation.

##### 5.1.4.3.2	Control Plane based security procedures for 5G ProSe UE-to-Network Relay

Control Plane-based security procedures for 5G ProSe UE-to-Network Relay call flow and procedure is defined in TS 33.503 [29]. It is characterised by the following principles:

-	5G ProSe UE-to-Network Relay's NAS signalling is used for the control plane based security procedure to authentice and authorize a 5G ProSe Remote UE.

-	The 5G ProSe UE-to-Network Relay can be configured to use a set of slices supporting Control Plane based security procedure. An AMF supporting Control Plane based security procedure for 5G ProSe UE-to-Network Relay is selected as part of the slice. If 5G ProSe UE-to-Network Relay enabled UE supports Control Plane based security procedure, and the network accepted the requested S-NSSAI supporting Control Plane based security procedure, the 5G ProSe UE-to-Network Relay enabled UE shall include in discovery messages the RSCs with the Control Plane Security Indicator set, as specified in clause 5.1.4.1, in addition to the RSC(s) without Control Plane Security Indicator, ,. The Relay UE shall use the control Plane security if RSC with Control Plane Security Indicator set is used by the Remote UE to establish the connection. Otherwise, if RSC without Control Plane Security Indicator is used by the Remote UE to establish the connection, the Relay UE shall use the user Plane security procedure as specified in clause 5.1.4.3.3.

-	The AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in clause 6.3.4 of TS 23.501 [4] using the identification information the 5G ProSe Remote UE provided as specified in clause 6.3.3.3.2 of TS 33.503 [29].

-	If the 5G ProSe Remote UE is configured by HPLMN to use control plane security procedure, the 5G ProSe Remote UE's HPLMN AUSF shall support control plane based security procedure.

-	If a network intends to use control plane security procedure, then all the AMFs or the all AMFs within the network slices that the 5G ProSe UE-to-Network Relay uses shall support the control plane based security procedures.

NOTE:	If the control plane security procedure is not supported, then the 5G ProSe Remote UE can select another 5G ProSe UE-to-Network Relay or user plane based security can be used.

##### 5.1.4.3.3	User Plane based security procedures

User Plane-based security procedures for 5G ProSe UE-to-Network Relay are defined in TS 33.503 [29].

* * * End of Changes * * *