SA WG2 Meeting #S2-152e(e-meeting)	S2-2205829

17 - 26 August 2022, Electronic Meeting	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI#3, Sol #41: update to remove one editor’s note, clarification of impacts and correction/clarification of references

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution provides updates to solution #41 by removal of one EN, impacts clarification and correcting/clarifying references.

# 1. Introduction

This contribution provides following updates to solution #41:

- Update of the references, alignment of references numbering with section 2 and addition of references to relevant sections. New IETF reference introduced reflecting the progress of the work on RFC.

- Clarification on NG-RAN and PCF impacts.

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clauses 2 and 6.

* Start of changes *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]	IEEE SA P1918.1: "Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions", https://standards.ieee.org/project/1918_1.html.

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[9]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[10]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[11]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

[12]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

[13]	IETF RFC 6190: "RTP Payload Format for Scalable Video Coding", May 2011.

[14]	IETF RFC 6437: "IPv6 Flow Label Specification", November 2011.

[15]	IETF RFC 8285: A General Mechanism for RTP Header Extensions.

[16]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[17]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[18]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[19]	ITU-T Recommendation H.264: "Advanced video coding for generic audiovisual services".

[20]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control", July 2003.

[21]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)", March 2016.

[22]	ISO/IEC 14496-12 | 15444-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format".

[23]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[24]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[25]	IETF RFC 1889: "RTP: A Transport Protocol for Real-Time Applications", January 1996.

[26]	3GPP TS 29.998: "Open Services Architecture (OSA) Application Programming Interface (API) part 2".

[27]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[28]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[29]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[30]	IETF RFC 5888 (2010): "The Session Description Protocol (SDP) Grouping Framework", G. Camarillo, H. Schulzrinne.

[31]	IETF RFC 3261 (2002): "SIP: Session Initiation Protocol", J. Rosenberg et al.

[32]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[33]	IETF RFC 3665: "Session Initiation Protocol (SIP) Basic Call Flow Examples", A. Johnston et al.

[34]	WebRTC TR: https://www.w3.org/TR/webrtc/.

[35]	IETF RFC 7272: "Inter-Destination Media Synchronization (IDMS) Using the RTP Control Protocol (RTCP)".

[36]	IETF RFC 8311: Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation.

[37]	IETF Internet-draft - L4S: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture", Briscoe et al, Nov 2020, https://datatracker.ietf.org/doc/draft-ietf-tsvwg-l4s-arch/.

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[39]	EU project RITE: "Data Center to the Home", https://riteproject.eu/dctth/.

[40]	L4S: "Linux Kernel tree containing patches for TCP Prague and the dualpi2 qdisc", https://github.com/L4STeam/linux/.

[41]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1", June 1999.

[42]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)", May 2015.

[43]	IETF draft-ietf-quic-http-34: "Hypertext Transfer Protocol Version 3 (HTTP/3)", February 2021.

[44]	IETF RFC 6994: "Shared Use of Experimental TCP Options", August 2013.

[45]	IETF draft-ietf-tsvwg-udp-options-18: "Transport Options for UDP", March 2022.

[46]	ITU-T Recommendation H.265: "High efficiency video coding".

[47]	ITU-T Recommendation H.266: "Versatile video coding.

[48]	draft-ietf-avtcore-rtp-vvc-14: "RTP Payload Format for Versatile Video Coding (VVC)".

[49]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[50]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[51]	IETF RFC 9001: "Using TLS to Secure QUIC".

[52]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[53]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[54]	draft-ietf-masque-connect-udp: "UDP Proxying Support for HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[55]	draft-ietf-quic-http: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[56]	draft-ietf-masque-h3-datagram: "Using Datagrams with HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[57]	draft-ietf-httpbis-h3-websockets: "Bootstrapping WebSockets with HTTP/3".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[58]	3GPP TR 38.838: "Study on XR (Extended Reality) evaluations for NR".

[59]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[X]	IETF Internet-draft - L4S: “Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)” Briscoe et al, July 2022, https://datatracker.ietf.org/doc/draft-ietf-tsvwg-ecn-l4s-id/

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

## 6.41	Solution #41: Use of ECN bits for L4S to enable codec/rate adaptation to meet requirements for services

### 6.41.1	Key Issue mapping

This solution addresses KI#3.

### 6.41.2	Description

L4S, "Low Latency, Low Loss and Scalable Throughput", is an network service using AQM-like mechanism which, instead of dropping packets, uses link state indications and rate adjustments proportional to the queue delay.

L4S is subject to standardization in IETF drafts [37], [X]. L4S has been demonstrated in the RITE EU project [39]. Congestion control algorithms that support L4S are described, e.g. in [40].

This solution proposes that the 5G System uses ECN bits defined in RFC 8311 [36] as identifier to be used on IP packets for the new network service called low latency, low loss and scalable throughput (IETF drafts [37], [X]).

In this solution the network is configured by existing means to map desired, L4S enabled packet flows on a pre-configured 5QI and thus QoS Flow.

To address service requirements, by using ECN bits for marking of payload packets as specified in RFC 8311 [36] and in accordance with IETF drafts [37] and [X], the NG-RAN exposes current load level.

The solution is based on following two components:

-	Use of ECN bits in NG-RAN for L4S in accordance with IETF drafts [37] and [X].

-	Enablement of using ECN bits for L4S.

NOTE:	This solution is applicable only in case user terminal and Application Server support L4S defined in RFC 8311 [36] and IETF drafts [37], [X] and use protocols that support L4S feedback between user terminal and application server.

#### 6.41.2.1	Use of ECN bits in NG-RAN for L4S

It is assumed that the varying radio conditions and resource availability in NG-RAN are the main contributors to the need for the application to adapt its rate accordingly.

Given that it is NG-RAN that has the visibility of the resource availability and sudden changes on the radio interface that impact the performance in terms of latency, any fast reaction to trigger rate adaptation, that is required for services with tight latency requirements and benefit from bounded latency, must be triggered by NG-RAN. NG-RAN makes use of ECN bits for marking of payload packets as specified in RFC 8311[36] and in IETF drafts [37], [X] to support L4S. ECN bits marking interacts with the application layer, wherein the application layer triggers rate adaptation based on feedback using ECN bits. In this solution NG-RAN makes use ECN bits marking for both, DL and UL direction.

NOTE:	The criteria that RAN perform the marking is up to RAN implementation.

Editor's note:	Whether RAN can support such a marking need to be coordinated with RAN WGs.

#### 6.41.2.2	Enablement of using ECN bits marking for L4S

To enable ECN bits marking for L4S, an existing or a separate QoS flow can be used for L4S traffic. In context of this descriptive text for readability reasons, we refer to it as an 'L4S QoS Flow'. It shall be noted that this solution does not propose to introduce a new concept and instead it reuses the 5QI framework. Namely, use of a certain QoS flow for L4S traffic, enabling ECN bits marking treatment in NG-RAN, is achieved via a preconfigured 5QI value. One or several 5QI(s) may be defined for this purpose by the operator in a deployment.

There are 3 main principles to establish a L4S QoS flow within a PDU session:

-	Statically: At PDU session establishment the L4S QoS flow is always established. This by either a pre-configured PCC rule if PCF is used, or by configuration in SMF.

NOTE:	By a static configuration L4S support is ensured and can be used by endpoints (UE and Server(s)) supporting L4S

-	Dynamically based on 5GC configurations: SMF installs a Detection filter in UPF. This is either based on a request from PCF or from local configuration. The Detection filter can either use the ECN bits for L4S in the IP header or the XR server IP address(es). When L4S traffic is detected, the UPF notifies SMF. If PCF is used, SMF notifies PCF which triggers a L4S QoS flow establishment, or if PCF is not used, SMF is configured to establish a L4S QoS flow.

-	Dynamically based on AF request: The AF makes an authorization request either via NEF (untrusted) or directly to PCF, which triggers a Session modification procedure according to clause 4.3.3 in TS 23.502 [3].

The filters provided by SMF in the QoS rule to the UE and PDR to the UPF to identify traffic to be routed onto the L4S QoS flows can be a combination of existing filters and/or the ECN bits for L4S in the IP packet header of the XR service.

Editor's note:	Other means to enable of L4S use are FFS.

### 6.41.3	Procedures

This solution re-uses existing procedures (TS 23.502 clauses 4.3.2 PDU Session Establishment, 4.3.3 PDU Session Modification, 4.16.4 SM Policy Association Establishment and 4.16.5.1 SMF initiated SM Policy Association Modification).

### 6.41.4	Impacts on services, entities and interfaces

NG-RAN:

- support for ECN bits marking for L4S of payload packets as described in clauses 6.41.1, 6.41.2 using RFC 8311 [36] and in accordance with IETF drafts [37] and [X].

- Existing functionality and protocols: 5QI preconfiguration to identify a QoS Flow subject to ECN bits marking for L4S of payload packets as described in RFC 8311 [36] and in accordance with IETF drafts [37] and [X].

PCF:

- Existing functionality and protocols: provide a PCRT to detect start of L4S traffic and to provide PCC Rules that includes SDF templates to identify L4S traffic and QoS control information to allocate a preconfigured 5QI value for L4S traffic. When the SMF reports stop of L4S traffic to the PCF, the PCF removes the PCC Rule that was installed when the start of L4S traffic was reported see TS. 23.503 Table 6.1.3.5-1: Access independent Policy Control Request Triggers relevant for SMF and PCRT: Start of application traffic detection and Stop of application traffic detection.

* End of change *