SA WG2 Meeting #152E (e-meeting)	S2-2205843r02

Aug 17 – 26, 2022	(was S2-220XXXX)

Source:	vivo, China Mobile

Title:	KI#6, Conclusions

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Introduction of conclusions for KI#6.

1. Proposal

This paper proposes the following conclusions for KI#6 to TR 23.700-60.

* Start of change *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	Conclusions for KI#6

The following aspects are concluded as principles for the normative work:

-	RAN splits the RT latency into an UL PDB and a DL PDB. The UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency.

-	The PCF shall generate one PCC rule including the RT latency for the media stream, for both the UL and the DL direction. The RT latency is represented by 5QI and an RTT indication. Based on the RTT indication, RAN figures out the RTT latency by doubling the PDB associated with the 5QI.

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow needs to meet that the RT delay overhead doesn’t exceed the doubling of the 5GS delay. The AF provides the RT latency requirement to the PCF during AF session with required QoS procedure.

* End of changes *