SA WG2 Meeting #152E (e-meeting)	S2-2205843r03

Aug 17 – 26, 2022	(was S2-220XXXX)

Source:	vivo, China Mobile

Title:	KI#6, Conclusions

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Introduction of conclusions for KI#6.

1. Proposal

This paper proposes the following conclusions for KI#6 to TR 23.700-60.

* Start of change *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	Conclusions for KI#6

The following aspects are concluded as principles for the normative work:

-	Based on RT latency requirement, the 5GS shall split the RT latency into an UL PDB and a DL PDB. The UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency. Based on operator policy, the RT latency can be split by the PCF or the RAN.

-	If the PCF split the RT latency into an UL PDB and a DL PDB,	 the PCF shall generate two PCC rules for the media stream, one for the UL and one for the DL direction, and assigns the 5QIs according to the PDBs respectively. 	The PCF may also issue two QoS monitoring policies to request monitor UL delay and DL delay respectively. Based on the received QoS monitoring result, the PCF can adjust the UL PDB and the DL PDB, so that UL/DL PDB fits better to the new situation while the RT latency is not exceeded.

-	If the RT latency is split by the RAN , the PCF shall generate one PCC rule including the RT latency for the media stream, for both the UL and the DL direction. The RT latency is represented by 5QI and an RTT indication. Based on the RTT indication, RAN figures out the RTT latency by doubling the PDB associated with the 5QI. The NG-RAN shall ensure the RT latency is not exceeded.

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow needs to meet that the RT delay overhead doesn’t exceed the doubling of the 5GS delay. The AF provides the RT latency requirement to the PCF during AF session with required QoS procedure.

* End of changes *