SA WG2 Meeting #152E (e-meeting)	S2-2205843r04

Aug 17 – 26, 2022	(was S2-220XXXX)

Source:	vivo, China Mobile, Tencent

Title:	KI#6, Conclusions

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Introduction of conclusions for KI#6.

# 1. Proposal

This paper proposes the following conclusions for KI#6 to TR 23.700-60.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Conclusions for KI#6

The following aspects are concluded as principles for the normative work:

-	Based on RT latency requirements, the 5GS shall split the RT latency into UL PDB and  DL PDB. The UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency.

-	The UL and DL PDBs applies to two different QoS Flows.

-		The PCF shall generate two PCC rules for the XRM service, one for the UL and one for the DL direction, and assigns the 5QIs according to the PDBs respectively. The PCF may generate alternative QoS parameter sets for UL/DL QoS Flows according to RT latency requirement.

The PCF may also issue two QoS monitoring policies to request monitoring of UL delay and DL delay respectively. Based on the received QoS monitoring result, the PCF can adjust the UL PDB and the DL PDB, so that UL/DL PDB fits better to the new situation while the RT latency is not exceeded.

-	When PDB of one of UL/DL QoS Flows cannot be satisfied, RAN may choose the proper Alternative QoS Profiles for both of UL/DL QoS Flows to fulfil the RT latency requirement. RAN report these changes via QoS Notification Control procedures.

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF if PCF doesn’t perform such pre-configuration. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow need to meet that the RT delay overhead doesn’t exceed the doubling of the 5GS delay. The AF provides the RT latency requirements to the PCF during AF session with required QoS procedure.

* End of changes *