SA WG2 Meeting #152E (e-meeting)	S2-2205880

17 - 26 August, 2022, Electronic meeting	(revision of S2-2203919r03)

Source:	Ericsson

Title:	New WID: Enhancements to usage of Target NSSAI

Document for:	Endorsement

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Enhancements to usage of Target NSSAI

Acronym: TEI18_ TARGET_NSSAI

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900032 | Enhancement of Network Slicing Phase 2 | Related functionality that is proposed to be enhanced was introduced by this Work Item. |

# 3	Justification

Currently, the NSSF or AMF can decide based on a policy consistent with the agreement with the customer (i.e."based on network policies that are in line with customer and operator agreements"), to provide a TARGET NSSAI to the RAN that excludes one or more or even all of the S-NSSAIs in the Allowed NSSAI.

This decision has to be per UE as the relative importance of a slice relative to other slices is not the same for all the UEs. The AMF cannot however determine the importance of a slice for a UE without receiving the information from the UDM (or PCF if preferred albeit a second interaction is required). In fact the information on the subscribed slices comes from the UDM (for both Roamers and non roamers) and it is natural that any relative importance of the S-NSSAIs in the subscription for the UE has to come from the UDM also (or PCF may be alternate solution but then a second interaction is required after receivinng the Subscribed s-NSSAIs at AMF - TBD if this meets energy efficiency requirements)

The TARGET NSSAI is defined locally at AMF or provided by the NSSF. The NSSF cannot determine the relative importance of a network slice per UE (the NSSF only provides decisions that are UE independent without receiving per UE information from AMF), so this information has to come from the AMF. It also then follows the NSSF has to receive the S-NSSAIs relative importance from the AMF in order to pick the right target NSSAI for the UE.

At the moment when NG-RAN received the Target NSSAI (and the associated RFSP), it may occur that the serving NG-RAN node is not able to identify a redirection target cell/frequency that supports the network slices in the Target NSSAI. In this case, the serving NG-RAN node logic is required to revert to operate based on TARGET NSSAI

“The RFSP index associated to the Target NSSAI is considered if the RAN succeeds to redirect the UE to a new TA outside the RA, otherwise the RFSP index of the Allowed NSSAI is considered.”

For the NG-RAN node to be able to continue searching for a redirection target cell/frequency the Target NSSAI and the (and the associated RFSP) needs to be stored in the UE context in the NG-RAN and the target NSSAI and its RFSP only needs to be considered for RRM. to achieve that, the AMF shall provide to the NG-RAN the indication to store the Target NSSAI and use it for RRM . If this indication is sent we can then wite in the standards this:

“The RFSP index associated to the Target NSSAI is considered while the target NSSAI is stored if received with an indication to store it, until the Target NSSAI is removed by the AMF ”

the target NSSAI can be removed e.g. because the AMF ha received a new registration request that make the existing target NSSAI no longer needed.

With reference to figure 1, where it is highlighted S-NSSAI 2 works on band 2 in TA2 and S-NSSAI1 works in set of TAs included in a RA (TA1*) in Band 1.

FIGURE 1

The UE may be interested in using S-NSSAI 2 and S-NSSAI1 at time T. Then it attempts to register from TA12. The UE gets Allowed NSSAI = S-NSSAI 1 and TARGET NSSAI = S=NSSAI 2 as the AMF/NSSF determine S-NSSAI 2 is more important. However, when the UE in TA12, the RAN cannot redirect to TA2 and today the RAN would continue to operate RRM for the UE based on Allowed NSSAI only. This however may prevent the attempt to redirect to TA2 if the UE moves to TA13.

Therefore, the AMF shall be able to indicate to the RAN to store the TARGET NSSAI and its RFSP in the UE context and use it for RRM of the UE. Then this is associated to also the capability to transfer the Target NSSAI and its RFSP to the next gNB if AMF policy is to do so.

Also, there can be candidate target cells/TAs supporting alternative sets of S-NSSAIs that can be used as Target NSSAI. (see figure 2).

Let's suppose that a UE can request 3 S-NSSAIs (1,2,3). Let's also suppose the UE is registered in TAI 1 where the S-NSSAI 1 is supported, but then S-NSSAI 1 is not supported in any of TAI 2, TAI 3, and also S-NSSAI 2 is supported in TAI2 and not in TAI3. TAI1, and S-NSSAI 3 is supported in TAI3. but not in TAI1 and TAI2.

FIGURE 2

The current specifications are clear the Target NSSAI must be formed so it can be an Allowed NSSAI in the TAs that can be targeted.

“The Target NSSAI shall only include S-NSSAIs that can be provided in an Allowed NSSAI for the UE.”

Hence providing S-NSSA2 and S-NSSAI3 in the target means that S-NSSAI 2,3 can be allowed together. We therefore propose that to address similar cases (this being the simplest one) alternative Target NSSAIs ranked in priority order are needed each with its RFSP.

with reference to figyure 3

With reference to Figure 3: The UE is registered in a RA with S-NSSAI 1 and requests S-NSSAI1, S-NSSA2. The AMF/NSSF detect some TA2 where Both can work that is overlapping with the RA in some location, so it assigns Target NSSAI = (S-NSSAI1, S-NSSAI2) and indicates to store it. However, if we perform RRM based on target NSSAI then when the UE arrived at TA11, the RAN will redirect the UE to a cell in TA3 as it is instructed to do so based on TARGET NSSAI RRM (the TA3 cell meets all the constraints and criteria to be selected based on Target NSSAI as it supports a subset of Target), thus potentially causing an unnecessary redirection/handover.  Similar considerations would apply if the UE moved to TA15. We conclude therefore that the TARGET NSSAI and its RFSP should be scoped to some TAIs of the RA only.

FIGURE 3

Therefore, we propose that the Target NSSAI RFSP is used only if the target cell supports more S-NSSAIs of the target NSSAI than the current cell.

# 4	Objective

The objective is to enhance the usage of the Target NSSAI by:

-	Provide the S-NSSAIs priorities to the AMF from UDM/PCF (TBD) to enable a per UE decision when it is required to exclude certain Allowed S-NSSAIs from the target NSSAI

-	Provide the S-NSSAI priorities to NSSF for NSSF to decide the target NSSAI

-	Enable the NG-RAN to store the Target NSSAI and the associated RFSP in the UE context and conditionally use it for RRM instead of the Allowed NSSAI and its RFSP if the AMF requests the RAN to do so.

-	If indicated by the AMF, The NG-RAN stores the Target NSSAI and the associated RFSP in the UE context until AMF indicates to the NG-RAN to remove the Target NSSAI and the associated RFSP (i.e. the information is no longer valid).

-	Enable the Target NSSAI and the associated RFSP to be made available to a target NG-RAN node upon mobility by:

-	AMF re-sends the same Target NSSAI and associated RFSP.

-	Allow the AMF to indicate alternative Target NSSAIs to the NG-RAN ranked in priority order.

-	Clarify that NG-RAN uses the associated RFSP to prioritize cells in the case there are no target cell/TA supporting all the S-NSSAIs of the Target NSSAI.

-	Clarify that NG-RAN due to receiving a Target NSSAI, does not redirect the UE to cells/TAs that does not support more S-NSSAIs of the Target NSSAI than the current TA

NOTE:	Clarify that the AMF may determine the use of Target NSSAI/RFSP also considering whether the UE supports normative work resulting from KI#5 functionality of FS_eNS_Ph3

Note: This work will require 0.75 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Add description that NG-RAN can store the Target NSSAI and associated RFSP in UE context and retry when target cell becomes available, AMF can re-send the Target NSSAI and associated RFSP when NG-RAN node is changed. | SA#99 (Mar 2023) | Target completion to be determined at SA2#152 |
| 23.502 | Add description that Target NSSAI can be also in xyz NGAP message. | SA#97 (Sep 2022)? |  |

# 6	Work item Rapporteur(s)

xxxx, Ericsson (@ericsson.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Impacts to NGAP is to be handled by RAN3.

# 9	Supporting Individual Members

Supporting IM name

Deutsche Telekom

Ericsson

NEC

Orange – to be confirmed

Nokia

Nokia Shanghai Bell