3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205887r03

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	China Mobile, CATT, vivo

Title:	FS_5MBS_Ph2 KI#3, Sol#11 Update for making clarification

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This paper proposes clarification to TR 23.700-47 Solution #11.

# Proposal

It is proposed to apply the changes below to solution#11 in TR 23.700-47.

*** Start of changes***

### 6.11.3	Procedures

#### 6.11.3.1	on-demand multicast MBS session management

Figure 6.11.3.1-1: On-demand multicast MBS session establishment

The key steps of the procedure for this solution are as follows:

1.	MBS Session Creation with PCC procedure is performed as specified in clause 7.1.1.3 of TS 23.247 [4], where the AF may perform a Service Announcement towards the UEs.

2.	The AF decides to set or update multicast related service parameters, including the multicast indication, for specific services/applications. The AF may, based on local configuration or triggered by e.g. event report from the NEF, subscribe to or requests network analytics information (e.g. Observed Service Experience analytics) from the NWDAF as specified in TS 23.288 [10], and decide to update the multicast related service parameters based on the analytics information.

The AF provides updated service parameters, which include the multicast indication and a MBS Session ID for the multicast MBS session to be established, to the PCF via the NEF by using Nnef_AFsessionWithQoS_Update request (as defined in clause 4.15.6.6a of TS 23.502 [3])). The "AF session with required QoS update" procedure continues as defined in TS 23.502 [3] and the PCF receives the updated service parameters.

3.	The PCF initiates SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] to notify the SMF, with the multicast MBS session ID.

Editor’s note: PCF identifying the UE is based on UE’s private IP address. How to handle the case that only public IP address is available is FFS.

4.	Steps 2-19 specified in clause 7.2.1.3 of TS 23.247 [4] are reused to complete on-demand multicast MBS session establishment, for the UEs subscribed to the multicast MBS Session and with an associated PDU Session established for the multicast MBS Session, and Namf_Communication_N1N2MessageTransfer is used in step 5 instead of Nsmf_PDUSession_UpdateSMContext response message. The UE is informed of successfully joining the multicast MBS session upon receiving the PDU Session Modification Command which contains the MBS Session ID.

Editor’s note: Steps 2-19 in clause 7.2.1.3 of TS 23.247 [4] is intended for UE requested PDU Session Modification procedure, how it is reused for the procedure in this solution is FFS.

### 6.11.4	Impacts on services, entities and interfaces.

AF:

-	The AF needs to support triggering the on-demand multicast MBS Session establishment, based on e.g. the network analytics information provided by the NWDAF.

PCF:

-	The PCF is enhanced to provide to the SMF(s) the multicast service related policy, i.e. MBS session ID that the AF request UE to join, for triggering the SMF(s) to complete the on-demand multicast MBS session establishment.

*** End of changes ***