3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205890r05

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI #2, Sol #9: Update to EN and clarification

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This paper proposes clarification to TR 23.700-81 clause 6.9 Solution #9: NWDAF-assisted application detection.

Discussion

TR 23.700-81 Clause 6.9 includes Solution #12: NWDAF-assisted application detection, this proposal makes the following clarification.

1) This solution is to address the use case as follows:

AF previously provided PFD information to NEF (PFDF), when the AF updates the PFD information, the AF:

- does not provision the updated PFD information to NEF (PFDF) in time or;

- does not provision the updated PFD information to NEF (PFDF) anymore.

So sol#9 only provides analytics including new PFDs for updated PFDs for known applications. The FFS related to unknown applications can be removed.

2) Current PFD information in use is kept in the UDR, which the NWDAF could collect. However, ADRF is not supposed to store historical PFD information, it seems not necessary or helpful to newly define the related procedures. So suggest no impact on ADRF.

3) Regarding the case that there is overlapping PFD information provided by NWDAF and provided by AF, this proposal intends to make the mechanism simpler, i.e., the AF subscribes to the NWDAF to request analytics for application detection carrying the application ID, based on the PFD analytics information notified by the NWDAF, it is only up to the AF to determine whether PFD information needs to further create/update/delete PFD information corresponding to the application ID, so the issue about overlapping PFD information provided by NWDAF and provided by AF doesn’t exist.

Proposal

It is proposed to apply the changes below to solution#9 in TR 23.700-81.

*** Start of changes***

6.9	Solution #9: NWDAF-assisted application detection

6.9.1	Description

This solution is proposed to address Key Issue #2: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is use a set of packet headers which usually contains source and destination address. The application ID can be used for referring to the UPF's specific application detection filter, and AF may provide PFDs according to clause 4.18 of TS 23.502 [3] to update application detection filter information associated with the application ID. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF delivers PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [2].

However, as indicated by the KI description, the ASP may provide an initial PFD information but does not update it in time or does not update it anymore, then it is unknown how the UPF detect the application traffic.

The objective of this solution is to provide analytics on application traffic. The proposed solution provides analytics including new PFD information for updated PFD information for known applications by extracting network traffic signature and inferring application id. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload characteristics (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502 [3].

The NWDAF could collect the current PFD information in use (including Application ID, IP 3-tuple, URL, Domain name information) from the UDR via NEF (PFDF).

Based on PFD information from NEF/UDR and traffic information from UPF, new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) for the existing Application ID could be derived by the NWDAF. The new PFD information provided by the NWDAF can be used by the NEF for provisioning SMF/UPF.

The below is the principles of the solution:

-	A consumer (e.g. NEF(PFDF) or AF) requests to the NWDAF to provide PFDs for newly detected applications.

-	The NWDAF requests to provide information on SDF that does not match an installed PDR at the SMF/UPF.

-	The NWDAF analyses the collected data to generate unique traffic patterns, and provides it to the consumer.

-	The consumer i.e. NEF or AF decides to create/update/delete PFD associated with an application ID to enable the detection of application traffic in the future (out of scope).

6.9.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. The detailed data are described in Table 6.9.2-1.

Table 6.9.2-1: Input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE that uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Start/end time | UPF | Start and end time of traffic detection. |
| Data volume | UPF | Measured data traffic volume (per UL/DL). |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL). |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL). |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL). |
| IP 3-tuple | UPF | to Identify a service flow of the UE that uses the application. |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow. |
| Size of packets | SMF/UPF | Average size of packets. |
| PFD Information | NEF(PFDF) | PFD Information stored in the NRF and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL | NEF(PFDF) | the significant parts of the URL to be matched, e.g. host name. |
| > Domain Name | NEF(PFDF) | a Domain name matching criteria and information about applicable protocol(s). |

NOTE:	Extensive reporting of all traffic flows may conflict with requirement to avoid extra UPF load in the key issue. The issue regarding the performance of UPF user plane traffic handling will depend on the outcome of FS-UPEAS SID.

6.9.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.9.3-1.

The output analysis can be used to provision new PFDs for known applications.

Table 6.9.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| URL | the significant parts of the URL to be matched, e.g. host name defined in. |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

6.9.4	Procedures

The procedure depicted in 6.9.4-1 shows new application detection analytics.

Figure 6.9.4-1: A procedure to extract application characteristics

1.	The consumer NF (e.g. AF) subscribes to the NWDAF to request analytics for application detection (or via the NEF in untrusted domain). This subscription maybe is triggered by local configuration or OAM. The Analytics Filter Information may optionally include the UE ID, S-NSSAI and/or DNN, Application ID.

2-. 		The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF).

3.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets corresponding to Application ID from the SDF not matching installed PDRs.

4.	The NWDAF derives PFD analytics.

5.	The NWDAF notifies the analytics consumer NF with observed PFD

Information. Based on the observed PFD information provided by the NWDAF, the consumer determines to create/update/delete PFDs information and if needed send request to the NEF(PFDF) by invoking the Nnef_PFDManagement_Create/Update/Delete service containing Application Identifier and one or more sets of PFDs, and step 1-5 defined in clause 4.18.2 in TS 23.502 are executed.

6.9.5	Impacts on existing nodes and functionality

NWDAF:

-	Needs to provide a new analytic for application detection information.

-	Needs to process user plane data for extracting traffic characteristics.

UPF:

-	Needs to report URL, Domain name part, and IP 3-tuples of packets from application to NWDAF.

-	Needs to provide requested sessions statistics including data volume, data duration, QoS flow bit rate and packet transmission.

AF or NEF(PFDF):

-	Supports to consume new PFD information from NWDAF.

*** End of changes ***