SA WG2 Meeting #152e	S2-2205919r02

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, AT&T, Deutsche Telekom

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

1 Discussion

2 Proposal

It is proposed to update TR 23.700-18 as follows.

Start of change

8	Conclusions

8.1   Key Issue #1: Traffic Steering Policy and SFC Enhancements

It is recommended to use the following principles and procedures as the basis for the normative work.

-	The PCF determines a policy per SDF/application for the purpose of steering the subscriber's traffic to appropriated N6 service functions deployed by the operator or a 3rd party service provider. The policy is expressed in a Traffic Steering Policy (TSP) ID that may be separate in UL and DL directions.

-	The TSP ID refers to a traffic steering behaviour that is configured in the SMF/UPF.

-	The PCF provides the TSP ID in the PCC rules to SMF.

-	The SMF provisions corresponding PDRs, FARs, QERs etc to support SFC. In particular, the SMF creates a FAR with the Forwarding Policy parameters set to the TSP ID. No impacts to N4 rules

-	The UPF performs the necessary actions based on the Forwarding Policy, to enforce the forwarding policy referenced by the SMF, e.g. performing packet encapsulation, packet marking and routing the traffic towards the service functions within the N6-LAN.

-	It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

-	Support the N6-LAN traffic steering control and AF-influenced traffic steering control to be applicable to the same traffic simultaneously.

-