SA WG2 Meeting #152e	S2-2205919r02

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, AT&T, Deutsche Telekom

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

# 1 Discussion

# 2 Proposal

It is proposed to update TR 23.700-18 as follows.

Start of change

# 8	Conclusions

## 8.1   Key Issue #1: Traffic Steering Policy and SFC Enhancements

It is recommended to use the following principles and procedures as the basis for the normative work.

-

-

-	Support the N6-LAN traffic steering control and AF-influenced traffic steering control to be applicable to the same traffic simultaneously.

-	5GC may receive from the AF policies associating for one, a group or all UE(s) some traffic (filter) with a SFC Id together with optional metadata to send to the SFC once the corresponding traffic has been identified

Editor’s Notes: it is still FFS whether metadata are supported

-	The PCF determines a policy per SDF/application for the purpose of steering the subscriber's traffic to appropriated N6 service functions deployed by the operator or a 3rd party service provider. The policy is expressed in a Traffic Steering Policy (TSP) ID that may be separate in UL and DL directions.

-	NEF checks whether the indicated SFCP ID and Metadata correspond to an authorized SFC policy for the AF. The NEF may need to parse or interpret the Metadata e.g. to check whether the requested Metadata is on the agreed SLA list, but 5GC CP does not make any other use of the Metadata apart from forwarding them via the PCF/SMF to the UPF implementing the SFC functionality.

-

-

-	the SFC ID is mapped by the PCF to a TSP ID that refers to a traffic steering behaviour that is configured in the SMF/UPF. The SMF/UPF don’t need to be aware of SFC ID

Editor’s Notes: it is still FFS the mapping between SFC ID and TSP ID is 1-1 i.e. whether the SFC ID is actually a TSP ID

-	The PCF provides the TSP ID in the PCC rules to SMF.

-	The SMF provisions corresponding PDRs, FARs, QERs etc to support SFC. In particular, the SMF creates a FAR with the Forwarding Policy parameters set to the TSP ID.

Editor’s Notes: if metadata are supported then PCC rules and FAR are modified to support the (transparent) transfer of metadata

-	The UPF transforms the TSP ID into SFP configuration including the way to indicate over N6 traffic belonging to SFC; there may be different SFP for UL and DL traffic. This mapping is controlled by local configuration.

-	It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

-

-

-

-