SA WG2 Meeting #152e	S2-2205919r12

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, AT&T, Deutsche Telekom

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

# 1 Discussion

# 2 Proposal

It is proposed to update TR 23.700-18 as follows.

Start of change

# 8	Conclusions

## 8.1   Key Issue #1: Traffic Steering Policy and SFC Enhancements

It is recommended to use the following principles and procedures as the basis for the normative work.

-	Support the N6-LAN traffic steering control and AF-influenced traffic steering control to be applicable to the same traffic simultaneously.

-	5GC may receive from the AF policies associating for one, a group or all UE(s) some traffic (filter) with a SFC ID together to send to the SFC once the corresponding traffic has been identified.

Editor’s Notes: it is still FFS whether metadata are supported.

Editor’s Note: It is FFS whether SFC ID needs to be provided per traffic direction.

-	The PCF determines a policy per SDF/application for the purpose of steering the subscriber's traffic to appropriated N6 service functions deployed by the operator or a 3rd party service provider. The policy is expressed

in a Traffic Steering Policy (TSP) IDs that may be different in UL and DL directions

.

-	PCF checks whether the indicated SFC IDs correspond to an authorized SFC policy for the AF.

Editor’s Note: It is FFS whether NEF or PCF performs the check whether the indicated SFC IDs correspond to the agreed SLA list.

-	The PCF maps the SFC IDs into

a TSP ID(s) (one per direction) that refers to a traffic steering behaviour that is configured in the SMF/UPF. The SMF/UPF don’t need to be aware of SFC IDs

.

-	The PCF provides the TSP ID(s) in the PCC rules to SMF. The TSP ID refers to a traffic steering behaviour that is configured in the SMF/UPF.

-	The SMF provisions corresponding PDRs, FARs, QERs etc to support SFC

-  creating a FAR with the Forwarding Policy parameters set to the TSP ID. The UPF serving as PSA uses the TSP ID to steer traffic over N6. In this case, the TSP ID identifies a specific Service Function Path in the SFC.

- It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

Editor’s Note: It is FFS whether to progress the option where the SFC functionality is implemented in a UPF that can be dynamically inserted on N6 (only when requested by an AF) i.e., the option that does not have the constraint of static configuration as described in the bullet above.

Editor’s Notes: if metadata are supported then PCC rules and FAR are modified to support the (transparent) transfer of metadata; in that case, the UPF is assumed to provide these metadata together with the traffic sent to the N6. How the UPF forwards the metadata on N6 is FFS.