SA WG2 Meeting #S2-152E	S2-2205925

17 – 26 August, 2022, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	Updates to solution 19 for key issue 1

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes updates for solution 19 for key issue 1

# 1.	Discussion

This contribution aims to address some editor´s notes in solution 19 . However, editor´s notes for solution 19 that point to RAN dependencies will likely not be updated but the dependencies will be considered during the evaluation.

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

## 6.19	Solution #19: Procedures for Transmission mode tor inactive data reception

### 6.19.1	Introduction

This solution addresses Key Issue #1.

### 6.19.2	Functional description

This solution builds on top of solution 1.

UEs that transition to RRC inactive mode can move freely in their RAN notification area without notifying the RAN nodes. Their location is thus not known at cell level. At the moment only the RAN node serving a UE is aware that it is participating in an MBS multicast session. Additional RAN nodes not handling RRC connected UEs within the multicast session but within RAN notification areas of those UEs may need to be made aware of the multicast session and perform procedures for the multicast session, such as deciding whether to deliver data for the MBS session and choosing the delivery mode mode,  transmitting data of the multicast session and managing the related the shared delivery path.

Editor's note:	Related procedures need to be defined by RAN WGs. It also needs to be studied by RAN WGs whether it is possible to determine the presence of inactive UEs that desire to receive a multicast session in a cell and whether and how a RAN can decide whether to deliver multicast data in cells without RRC connected UEs.. 	Whether additional RAN nodes not handling RRC connected UEs within the multicast session need to be made aware of the multicast session and perform procedures for the multicast session needs to be decided by RAN WGs.

RAN nodes not serving any connected UEs in the MBS sessions may thus need to be aware of the MBS session. This solution describes two option to accomplish that with CN impacts:

Option A: RAN nodes inform their peers serving parts of RAN notification areas of UEs in an MBS session about the MBS session. The peer RAN nodes request assistance information from the CN via shared delivery establishment.

Option B: The MB-SMF informs all RAN nodes in a service area of an MBS session via the AMF about the MBS session

Editor's note:	The RAN-centric option A is in scope of RAN WGs to a large extent. Whether this option is selected thus depends on RAN WG.

As service areas are optional for multicast MBS sessions, all RAN nodes in a PLMN are possible affected by Option B.

ENOTE 1:	It may be preferable to only inform RAN nodes within RAN paging areas

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS session and trigger RRC-inactive UEs to transition to RRC-connected state. For the delivery mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated, but remain in RRC-inactive state for multicast data reception.

NOTE 2:	It is not assumed all the RAN node within the same RNA area have the same capability and choose the same delivery mode.

Editor's note:	. This assumption needs to be confirmed by RAN WGs. Procedures to support UEs that move between cells with different delivery mode need to be agreed by RAN WGs.

### 6.19.3	Procedures

.

#### 6.19.3.1	Moving a UE to RRC Inactive state and providing assistance information to additional RAN nodes (Option A)

Figure 6.19.3.1-1: NG-RAN node moves a UE in 5MBS session to RRC Inactive state

0.	UE joined an MBS session and receives MBS data while in RRC connected state

1.	NG-RAN 1 uses assistance information to decide whether to apply the delivery mode for RRC-inactive reception for an MBS session, and whether to move one or several UEs in MBS session to RRC inactive state.

2.	RAN internal procedures.

3.	NG-RAN 1 determines that part of the RAN notification areas of the UEs is served by RAN 2. It informs RAN 2 that handling of the MBS session for inactive UEs is required in that area.

Editor's note:	Depending on RAN decision, assistance information for MBS might be transferred in that step.

4.	The NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information as defined in solution 1for inactive reception in the shared delivery response.

5.	RAN internal procedures at NG-RAN 2 to handle MBS session.

NOTE 2:	Transfer of assistance information and shared delivery to additional RAN nodes has system impacts in SA2 scope.

#### 6.19.3.2	MBS service activation

Figure 6.19.3.2-1: MBS service activation

1.	When triggered according to step 11 of Figure 7.2.5.2-1 of TS 23.247 [4], AMF sends NGAP activation request message to NG-RAN nodes.

2.	Related RAN procedures. RAN nodes decides whether to deliver in mode for RRC inactive reception, and otherwise will perform RAN paging. For the delivery mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated, but remain in RRC-inactive state for multicast data reception.

3.	NG-RAN 1 determines that part of the RAN notification areas of UEs in MBS session is served by NG-RAN 2. It requests NG-RAN 2 to perform RAN paging for MBS session,

NOTE 1:	Depending on RAN decision, assistance information for MBS might be transferred in that step.

4.	If not yet done, the NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information (as defined in solution 1) for inactive reception in the shared delivery response.

5.	RAN internal procedures at NG-RAN 2 similar to step 2.

#### 6.19.3.3	Triggering MBS service announcement by MB-SMF (Option B)

NOTE 1:	Unlike the broadcast session start, the present procedure is only used to make the RAN nodes aware that an MBS multicast session enabling delivery for inactive rerception is ongoing. RAN nodes can then apply appropriate procedures to handle the MBS session, e.g. decide whether to transmit data and select the delivery mode

NOTE 2:	UEs still need to join the MBS multicast session as outlined in Clause 7.2.1.3 of TS 23.247 [4].

￼

Figure 6.19.3.3-1: Triggering MBS service announcement by MB-SMF (Option B)

1.	Multicast MBS session suitable for delivery mode for inactive reception is created at MB-SMF. The MB-SMF determines whethert an MBS session is suitable for inactive reception either based on an indication whether the delivery mode for inactive reception is enabled received from the AF (see solution 1) or based on the QoS oft he MBS session.

2-5.	MB-SMF informs RAN nodes in service area of multicast session via AMFs of multicast session and asks them to announce multicast session. The MB-SMF may defer step 2 until a Nmbsmf_MBSSession_ContextStatusSubscribe request is received from an SMF. In step 4, AMFs only select NG-RAN nodes in service area supporting delivery mode for RRC_inactive reception.

6. RAN nodes decide delivery mode for MBS session in a cell and whether to deliver data.

7.	RAN nodes may establish shared delivery as outlined in Clause 7.2.1.4 of TS 23.247 [4].

8.	RAN related procedures, e.g. indication of delivery mode or delivery of data.

Editor's note:	The steps 6 and 8 are to be decided by RAN WGs.

#### 6.19.3.4	MBS session release (Option A and B)

￼

Figure 6.19.3.3-1: MBS session release

1.	The procedures in Figure 7.2.2.2-1 of TS 23.248 are executed.

2.	After a grace period, the MB-SMF checks if shared delivery is still established towards RAN nodes. If so, it sends a request to each related AMF to release the MBS session

3.	The AMF determines stored RAN nodes for the MBS session and forwards the request to those RAN nodes.

### 6.19.4	Impacts on services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused.

In addition to impacts for solution 1:

NG-RAN:

-	Decide transmission mode for a multicast session in a cell.

-	Receive assistance information either aspart of request for service announcement or within shared delivery Establishment.

UE:

-	Receive MBS data in indicated transmission mode while MBS data is active.

MB-SMF:

-	inform RAN nodes in service area of multicast session (Option B).

-	provide assistance information as part of shared delivery establishment.

-	inform RAN nodes of MBS session release