3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205931r02

17 – 26 August 2022, Electronic

# Title:	LS on Monitoring of network time synchronization and relevant parameters

# Release:	Rel-18

# Work Item:	FS_5TRS_URLLC

Source:                 3GPP SA WG2

To:                           ITU-T Study Group 15

Study group contact email, tsbsg15@itu.int

Glenn Parsons, ITU-T SG15 Chairman, glenn.parsons@ericsson.com

Steffano Ruffini, Q13/15 Rapporteur, stefano.ruffini@ericsson.com

Cc:

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TR 23.700-25 version 0.3.0

https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3995

1. Overall Description:

As part of the Rel-18 FS_5TRS_URLLC study that is ongoing in 3GPP SA2, 3GPP intends to support the monitoring and reporting for timing synchronization status in the 5G system. The goal is to learn the status of the 5GS time source (e.g., degradation, failure, improvement, recovery) to evaluate if time synchronization services already in place for the Users Equipment (UEs) are impacted or not (e.g., whether the requested time synchronization error budget for a given UE can or cannot be fulfilled).

Considering the different potential synchronization architectures a 5G network could have, the 5G System may have available multiple sources of time/frequency like GNSS signal, Synchronous Ethernet (SyncE), PTP transport network (with Full Timing Support, Partial Timing Support, Assisted FTS, Assisted PTS), PPS input, etc.

SA2 is also discussing which clock quality information may be needed for a UE and/or a device attached to the UE to compare the quality of the 5GS clock against the quality of time information from other clocks that may be locally available to the UE/devices attached to the UE (e.g., a PTP-based clock or GNSS-based timing information).

Please note that the only clock quality information that can be currently provided to the UE using SIB/RRC according to 3GPP TS 38.331 clause 6.3.2 consists of:

- an indication whether the time indicates GPS time;

- uncertainty information, which indicates the uncertainty of the 5GS time as multiples of 25ns.

SA2 discussed about providing additional information such as traceability, clock synchronization state and frequency stability information to the UE.

In this context, 3GPP SA2 would like to kindly request ITU-T Study Group 15 for feedback on the following questions:

- What type of information about the 5GS time source are relevant to determine if time synchronization services already in place for the Users Equipment (UEs) are impacted or not?

- What

- information made available by the 5GS to the UE would be sufficient to enable a UE or device attached to the UE to compare the quality and accuracy of the 5GS clock against the quality of other clocks that may be locally available to the UE/devices attached to the UE?

2. Actions:

To ITU-T SG15

3GPP SA WG2 kindly requests ITU-T SG15 to provide answers for the questions raised above.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153E   	17th October to 26th October 2022   Electronic Meeting

TSG-SA2 Meeting #154   	14th November to 18th November 2022, TBD