3GPP SA WG2 Meeting #152E	S2-2205937r01

e-meeting, 17-26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusions for KI#3 for Congestion control

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes conclusions and way forward for KI#3 in TR 23.700-60

# Discussion

Proposes conclusion for KI#3.

# Proposal

*** Start of changes ***

# 8	Conclusions

## 8.x 	Key issues #3: 5GS information exposure for XR/media Enhancements

The following bullet points summarize the principles for the way forward to support congestion information exposure:

- 5G System uses ECN marking for the purpose of Low Latency, Low Loss and Scalable Throughput services (L4S [37], [38]) for uplink or downlink QoS Flows.

- NG-RAN or PSA UPF (based on congestion information reported from NG-RAN via GTP-U header) supports ECN marking for the purpose of L4S for uplink and downlink.

NOTE: Estimation of congestion information is up to RAN implementation.

- 5G System also supports API based exposure of congestion information towards AF as following:

  - The following information can be exposed when congestion happens:

    - QNC for GBR QoS Flow;

  - AF uses Nnef_AFSessionWithQoS to subscribe the above exposure to NEF/PCF, same as local exposure mechanism defined in TS 23.548[3].

  - Threshold may be provided by AF and sent to RAN/UPF via PCF/SMF. RAN/UPF judges whether the latest network congestion level should be reported based on the threshold.

  - Exposure path of Network Exposure defined in clause 6.4 of TS 23.548[3] is reused with extensions of GTP-U header and UPF/L-NEF services to exposure the above information.

  - Exposure path of RAN/UPF reporting congestion information via SMF/PCF/NEF can be supported.

*** End of changes ***