SA WG2 Meeting #S2-152E	S2-2205943

17 – 26 August, 2022, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	evaluation and interim conclusions for key issue 2

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: This paper proposes evaluation and interim conclusions for key issue 2

2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change (all new)***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	Key issue 2

8.X.1	Interim Conclusions

Editor's note:	RAN WGs will determine the feasibility of radio resource utilization optimization.

For conclusions, the following aspects will be considered:

-	For solutions where the broadcast MBS session is established multiple times towards a NG-RAN node, the NG-RAN node shall be able to identify the same MBS service if multiple broadcast MBS sessions established from different PLMNs and avoid multiple deliveries over radio.

-	A solution compatible with Rel-17 UEs is preferred.

The following interim conclusions will be taken into account:

- It should be possible to not establish all the shared delivery tunnels to the same NG RAN for the MBS sessions from different PLMNs for the same MBS service, or to establish one delivery tunnel of the MBS session to the same NG-RAN.

- The solution should support scenario where all RAN nodes are shared by PLMNs and the scenario where only part of the RAN nodes is shared by PLMNs.