SA WG2 Meeting #S2-152E	S2-2206025

17-26 August 2022, Electronic Meeting

Title:	[Draft] LS on ProSe Authorization information related to UE-to-UE Relay operation to NG-RAN

Response to:	-

Release:	Rel-18

Work Item:	FS_5G_ProSe_Ph2, NR_SL_relay_enh

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact Person:

Name:	LaeYoung Kim

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1	Overall description

SA2 is progressing Rel-18 FS_5G_ProSe_Ph2 in TR 23.700-33.

During the study, SA2 discussed the aspect on ProSe Authorization information related to UE-to-UE Relay operation sent from AMF to NG-RAN. This aspect is related to one of objectives for UE-to-UE Relay listed in Rel-18 NR_SL_relay_enh WID, i.e. "ii. Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3]".

The NR_SL_relay_enh WID reads that this aspect is up to SA2 and SA2 thinks that it is technically feasible to deliver the authorization information related to UE-to-UE Relay operation from AMF to NG-RAN as part of the NGAP message. However, SA2 has not concluded on how such information can be used for U2U Relay operation in NG-RAN.

Because how NG-RAN operation is performed to support UE-to-UE Relay operation, e.g. applying the network scheduled operation mode is within RAN2 remit and NGAP/XnAP is within RAN3 remit

, SA2 believes that coordination with RAN WGs is needed to make a decision on this aspect.

Regarding UE-to-UE Relay operation, it can be considered that "5G ProSe authorised" information sent by the AMF to NG-RAN may include one or more of the following:

1)	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

2)	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-UE Relay;

3)	whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE;

4)	whether the UE is authorized to act as a 5G ProSe Layer-3 U2U UE.

Please note that "U2U UE" corresponds to "source UE" and "destination UE", and the terms related to UEs that are involved in UE-to-UE Relay operation are under discussion and have not been finalized in SA2.

SA2 Question 1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation?

SA2 Question 2: If the answer to Q1 is yes, which bullet(s) need to be included?

2. Actions:

To RAN2 and RAN3:

ACTION: 	SA2 kindly asks RAN2 and RAN3 to provide feedback on the above questions.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153E	10 – 17 October 2022	e-meeting

3GPP TSG SA2#154	14 – 18 November 2022	TBD, Canada