SA WG2 Meeting #S2-152E	S2-2206044r01

17 - 26 August, 2022, E-Meeting

Source: 	NEC

Title: 	KI#3, New Sol: Providing weighted traffic offload policy for a set of UEs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_eNPN_ph2 / Rel-18

Abstract of the contribution: This paper proposes a solution to address KI#3: Policies for finer granular sets of UEs.

Discussion

As described in the KI#3, TR 23.700-08, this contribution focuses on impacts to 5GS needed to support providing traffic offload policy for such a set of UEs. In the context, the traffic offload is executed gradually from the current condition to the targeted condition. It starts with a lower weight and then gradually increase the weight by repeating the traffic offload operations. A new information needs to be studied for the objective.

---

5.3	KI#3: Policies for finer granular sets of UEs

5.3.1	Description

This key issue investigates the potential need and solutions for supporting offload policies for more granular sets of UE(s).

This key issue will study the following aspects:

-	how to identify set of UEs at a finer granularity that are associated with a dedicated offload policy, and how to express the set of UE in the offload policy;

-	impacts to 5GS needed to support providing traffic offload policy for such a set of UEs.

Proposal

Add the following solution to TR 23.700-48.

* * * First Change * * * *

6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 22 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 26 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 29 | X |  |  |  |  |  |  |
| 04: Support EAS (re-)discovery in VPLMN via HR PDU Session | 32 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU Session | 38 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 40 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 42 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 43 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 45 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 47 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 49 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 52 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 56 |  | X |  |  |  |  |  |
| 14: Group Management | 58 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 62 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 70 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 73 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 75 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 79 |  |  |  | X |  |  |  |
| 20: Global EASDF | 81 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 83 |  |  |  |  | X |  |  |
| 22: EAS discovery for federated Ops | 84 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 90 |  |  |  |  | X |  |  |
| 24: Reuse Option D after UL-CL insertion | 92 | X |  |  |  |  |  |  |
| 25: EAS discovery in VPLMN via V-EASDF for a HR PDU Session | 94 | X |  |  |  |  |  |  |
| 26: SM Policy for HR Session Breakout in VPLMN | 97 | X |  |  |  |  |  |  |
| 27: EAS discovery with dynamic setup of a LBO PDU Session | 100 | X |  |  |  |  |  |  |
| 28: Support edge computing in Roaming | 102 | X |  |  |  |  |  |  |
| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |  |  |  |  |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |  |  |  |  |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |  |  |  |  |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |  |  |  |  |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |  |  |  |  |
| 34: Selecting the same EAS/DNAI for collection of UEs | 116 |  |  |  | X |  |  |  |
| 35: Providing dedicated (re)location information as traffic routing information | 119 |  |  |  | X |  |  |  |
| 36: Providing dedicated (re)location information as EAS Deployment information | 123 |  |  |  | X |  |  |  |
| 37: (Re)location of same EAS and coordination across UEs | 126 |  |  |  | X |  |  |  |
| 38: EAS Discovery for EHE shared with other PLMN | 128 |  |  |  |  | X |  |  |
| 39: Support EAS relocation of inter-PLMN | 129 | X |  |  |  | X |  |  |
| 40: EAS discovery for shared EHE | 133 |  |  |  |  | X |  |  |
| 41: Controlling non-3GPP access of EC traffic via URSP and ATSSS | 136 |  |  |  |  |  | X |  |
| 42: Network-guided EC traffic switching | 139 |  |  |  |  |  | X |  |
| 43: Network-based solution for keeping EC traffic on 3GPP Access | 141 |  |  |  |  |  | X |  |
| 44: EAS traffic switching avoidance | 144 |  |  |  |  |  | X |  |
| 45: Application selected PDU Session | 146 | X |  |  |  |  | X |  |
| 46: Avoid UE switching on-going EC traffic away from 3GPP access | 147 |  |  |  |  |  | X |  |
| 47: Avoiding Switch Away Based on an SMF Indication | 149 |  |  |  |  |  | X |  |
| 48: Avoiding Switch Away Based on an Indication in the URSP | 150 |  |  |  |  |  | X |  |
| 49: URSP based solution to avoid UE to switch away from Edge PDU Session | 151 |  |  |  |  |  | X |  |
| 50: Obtain and maintain mapping table between IP address/IP range with DNAI | 152 |  |  |  |  |  |  | X |
| 51: EDI holding the IP address to DNAI mapping | 154 |  |  |  |  |  |  | X |
| 52: AF obtaining target DNAI provided by NEF | 155 |  |  |  |  |  |  | X |
| X: Providing weighted traffic offload policy for a set of UEs | X |  |  | X |  |  |  |  |

* * * Second Change * * * *

6.x	Solution XX (KI#3): Providing weighted traffic offload policy for a set of UEs

6.x.1	Introduction

In general, instance costs for EC services varies as time passes. Expensive instances for EC services at certain time may be supported by other lower cost instances with the same performance in another year. Application service provider or the operator supporting the EC services may consider to shift their EC services supported in another environment for certain users only under certain conditions and offload their traffic to the new edge hosting environment. In another case, reserved instances are deployed for a limited group of users and the rest of the users may be supported by on-demand or spot instances to reduce the total cost of the EC services.

Like these cases, the AFs can provide a request to support some offload policies only for certain set of UE(s) following a set of specific criteria such as Target UE Identifier(s), Spatial Validity Condition, Temporal Validity Condition specified in Table 5.6.7-1, TS23.501[x].

N6 Traffic Routing requirements is used to specify how the traffic to be offloaded is routed to the targeted edge hosting environment corresponding to each DNAI. As shown in Figure 6.x.2-1, weighted traffic offload enables application service provider or the operator supporting the EC services to specify offload weights to targeted DNAIs connecting to edge hosting environments. The application service provider or the operator supporting the EC services starts the traffic offload from small traffic volume and then increase gradually the offload volume.

Figure 6.x.2-1 Functional description about weighted traffic offload policy for a set of UEs

6.x.3	Procedures

- N6 Traffic Routing requirements in the AF Request in Table 5.6.7-1, TS23.501 [2] is used to specify how the traffic is routed to the targeted edge hosting environment corresponding to each DNAI.

A new information, weighted traffic offload, enables application service provider or the operator supporting the EC services to specify offload weights to targeted DNAIs connecting to edge hosting environments.

- To create a new request, the AF consumes Nnef_TrafficInfluence service exposed by the NEF. The content of the AF request is sent to the NEF and then stored in UDR by the NEF.

- In the UDR, the status change about the stored AF request triggers a Nudr_DM service to notify the event to the PCF(s). To be notified, the PCF(s) have subscribed to Nudr_DM service.

- The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl service operation for notification.

AF influenced Traffic Steering Enforcement Control information in Policy and charging control rule includes N6 traffic routing information per DNAI as described in Table 6.3.1, TS23.503 [13]. The weighted traffic offload information per DNAI is included in the N6 traffic routing information.

- When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to reconfigure the User plane of the PDU Session.

6.x.4	Impacts on services, entities, and interfaces

AF:

- Ability to provide weighted traffic offload information in AF Request that describes how the traffic to be offloaded is routed to the targeted edge hosting environment corresponding to each DNAI.

NEF:

- Ability to handle the weighted traffic offload information in AF Request via Nnef_TrafficInfluence service

PCF:

- Ability to provide weighted traffic offload information in AF influenced Traffic Steering Enforcement Control information that describes how the traffic to be offloaded is routed to the targeted edge hosting environment corresponding to each DNAI.

SMF:

- Ability to accept the weighted traffic offload information in N6 traffic routing information via Npcf_SMPolicyControl service