3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206049

# Elbonia, August 17 – 26, 2022	(revision of S2-2204314, 2687)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0109 | rev | 3 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of PCC procedure for 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The currently described PCC procedures for 5MBS are unnecessarily complicated. It is however possible to simplify and streamline them more, so that the behavior of NEF, AF and MB-SMF is closer to the one in the 5MBS procedures without PCC. The NEF – MB-PCF interaction can be moved up in the procedure so that an early check/authorization of the AF request by the MB-PCF becomes possible. When the MB-SMF is subsequently interacting with the MB-PCF, the MB-PCF has already the MBS session information available and can therefore directly provide the PCC rules in the response. With this, the number of interactions between MB-SMF and MB-PCF can be reduced. Furthermore, the early NEF – MB-PCF interaction has the advantage that, in case of a MB-PCF authorization failure (which should be more likely when many different AFs ask for the creation of an MBS session), all of the CN internal interactions (i.e. NEF to MB-SMF and MB-SMF to MB-PCF) are saved. Furthermore, for location dependent MBS and when the same MB-PCF is used, the MBS session information is only provided once to the MB-PCF by the NEF instead of repeatedly by every involved MB-SMF. All procedures (i.e. MBS Session Creation, Update and Deletion) can be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following updates and changes are proposed for the MBS session management procedures: - The structure of the PCC procedures for 5MBS are changed so that the NEF – MB-PCF interaction happens before the NEF – MB-SMF interaction. - The PCC procedures for 5MBS are further simplified by reducing the number of MB-SMF – MB-PCF interactions. - The parameters provided by the AF are now listed individually while a couple of them are described in a combined way as “MBS session information”. - The service operations are also updated and aligned with respect to the parameters (e.g. clause 9.2.3.1 is extended by a description of the content of the “MBS session information”). - A new service operation is added (9.2.2.x Npcf_MBSPolicyControl_Update service operation) to enable the MB-SMF to interact with the MB-PCF during the Update procedure and to inform about Policy Control Request Trigger conditions that have been met. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessarily complicated PCC procedures for 5MBS leading to high standard maintenance and implementation efforts. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10, new 6.X, 7.1.1.2, 7.1.1.3, 7.1.1.5, 7.1.1.6, 7.1.1.7, 9.1.3.6, 9.1.3.7, 9.2.2, 9.2.3, 9.4.3.2, 9.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.10	Policy control for Multicast and Broadcast services

### 6.10.1	General

The policy and charging control framework as defined in TS 23.503 [7] applies to Multicast and Broadcast services in the following aspects:

-	MBS Session binding: MBS Session binding is the association of an AF Session information to one and only one MBS Session. The PCF shall perform the session binding based on the MBS Session ID, i.e. TMGI or source specific IP multicast address.

-	QoS Flow binding: For an MBS Session, QoS Flow binding is the association of a PCC rule to a QoS Flow within an MBS Session. The MB-SMF performs QoS Flow binding for an MBS Session in the same way as the SMF for a PDU Session.

-	MBS policy information consists of:

-	PCC rules for MBS Session are used to provide policy for QoS flows: The following PCC rule parameters defined in Table 6.3.1 of TS 23.503 [7] are applicable for MBS:

-	Rule identifier.

-	Service data flow detection: Precedence, Service data flow template (only for IP PDU traffic).

-	Policy Control: 5G QoS Identifier (5QI), DL-maximum bitrate, DL-guaranteed bitrate, ARP, Priority Level, Averaging Window, Maximum Data Burst Volume.

-	Policy information can also be applicable for an entire MBS session. The following parameters defined for a PDU session in Table 6.4.1 of TS 23.503 [7] are applicable for an entire MBS session:

-	Authorized Session-AMBR.

-	Explicitly signalled QoS Characteristics.

-	Policy Control Request Triggers for MBS Session are used to define the conditions when the MB-SMF shall interact again with the PCF to request an update of the policy information for the MBS session by providing information on the condition(s) that have been met. The following Policy Control Request Triggers are defined for MBS:

-	MBS Session Update.

### 6.10.2.	MBS Session policy control data in UDR

The policy control profile information provided by the UDR at MBS Session establishment, using Nudr service for Data Set "Policy Data" and Data Subset "MBS Session policy control data" with the source specific multicast address used as MBS session ID or with AF Application identifier as data key is described in Table 6.10.2-1.

Table 6.10.2-1: MBS Session policy control subscription information

| Information element name | Description | Category |
| --- | --- | --- |
| 5QI(s) | Allowed 5QI(s) for a PCC rule of an MBS session (NOTE 1) | Optional |
| ARP | Maximum ARP for any PCC rule of an MBS session (NOTE 1) | Optional |
| Session-AMBR | Maximum Session-AMBR for all nonGBR QoS Flows of an MBS session (NOTE 1) | Optional |
| GBR | Maximum aggregated bitrate that can be provided across all GBR QoS Flows for an MBS session (NOTE 1) | Optional |
| NOTE 1: This information element may be used to decide whether to authorize received MBS session information. |  |  |

The policy subscription profile may be extended with operator-specific information.

# * * * * Second change * * * *

## 6.X	MBS Service information

Service information is used to describe and MBS session and is conveyed from AF to NEF, from AF/NEF/MBSF to the PCF, and from AF/NEF/MBSF to the MB-SMF

The MBS service information consists of an optional AF Application Identifier and the description of one or more data flows/media components. For each data flow/media component, the following information may be provided:

-	Flow description;

-	Media information (Media type, Media format);

-	QoS requirements (5G QoS parameters (i.e. 5QI, ARP, GFBR, MFBR, Session-AMBR) or bandwidth requirements or QoS reference or individual QoS parameters); and

-	Priority indicator.

The following MBS service information is mandatory to be supported: 5G QoS parameters (i.e. 5QI, ARP, GFBR, MFBR, Session-AMBR) for one data flow/media component.
Additional MBS service information is optional to be supported and related support may be restricted to some reference points in a deployment. (e.g. AF to NEF, and AF/NEF/MBSF to PCF)

# * * * * Third change * * * *

#### 7.1.1.2	MBS Session Creation without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the MBS session creation procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session creation procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: MBS Session Creation without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the network deployment and use case, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, SSM, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request ([MBS Session ID], MBS service type, MBS service information, [TMGI allocation request], [MBS service area], [Any UE indication], [start and end time of the MBS session], [MBS session state], [ingress transport address request indication], [Request for location-dependent session], [FSA ID(s)]). If step 1-6 has not been executed before, the AF may provide an MBS Session ID containing an SSM or it may request that the network allocates an MBS Session ID (i.e., TMGI). The AF provides the MBS service type (i.e. either multicast service or broadcast service) and MBS service information (as defined in clause 6.X). The AF may provide the "Any UE indication" (indicating whether a multicast MBS session is "open to any UEs"), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, the AF request may also indicate that the allocation of an ingress transport address is requested and that the AF request is for a location dependent MBS service.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

For broadcast communication, the AF may determine MBS FSA ID(s) for the broadcast session based on business agreements and include them in the description of the MBS session.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects an MB-SMF as ingress control node, possibly based on MBS service area. If an MBS Session ID containing a TMGI was received in step 8, NEF/MBSF finds the MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request ([MBS Session ID], MBS service type, [TMGI allocation request], MBS service information (as defined in clause 6.X), [MBS service area], [Any UE indication], [start and end time of the MBS session], [MBS session state], [ingress transport address request indication], [FSA ID(s)]) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session. The NEF/MBSF forwards all parameters it has received from the AF in step 8. If the MBSF decides to insert an MBSTF into the user plane for the MBS session, it also indicates that the allocation of an ingress transport address is requested even if this was not requested in step 8. The request also includes the Any UE indication if provided in step 8.

If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

For broadcast communication, if no MBS FSA ID(s) have been received, the MB-SMF selects MBS FSA ID(s) for the broadcast session based on local configuration.

12.	If a source specific multicast address is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally under consideration of the MBS service information.

14.	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, SSM and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the SSM received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, service requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6.

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 11. For broadcast communication, the MB-SMF includes any MBS FSA ID(s) selected in step 11. It also indicates the success or failure of reserving transmission resources.

17.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1 to request the allocation of resources to for the transmission of the broadcast session.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 16 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the SSM received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is SSM, the NEF/MBSF provides the allocated TMGI. If AF requested the allocation of an ingress transport address, the message also includes the allocated ingress address. For broadcast communication, the message also includes any MBS FSA ID(s) received in step 17.

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

# * * * * Fourth change * * * *

#### 7.1.1.3	MBS Session Creation with PCC

Figure 7.1.1.3-1: MBS Session Creation with PCC

Steps 1 to 7 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1. to 9:	Same as in Figure 7.1.1.2-1. Step 1 may include an application ID.

10.	The NEF/MBSF decides about the interaction with the PCF based on local configuration.

If the NEF/MBSF decided to interact with the PCF, steps 11 to 19 are performed, and in step 20 and an indication that the PCF has to be contacted is provided and the MBS session information is not provided .

If the NEF/MBSF decided to not interact with the PCF, steps 12 to 19 are skipped, and MBS session information is provided to the MB-SMF in step 20.

11.	[Conditional] If the MB-PCF did not receive an MBS session ID from the AF in step 8, the NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (1) message to the MB-SMF and the MB-SMF allocates a TMGI and returns the TMGI to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI, expiration time).

12.	[Conditional] If the NEF/MBSF receives the Request for location-dependent session from the AF, the NEF/MBSF first uses the BSF Discovery service to discover whether there is a PCF serving the MBS session with the MBS session ID by using Nbsf_management_Discovery operation. If there is a PCF registered for the MBS session ID, the NEF/MBSF interacts with that PCF and skips the following step 14.

NOTE 1:	For a location dependent MBS service multiple AF requests can occur and all of them have to be handled by the same PCF to ensure a common authorization and QoS allowance check as well as identical policy information for the MBS session.

13. [Conditional] If step 12 was not executed or the interaction with the BSF revealed that no PCF is registered for the MBS session ID, the NEF/MBSF discovers the MB-PCF candidates by interacting with the NRF and selects an MB-PCF, possibly based on MBS service area.

14. The NEF/MBSF sends an Npcf_MBSPolicy_Authorization_Create Request (MBS session ID, MBS service information (as defined in clause 6.X), [MBS service area]) to the MB-PCF. The NEF/MBSF forwards the MBS service area if received from the AF in step 8.

15.	[Optional] The PCF may retrieve authorization information for the MBS session from the UDR (see clause 6.10.2) and takes it into account for the subsequent authorization and QoS allowance check.

16. The MB-PCF determines whether the request is authorized and if the request is authorized, the MB-PCF derives the required QoS parameters based on the received MBS session information and determines whether this QoS is allowed. If the required QoS is allowed, the MB-PCF generates the policy information for the MBS session (as defined in clause 6.10) and stores it together with the MBS session ID.

17.	If the request is authorized and the required QoS is allowed, the PCF registers at the BSF that it handles the MBS session by using Nbsf_management_Register Request (MBS Session ID, PCF ID). It provides an identifier that the policy association is for MBS and the MBS Session ID, its own PCF ID and optionally its PCF set ID.

18.	The MB-PCF sends an Npcf_MBSPolicy_Authorization_Create Response (Result indication) to the NEF/MBSF.

If the request is not authorized or the required QoS is not allowed, the MB-PCF indicates so in the response to the NEF/MBSF which in turn informs the AF about it (by sending the Nnef_MBSSession_Create Response) and ends this procedure.

19. Same as step 10 in Figure 7.1.1.2-1.

20. Same as step 11 in Figure 7.1.1.2-1 with the difference that the MBS service information is not present if the NEF/MBSF has decided to interact with the PCF and thus provided the MBS service information to the MB-PCF in step 14. In addition, the NEF/MBSF provides the MBS Session ID and if the NEF/MBSF decided to interact with the PCF an indication that the PCF has to be contacted.

21.	If the MB-SMF does not receive the indication that the PCF has to be contacted, the MB-SMF decides whether to interact with the PCF. Otherwise it continues with step 29 and derives the required QoS parameters locally under consideration of the MBS service information. Otherwise, the MB-SMF first uses the BSF Discovery service to discover whether there is a PCF serving the MBS session with the MBS session ID by using Nbsf_management_Discovery operation. If there is a PCF registered for the MBS session ID, the MB-SMF interacts with that PCF and skips the following step 22.

22. [Conditional] If the interaction with the BSF revealed that no PCF is registered for the MBS session ID, the MB-SMF discovers the PCF candidates by interacting with the NRF and selects an MB-PCF, possibly based on MBS service area.

23.	The MB-SMF sends an Npcf_MBSPolicyControl_Create Request (MBS session ID, [MBS service information (as defined in clause 6.X)], [MBS service area]) for the MBS session towards the PCF. . The MB-SMF forwards the MBS service information, and the MBS service area to the PCF if received from the NEF/MBSF in the previous step 20.

If the PCF does not receive MBS service information from the MB-SMF, and has previously determined policy information for the MBS session (see step 16) corresponding to the MBS session ID received from the MB-SMF, the PCF continues with step 28. Otherwise, the PCF performs the subsequent steps 25 to 27.

24.	[Optional] The PCF may retrieve authorization information for the MBS session from the UDR (see clause 6.10.2) and takes it into account for the subsequent authorization and QoS allowance check.

25. The PCF determines whether the request is authorized and if the request is authorized, the PCF derives the required QoS parameters based on the received MBS service information and determines whether this QoS is allowed. If the required QoS is allowed, the PCF generates the policy information for the MBS session (as defined in clause 6.10) and stores it together with the MBS session ID.

26.	If the request is authorized and the required QoS is allowed the PCF registers at the BSF that it handles the MBS session by using Nbsf_management_Register Request (MBS Session ID, PCF ID). It provides an identifier that the policy association is for MBS and the MBS Session ID, its own PCF ID and optionally its PCF set ID.

27.	The PCF responds with Npcf_MBSPolicyControl_Create Response ([policy information for the MBS session (as defined in clause 6.10)], Result indication).

If the request is not authorized or the required QoS is not allowed, the MB-PCF indicates so in the response to the MB-SMF which in turn informs the AF about it (by sending the Nmbsmf_MBSSession_Create Response) and ends this procedure.

28. Same as step 12 in Figure 7.1.1.2-1.

29-37	Same as steps 14-22 in Figure 7.1.1.2-1.

NOTE 3:	Steps 33-36 can be executed in parallel to step 32.

# * * * * Fifth change * * * *

#### 7.1.1.5	MBS Session Deletion with PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.5-1: MBS Session Deletion with PCC

1.-3.	Same as in Figure 7.1.1.4-1.

For the interaction with the PCF in this procedure, the NEF/MBSF applies the same decision that was taken in step 10 in Figure 7.1.1.3-1during the MBS Session Creation with PCC procedure:

If the NEF/MBSF decided to interact with the PCF, steps 4 to 5 are performed.

If the NEF/MBSF decided to not interact with the PCF, steps 4 to 5 are skipped.

4.	The NEF/MBSF sends an Npcf_MBSPolicyAuthorization_Delete Request to the PCF that handles the MBS Session.

5.	The PCF sends an Npcf_MBSPolicyAuthorization_Delete Response to the NEF/MBSF.

6.	Same as step 4 in Figure 7.1.1.4-1.

6.	(Alt-B) The NEF/MBSF sends an NMBSPolicyAuthorization_Delete Request to the PCF that handles the Policy of the MBS Session.

13.	The PCF de-registers at the BSF that it handles the MBS session.

7.	The MB-SMF sends the Npcf_MBSPolicyControl_Delete Request to request the deletion of the SM Policy Association with the PCF.

8. The PCF sends the Npcf_MBSPolicyControl_Delete Response to the MB-SMF.

9.	The PCF de-registers at the BSF that it handles the MBS session.

10-19.	Same as steps 5-14 in Figure 7.1.1.4-1.

# * * * * Sixth change * * * *

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MBS service area and/or MBS service information. Updating MBS service information may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before the MBS Session Update procedure is started or after the MBS Session Update procedure is completed.

Figure 7.1.1.6-1: MBS Session Update without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update MBS service information (as defined in clause 8.X), or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by sending an Nnef_MBSSession_Update Request (MBS Session ID, [MBS service information], [MBS service area], [MBS session state (active/inactive)]).

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

2.	NEF checks authorization of AF.

3.	NEF/MBSF sends Nmbsmf_MBSSession_Update Request to MB-SMF forwarding the updated information received from the AF in step 1.

4.	The MB-SMF derives any updated QoS parameters locally under consideration of the updated MBS service information. This may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

5.-6.	MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), 7.2.6 (for QoS updates and service area updates).

8.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

9.	MB-SMF responds to the NEF/MBSF with a Nmbsmf_MBSSession_Update Response.

10.	NEF/MBSF responds to the AF with a Nnef_MBSSession_Update Response.

# * * * * Seventh change * * * *

#### 7.1.1.7	MBS Session Update with PCC

For local multicast services and location dependent multicast services, the AF may perform a Service Announcement towards UEs to update the MBS service area before the MBS Session Update procedure is started or after the MBS Session Update procedure is completed.

Figure 7.1.1.7-1: MBS Session Update with PCC

1.-2.	Same as in Figure 7.1.1.6-1.

3.	For the interaction with the PCF in this procedure, the NEF/MBSF applies the same decision that was taken in step 10 in Figure 7.1.1.3-1during the MBS Session Creation with PCC procedure unless the MBS session update only relates to an activation or deactivation of the MBS session and thus no PCC interactions are required.

If the NEF/MBSF decided to interact with the PCF, steps 4 to 6 are performed and in step 7 an indication that the PCF has to be contacted is provided, and MBS service information is not provided.

If the NEF/MBSF decided to not interact with the PCF, steps 4 to 6 are skipped and MBS service information is provided to the MB-SMF in step 7.

4. 	NEF/MBSF sends an Npcf_MBSPolicy_Authorization_Update Request (application session context, [MBS service information], [MBS service area]) to the PCF forwarding the updated information received from the AF in step 1.

5. 	The PCF determines whether the update is authorized and if the update is authorized, the PCF derives the update for the QoS parameters based on the received MBS session information and determines whether this new QoS is allowed. If the new QoS is allowed, the PCF updates the policy information for the MBS session (as defined in clause 6.10) accordingly.

6. 	The PCF sends an Npcf_MBSPolicy_Authorization_Update Response (Result indication) to the NEF/MBSF.

If the update is not authorized or the new QoS is not allowed, the PCF indicates so in the response to the NEF/MBSF which in turn informs the AF about it (by sending the Nnef_MBSSession_Update Response) and ends this procedure.

7. 	The NEF/MBSF sends Nmbsmf_MBSSession_Update Request to the MB-SMF forwarding the updated information received from the AF in step 1.

If the NEF/MBSF has decided to interact with the PCF and thus provided the updated MBS service information to the PCF in step 4, an indication that the PCF has to be contacted is provided, but the updated MBS service information is not forwarded.

8. 	If the MB-SMF does not receive the indication that the PCF has to be contacted, the MB-SMF decides whether to interact with the PCF. If it decides not to interact with the PCF, it continues with step 12 and derives the update for the QoS parameters locally under consideration of the MBS session information. Otherwise, the MB-SMF sends an Npcf_MBSPolicyControl_Update Request (MBS Policy Association ID, [MBS service information], [MBS service area]) for the MBS session towards the PCF. The MB-SMF forwards the MBS service information and the MBS service area to the PCF if received from the NEF/MBSF in the previous step 7.

If the PCF does not receive MBS session information from the MB-SMF, the PCF identifies any updated policy information for the MBS session (as defined in clause 6.10) corresponding to the MBS session ID received from the MB-SMF and continues with step 11. Otherwise, the PCF performs the subsequent step 10.

9. 	The PCF determines whether the update is authorized and if the update is authorized, the PCF derives the update for the QoS parameters based on the received MBS session information and determines whether this new QoS is allowed. If the new QoS is allowed, the PCF updates the policy information for the MBS session (as defined in clause 6.10) accordingly.

10.	The PCF responds with Npcf_MBSPolicyControl_Update Response ([updated policy information for the MBS session], Result indication).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the MB-SMF which in turn informs the AF about it (by sending the Nmbsmf_MBSSession_Update Response) and ends this procedure.

11.-16.		Same as steps 5-10 in Figure 7.1.1.6-1.

# * * * * Eigth change * * * *

#### 9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location-dependent MBS session, the part of the MBS session within a service area. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI allocation request, MBS Service Type (multicast or broadcast).

Input, Optional: DNN, S-NSSAI, MBS service area, MBS activation time, MBS termination time, MBS service information (as defined in clause 6.X), Input Transport Address Request, session activity status (active/inactive), indication that the PCF has to be contacted. For a multicast session, indication that any UE may join. For a broadcast session, MBS FSA ID(s). For subscription to notifications event ID(s), Notification Target Address, Request for location-dependent session.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

# * * * * Nineth change * * * *

#### 9.1.3.7	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSSession_Update

Description: Update the established multicast session or broadcast session, or for a location-dependent MBS session, the part of the MBS session within a service area, e.g. QoS update.

Input, Required: MBS Session ID.

Input, Optional: MBS service information (as defined in clause 6.X), MBS service area, session activity status (active/inactive), for a broadcast session, MBS FSA ID(s), Area Session ID, indication that the PCF has to be contacted.

Output, Required: Result Indication.

Output, Optional: Cause, MBS FSA ID(s).

# * * * * Tenth change * * * *

### 9.2.2	Npcf_MBSPolicyControl service

#### 9.2.2.1	General

Service description: NF Service Consumer, e.g. MB-SMF, can create and manage a MBS Policy Association in the PCF through which the NF Service Consumer receives policy information for the MBS Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. MB-SMF, with policy information for the MBS Session that may contain (the details are defined in clause 6.10):

-	Policy information applicable for an entire MBS Session.

-	PCC rule information.

-	Policy Control Request Trigger information.

At MBS Session establishment the NF Service Consumer, e.g. MB-SMF, requests the creation of a corresponding MBS Policy Association with the PCF (Npcf_MBSPolicyControl_Create) and provides relevant parameters about the MBS Session to the PCF.

-	When the PCF has created the "MBS Policy Association", the PCF may provide policy information as defined above.

When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB-SMF, requests the update (Npcf_MBSPolicyControl_Update) of the MBS Policy Association by providing information on the condition(s) that have been met. The PCF may provide updated policy information for the MBS session to the NF Service Consumer.

The PCF may at any time provide updated policy information for the MBS session (Npcf_MBSPolicyControl_UpdateNotify).

At MBS Session Release the NF Service Consumer, e.g. MB-SMF, requests the deletion of the corresponding MBS Policy Association.

#### 9.2.2.2	Npcf_MBSPolicyControl_Create service operation

Service operation name: Npcf_MBSPolicyControl_Create

Description: The NF Service Consumer can request the creation of a MBS Policy Association and provide relevant parameters about the MBS Session to the PCF.

Inputs, Required: MBS session ID, DNN, S-NSSAI.

Inputs, Optional: MBS service information (as defined in clause 6.X), MBS service area.

Outputs, Required: Success or Failure. In case of Success, MBS Policy Association ID and Policy information for the MBS Session (as defined in clause 6.10).

Outputs, Optional:  In case of Failure, the service information that can be accepted by the PCF.

#### 9.2.2.3	Npcf_MBSPolicyControl_UpdateNotify service operation

Service operation name: Npcf_MBSPolicyControl_UpdateNotify

Description: Provides to the NF Service Consumer, e.g. MB-SMF, updated Policy information for the MBS Session.

Inputs, Required: MBS Policy Association ID.

Inputs, Optional: Policy information for the MBS Session (as defined in clause 6.10).

Outputs, Required: Success or Failure.

Outputs, Optional: None.

#### 9.2.2.4	Npcf_MBSPolicyControl_Delete service operation

Service operation name: Npcf_MBSPolicyControl_Delete

Description: The NF Service Consumer (i.e. MB-SMF) can request the deletion of the MBS Policy Association and of the associated resources.

Inputs, Required: MBS Policy Association ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

#### 9.2.2.x	Npcf_MBSPolicyControl_Update service operation

Service operation name: Npcf_MBSPolicyControl_Update

Description: The NF Service Consumer can request an update of the MBS Policy Association.

Inputs, Required: MBS Policy Association ID.

Inputs, Optional: MBS service information (as defined in clause 6.X), MBS service area, Information on the Policy Control Request Trigger condition that has been met.

Outputs, Required: Success or Failure. In case of Success, Policy information for the MBS Session (as defined in clause 6.10).

Outputs, Optional: In case of Failure, the service information that can be accepted by the PCF.

# * * * * Eleventh change * * * *

### 9.2.3	Npcf_MBSPolicyAuthorization Service

#### 9.2.3.1	General

Service description: This service is to authorise an AF / NEF / MBSF request for an MBS service and to create policies as requested by the authorized AF for the MBS Service. This service also allows the NF consumer to subscribe/unsubscribe the notification of events.

#### 9.2.3.2	Npcf_MBSPolicyAuthorization_Create service operation

Service operation name: Npcf_MBSPolicyAuthorization_Create

Description: Authorize the request, and optionally determines and installs MBS Policy Control Data according to the information provided by the NF Consumer.

Inputs, Required: MBS session ID,.

Inputs, Optional: MBS service area, DNN if available, S-NSSAI if available, MBS service information (as defined in clause 6.X),  .

Outputs, Required: Success (application session context) or Failure (reason for failure).

Outputs, Optional: In case of Failure, the service information that can be accepted by the PCF.

#### 9.2.3.3	Npcf_MBSPolicyAuthorization_Update service operation

Service operation name: Npcf_MBSPolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: MBS service information (as defined in clause 6.X), MBS service area.

Outputs, Required: Success or Failure (reason for failure).

Outputs, Optional: In case of Failure, the service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_MBSPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

#### 9.2.3.4	Npcf_MBSPolicyAuthorization_Delete service operation

Service operation name: Npcf_MBSPolicyAuthorization_Delete

Description: Provides means for the NF Consumer to delete the context of application level session information.

Inputs, Required: Identification of the application session context.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

# * * * * Twelth change * * * *

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location-dependent MBS session, the part of the MBS session within a service area. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI allocation request, MBS Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be Cell ID list and/or TAI list, or geographical area information, or civic address information), MBS start time, MBS stop time, MBS service information (as defined in clause 6.X), Input Transport Address Request, session activity status (active/inactive), for subscription to notifications notification target address. For a multicast session, indication that any UE may join the multicast session. Request for location-dependent session. For a broadcast session, MBS FSA ID(s).

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

# * * * * Thirteenth change * * * *

#### 9.4.3.3	Nnef_MBSSession_Update service operation

Service operation name: Nnef_MBSSession_Update

Description: This service is used by the NF service consumer to update the established multicast session or broadcast session, or for a location-dependent MBS session, the part of the MBS session within a service area, e.g. QoS update. For multicast MBS Session, this service is also used to activate or deactivate the MBS Session.

Input, Required: MBS Session ID.

Input, Optional: MBS service information (as defined in clause 6.X), MBS service area, session activity status (active/inactive), Area Session ID. For a broadcast session, MBS FSA ID(s).

Output, Required: Result Indication.

Output, Optional: Cause, MBS FSA ID(s).

# * * * * End of changes * * * *