3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206079

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] LS on FS_5MBS_Ph2 progress

Response to:	-

Release:	Release 18

Work Item:	FS_5MBS_Ph2, NR_MBS_enh

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:	RAN1

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

The study of FS_5MBS_Ph2 in SA2 is starting the evaluation and conclusion phase. The key issues and candidate solutions are documented in the latest version of TR 23.700-47 (link).

Mapping of Solutions to Key Issues can be found in clause 6.0 of TR 23.700-47. KIs (#1, #2 and #6) in TR 23.700-47 are related to RAN WGs.

To finish the study, SA2 would like to ask RAN2 and RAN3 give feedback for the following aspects:

- SA2 assumes that it is an NG-RAN decision to select the mode how to deliver MBS data, and whether to transition UEs in an MBS session to RRC Inactive state. SA2 is discussing "assistance information" that can be provided by the core network (possibly based on input from the AF) to assist NG-RAN in those decisions:
Some UEs might be not suitable sent to RRC Inactive state (e.g., UEs of likely talkers in an MBS session). Further, SA2 is discussing whether AFs can recommend not to apply the delivery mode for inactive reception for MBS sessions which are particularly sensitive for packet loss, and/or can provide priority information of MBS session enabling RAN nodes to select the delivery node among MBS sessions. SA2 would like to check with RAN WG on whether that the existing QoS parameters of MBS QoS flows can be used by NG RAN to select the mode how to deliver MBS data. To this end, SA2 assumes a set of parameters (i.e., assistant information) are needed to be provided to RAN nodes..

In addition, SA2 assumes that backward compatibility with Rel-17 UEs should be provided and that NG RAN will thus need to know whether the UEs it serves have the Rel-18 MBS capability to receive multicast in RRC_Inactive state. SA2 would like to ask if the UE radio capability provided directly from UE to NG-RAN will contain the information whether the UE supports Rel-18 MBS capability.

SA2 also discusses whether information on whether the UE prefers to receive MBS data in connected or inactive mode is required (e.g. to enable a UE to save power while in inactive state or to achieve more reliable transmission) and if so, whether this information can be directly transferred from UE to NG RAN, and would welcome related feedback from RAN.

- Upon MBS session activation, the UEs that have previously joined the MBS session and are in RRC Inactive state, may either be kept in RRC Inactive state, or be transitioned to RRC Connected state to receive the MBS session data, depending on NG RAN decision. SA2 assumes that it will continue to inform RAN nodes about MBS session activation to enable NG RAN to send appropriate signalling as determined by RAN groups to such UEs. It is understood that IDLE UE will need to transition to connected state to start receiving the MBS data and CN initiated group paging (as defined in Rel-17) is thus still required for such UEs; it is assumed that RRC Inactive UEs could also receive this CN initiated group paging. Hence when an RRC inactive UE receives the CN-initiated group paging or RAN signalling related to the activation of the MBS session , SA2 assumes that RAN will define appropriate signalling to inform the UE whether it need go back to RRC connected state to receive the MBS session data.

- Regarding the mobility within RNA, the UE in RRC Inactive state should be able to receive DL multicast MBS data within its RNA. Currently SA2 is discussing the following alternatives:

  - If the UE moves to a cell within the RNA where no data of the MBS session are transmitted, UE sends RRC signalling (to be determined by RAN;) that trigger the NG-RAN node to obtain the UE context (including information that the UE is participating in which MBS session). NG RAN then establishes the shared tunnel for the multicast session with the MB-SMF and obtains additional information about the multicast session within it. .

  - When UE(s) in an MBS session transition to RRC_inactive state, Anchor NG-RAN node informs  neighbour NG-RAN nodes (within the RNA of the UE and the MBS service area) that transmission of the MBS session in that area is required and thus triggers the neighbour NG-RAN nodes to establish the shared tunnel for the multicast session with the MB-SMF and obtains additional information (including assistance information) about the multicast session within it, and to decide whether and how to deliver data for the MBS session.

  - When UE(s) in an MBS session transition to RRC_inactive state, Anchor NG-RAN node informs  neighbour NG-RAN nodes (within the RNA of the UE and the MBS service area) that transmission of the MBS session in that area is required and also forwards MBS data to be transmitted to those RAN nodes.

The differences of the alternatives are related to how and when to provide information about the MBS session to other RAN nodes within the RNA, e.g. before and after the UE moves to a new cell within the RNA, and how to transfer MBS data to other RAN nodes This also includes how to manage the shared tunnel in other RAN nodes., e.g. when the shared tunnel can be released.

- Regarding the MOCN RAN sharing for broadcast, SA2 has several alternatives and they assume MOCN RAN nodes can identify the same MBS services by the information provided by 5GC. SA2 considers backward compatibility with Rel-17 UEs as important.  SA2 would like to know whether there are any Radio impact amongst the solutions for KI#2 from RAN perspective, and would welcome RAN feedback.

Progress and further details can be seen in TR 23.700-47, and SA2 may further request the feedback of other RAN-related issues in the TR in future meetings.

2. Actions:

To RAN2, RAN3:

ACTION: 	SA2 kindly asks RAN2 and RAN3 take the above information into consideration and provide their views on the above aspects, if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153e   	Oct 10 – 14, 2022 				Elbonia

TSG-SA2 Meeting #154   	Nov 14 – 18, 2022 				Canada, CA