3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206079r07

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

# Title:	[DRAFT] LS on FS_5MBS_Ph2 progress

# Response to:	-

# Release:	Release 18

# Work Item:	FS_5MBS_Ph2, NR_MBS_enh

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:	RAN1

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

The study of FS_5MBS_Ph2 in SA2 is starting the evaluation and conclusion phase. The key issues and candidate solutions are documented in the latest version of TR 23.700-47 (link).

Mapping of Solutions to Key Issues can be found in clause 6.0 of TR 23.700-47. KIs (#1, #2 and #6) in TR 23.700-47 are related to RAN WGs.

SA2 kindly requests RAN2 and RAN3 for feedback on the potential solutions and areas pointing to RAN WGs dependency in TR 23.700-47 to facilitate SA2 reaching conclusions on these key issues, below are some concrete questions:

- SA2 understands that it is NG-RAN decision on how to deliver MBS data to the UEs and whether to transition UEs receiving MBS data in an MBS session to RRC Inactive state. SA2 is discussing "assistance information" that can be provided by the core network (possibly based on input from the AF) to assist NG-RAN in those decisions.

Some UEs might not be suitable to be sent to RRC Inactive state (e.g., UEs of "privileged" users in a multicast group). Further, SA2 is discussing whether AFs can recommend not to enable the function in NG-RAN for inactive reception for MBS sessions which are particularly sensitive for packet loss. SA2 would like to check with RAN WG on whether the existing QoS parameters of MBS QoS flows can be used by NG RAN to decide how to deliver the MBS data..

In addition, SA2 assumes that backward compatibility with Rel-17 UEs should be ensured and that NG RAN will thus need to know whether the UEs it serves have the Rel-18 MBS capability to receive multicast in RRC_Inactive state. SA2 would like to ask if the UE radio capability provided directly from UE to NG-RAN will contain the information whether the UE supports Rel-18 MBS capability.

- SA2 assumes, upon MBS session activation, the UEs that have previously joined the MBS session and are in RRC Inactive state, may either be kept in RRC Inactive state, or be transitioned to RRC Connected state to receive the MBS session data, depending on NG RAN decision. The core network will continue to inform RAN nodes about MBS session activation to enable NG RAN to send appropriate signalling to the UEs in the multicast group. SA2 would like to clarify with RAN WGs whether the assumption that IDLE UE will need to transition to connected state to start receiving the MBS data and CN initiated group paging (as defined in Rel-17) is thus still required for such UEs.

- Regarding the mobility within RNA, SA2 assumes the UE in RRC Inactive state should be able to receive DL multicast MBS data within its RNA and the solution will be determined by RAN WGs as RRC_INACTIVE mobility is under the remit of RAN WGs.:

- Regarding the MOCN RAN sharing for broadcast, SA2 has several alternatives for this key issue#2. Some solutions assume MOCN RAN nodes can identify the same MBS services by the information provided by 5GC while some solutions can identify the MBS service is for MOCN RAN nodes based on configuration. SA2 considers backward compatibility with Rel-17 UEs as important.

SA2 is discussing whether it is feasible to use a single TMGI with additional MOCN flag or different TMGIs with additional identifier for multiple MBS broadcast sessions transferring the same content for different PLMNs.

SA2 also would like to know whether there is any radio impact amongst the solutions for KI#2 from RAN perspective and would welcome RAN feedback.

Progress and further details can be seen in TR 23.700-47, and SA2 may further request the feedback of other RAN-related issues in the TR in future meetings.

2. Actions:

To RAN2, RAN3:

ACTION: 	SA2 kindly asks RAN2 and RAN3 take the above information into consideration and provide their feedback.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153e   	Oct 10 – 14, 2022 				Elbonia

TSG-SA2 Meeting #154   	Nov 14 – 18, 2022 				TBD