3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206086

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion of KI#5 of FS_5MBS_Ph2

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: Provides the conclusion of KI#5.

# 1. Introduction

S2-2205388 (FS_5MBS_Ph2 Status Report) lists the work plan for the future meetings as follows:

-	SA2#152, Aug (2TUs): Solutions, evaluations, conclusions;

-	SA2#153, Oct (1TU): evaluations, conclusions: Approval of MBS_Ph2 WID;

-	SA2#154, Nov (0.5 TU): conclusions: Adjustment/issues depends on RAN progress.

SA2#152E meeting would be the first meeting for evaluation/conclusion, and the last meeting for having new solutions.

KI#5 is about Coexistence with existing power saving mechanisms for capability-limited devices, and the description is excerpted from TR 23.700-47 as follows:

## 5.5 Key Issue #5: Coexistence with existing power saving mechanisms for capability-limited devices ### 5.5.1 Description Capability-limited devices may use power-saving mechanisms to extend their battery live. Existing power saving mechanisms include MICO (Mobile Initiated Connection Only) mode, DRX (Discontinuous Reception), eDRX (Extended Discontinuous Reception). MBS content should be transmitted to all devices at the same time to save transmission resources. However, the existing power saving mechanisms may prevent devices from receiving MBS content (for instance group messages). This KI will study the following issue: - Whether and how to support MBS content (for instance group message) delivery for capability-limited devices by considering coexistence of existing power saving mechanisms and MBS. NOTE: In SA4, co-existence between power saving mechanism and eMBMS has been studied in TR 26.850 [5]. The study result in TR 26.850 [5] could be taken into consideration during the solution study of this KI.

Totally 3 solutions were proposed in previous meetings and they in general address the following aspects:

-	Way of Inform UE the scheduled time: addressing the way of informing UE when to start to receive the MBS session data.

-	Granularity of the scheduled time: In the solutions, the granularity could be the DRX-level time information, or SDP level, which could be in seconds.

-	5GC enhancement: Whether the solution needs to enhance the 5GC.

-	UE behaviour: Enhancement at UE side to support the Power saving mechanism.

All of the three solutions seems to have the same target – providing UE the scheduled time, therefore now it is a proper time to start the evaluation. Note that this KI may have the dependencies with other SID (e.g., Redcap Rel-18), therefore it is proposed to add an EN to further enhance the solution if needed.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.5	Coexistence with existing power saving mechanisms for capability-limited devices

The following principles are applied for normative work to enable on demand multicast MBS session:

-	If needed, the UE shall be able to be awake from deep sleep to receive the data of the MBS session.

-	If the NG-RAN node knows the UE is within a certain MBS session, the NG-RAN node will not configure the long eDRX parameter for the UE upon transiting the UE into CM-CONNECTED with RRC-Inactive state. The UE shall disable the MICO mode, after it successfully joins the MBS session.

-	The UE may be configured by the AF with specific times to perform MBS procedures, and wakes up from deep-sleep if needed at those times. The UE may also receive MBS service announcements and/or MBS broadcast delivery at those times.

-	The UE can determine to not go to deep sleep during the remainder of the current MBS data transfer.

-	For data transmission at times known when the service announcement is performed (e.g. periodic times), the AF shall be able to inform UE about the scheduled time of the MBS service (e.g., start time, end time, repetition period) in the service announcement message.

-	Mechanism of Rel-17 for MBS session procedures shall be used as the basis.

-	For data transmissions at times not known when the service announcement is performed, the AF uses a deferred activation request for an MBS session with a deferral time longer than a wakeup period of the UEs. This information is propagated through MB-SMF and SMFs to AMFs serving UEs within the MBS session. When the UEs wake up, they obtain a notification of the data transmission time from the AMF.

# * * * * End of changes * * * *