3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206086r03

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion of KI#5 of FS_5MBS_Ph2

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: Provides the conclusion of KI#5.

# 1. Introduction

S2-2205388 (FS_5MBS_Ph2 Status Report) lists the work plan for the future meetings as follows:

-	SA2#152, Aug (2TUs): Solutions, evaluations, conclusions;

-	SA2#153, Oct (1TU): evaluations, conclusions: Approval of MBS_Ph2 WID;

-	SA2#154, Nov (0.5 TU): conclusions: Adjustment/issues depends on RAN progress.

SA2#152E meeting would be the first meeting for evaluation/conclusion, and the last meeting for having new solutions.

KI#5 is about Coexistence with existing power saving mechanisms for capability-limited devices, and the description is excerpted from TR 23.700-47 as follows:

## 5.5 Key Issue #5: Coexistence with existing power saving mechanisms for capability-limited devices ### 5.5.1 Description Capability-limited devices may use power-saving mechanisms to extend their battery live. Existing power saving mechanisms include MICO (Mobile Initiated Connection Only) mode, DRX (Discontinuous Reception), eDRX (Extended Discontinuous Reception). MBS content should be transmitted to all devices at the same time to save transmission resources. However, the existing power saving mechanisms may prevent devices from receiving MBS content (for instance group messages). This KI will study the following issue: - Whether and how to support MBS content (for instance group message) delivery for capability-limited devices by considering coexistence of existing power saving mechanisms and MBS. NOTE: In SA4, co-existence between power saving mechanism and eMBMS has been studied in TR 26.850 [5]. The study result in TR 26.850 [5] could be taken into consideration during the solution study of this KI.

Totally 3 solutions were proposed in previous meetings and they in general address the following aspects:

-	Way of Inform UE the scheduled time: addressing the way of informing UE when to start to receive the MBS session data.

-	Granularity of the scheduled time: In the solutions, the granularity could be the DRX-level time information, or SDP level, which could be in seconds.

-	5GC enhancement: Whether the solution needs to enhance the 5GC.

-	UE behaviour: Enhancement at UE side to support the Power saving mechanism.

All of the three solutions seem to have the same target – providing UE the scheduled time, therefore now it is a proper time to start the evaluation. Note that this KI may have the dependencies with other SID (e.g., Redcap Rel-18), therefore it is proposed to add an EN to further enhance the solution if needed.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.5	Coexistence with existing power saving mechanisms for capability-limited devices

The following principles are applied for normative work to allow capability-limited device to receive  multicast/broadcast MBS data:

-	When the UE needs to be awake due to MBS coinciding with the UE already being in connected mode due to other reasons, the UE follows normal connected mode procedures.

-	When the UE needs to be awake due to MBS coinciding with the UE already being in idle mode and reachable (e.g. in active time in MICO or PTW for eDRX) the UE follows normal idle mode procedure.

-	When the UE needs to be awake due to MBS coinciding with the UE being in idle mode and in deep sleep, i.e. unreachable for paging to the network, the UE leaves the deep sleep state only to perform procedures related to MBS service.

-	If the MBS service does not require the UE to transition to connected mode, i.e. the UE receives MBS service in broadcast mode and therefore can be in idle mode, then the UE does not update the AMF to become reachable for paging. The UE would therefore still be considered unreachable for paging in the AMF. This minimizes the signalling between the UE and the network.

-	If the MBS service requires the UE to transition to connected mode (e.g. reception in MBS multicast mode) then the UE performs regular procedures for MBS multicast mode defined in TS 23.247 [4]. This would therefore make the UE become reachable in the network for other unicast services.

-	When the UE is in the middle of an MBS data transfer, and the UE is scheduled to move to deep sleep due to power saving, e.g. end of PTW for extended idle mode DRX, expiration of active time for MICO or the UE transitioning from CM-CONNECTED to CM-IDLE in the case of MICO with no active time, then the UE does not go to deep sleep during the remainder of the current MBS data transfer.

NOTE 1:	If at the end of the current MBMS data transfer, the UE knows there is another MBMS data transfer scheduled soon, in that case depending of the time between MBS data transfers, the UE can decide to go to sleep between MBS data transfers.

There are two possible ways the UE can be notified of an upcoming MBS broadcast session start:

-	If MBS User Services defined in TS 26.502 [11] is used, the UE needs to receive MBS service announcement while awake (i.e. while in connected mode, or while idle mode during PTW for extended idle mode DRX, or active time for MICO). The UE wakes up if not already awake for MBS service reception based on the schedule received in the service announcement. For this option, the MBS service announcement may be provided via MBS broadcast service announcement or via any of the possible unicast service announcement delivery mechanisms. In case the MBS service announcement is provided in application layer, similar mechanisms need to be provided.

NOTE 2:	In order to allow all UEs using power saving function to receive the service announcement in time to be able to receive the MBS broadcast data delivery, the application server needs to be aware of the maximum unreachable period of the UEs.

-	The UE may be configured by the application server with specific times to perform MBS procedures, and wakes up from deep-sleep if needed at those times. The UE may also receive MBS service announcements and/or MBMS broadcast delivery at those times (if needed).

NOTE 3:	The configuration (e.g. TMGI, start time) is out of scope of 3GPP and assumed to be performed between application server and UE at application layer. The application server needs to initiate MBS bearer service procedures during those time intervals.

-

# * * * * End of changes * * * *