3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206095r01

Elbonia, August 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	Evaluation for KI#4

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Evaluation for the candidate solutions mapping for KI#4captured in the TR 23.700-05 V0.3.0.

1. Discussion

This contribution proposes the evaluation for the candidate solutions mapping for KI#4captured in the TR 23.700-05 V0.3.0.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V0.3.0.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.A4 Evaluation for KI#4

High level aspects of the solutions for KI#4:

- Solution#6 allows the roaming of MBSR and provides means by which the IAB-UE is configured with PLMN ID list, geographic areas and OAM server FQDN. The roaming operation of IAB-UE is authorized by the 5GC based on the Access and Mobility Subscription data. The feasibility to support the roaming of IAB-DU using IAB node integration and inter-IAB-donor gNB mobility procedure needs RAN WGs evaluation.

- Solution#10 reconfigures the PLMN ID in which the MBSR to be allowed operate as a IAB node in this PLMN and allows the IAB-UE to obtain the OAM server address and the security information required to access the OAM from the 5GC during the registration. However, the solution does not specify how the 5GC determines the OAM server address and the security information. In addition, the security aspects need SA3 evaluation.

# * * * * End of changes * * * *