SA WG2 Meeting #152e	S2-2206103

# August 17th – 26th, 2022; Elbonia          	  		 	   (revision of S2-220)

Title:	[draft] LS on 3GPP 5G System acting as a Detnet Node

Response to:

Release:	Rel-18

Work Item:	FS_DetNet

Source:	Nokia, Will be SA2

To:	IETF "Deterministic Networking (detnet)" WG

Cc:

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	SP-211633.zip

1. Overall Description:

3GPP SA2 is working on extensions to the 5G System Time Sensitive Communications (TSC) framework for the support of IETF’s DetNet deterministic networking framework in a Study Item titled “Extensions to the TSC Framework to support DetNet” as documented in SP-211633.

With respect to IETF DetNet specifications, the study makes the following assumptions:

-	The study considers 5GS (5G system) acting as a DetNet node in the DetNet domain.

-	The study only considers the IP case, MPLS is not in the scope.

-	The study considers the DetNet forwarding sub-layer related functions that are applicable to the 5GS. For the IP case according to RFC 8939 clause 1, no service sub-layer function needs to be defined.

The 5GS DetNet Node has interfaces towards the fixed network (N6 interface in 5GS) and on each User Equipment (UE). It is able to forward IP packets between any of its interfaces. It is connected to the DetNet Controller for centralized configuration. The 5GS DetNet Node exposes information about its interfaces and neighbour nodes to the Controller. The 5GS DetNet Node receivs DetNet configuration including flow identification and QoS requirements from the Controller.

The 3GPP SA2 would like to ask IETF DetNet WG guidance on the following points:

- DetNet YANG data model currently only includes end-to-end QoS requirements for the DetNet flow such as max-latency and max-loss. On the other hand, the 5G DetNet Node needs to be aware of the max-latency and max-loss requirements specific to the Node.

1. Is there any work chartered in the IETF to determine the Node specific traffic requirements?

2. One approach considered in 3GPP has been to extend the DetNet YANG data model in such a way that it can contain the Node specific requirements as well. Would this approach be very specific to 3GPP and 5GS DetNet Node in which case it would be appropriate for the 3GPP to standardize the required YANG model extension, or would this be of more general interest for IETF DetNet deployments so that the standardization should take place in the IETF? Where possible 3GPP prefers a generic approach defined by IETF.

- 3GPP is considering to specify to the possibility for reporting the 5GS node, interface and neighbor information to DetNet controller to help build up the network topology information. The reported information may include the DetNet node ID, the list of interfaces (including their IP address and prefix), and when applicable, the list of neighbouring nodes. The reporting can be based on RFC 8343 and RFC 8344.

Would it be appropriate for the 3GPP to standardize the reporting of the information to the DetNet controller this way? Is there any other information that is required by DetNet to be reported to the DetNet controller?

Please Note that 3GPP is going to do the normative work in Q1 2023 about 5G System Time Sensitive Communications (TSC) framework for the support of IETF’s DetNet and thus would need IETF guidance about whether IETF plans to work on this aspect in 2023.

2. Actions:

To IETF "Deterministic Networking (detnet)" WG:

ACTION: 	SA2 kindly asks IETF "Deterministic Networking (detnet)" WG to take the above information into account and to provide any feedback.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#153e | October, 10-14th | Elbonia | Any (e-meeting) |
| SA2#154 | November, 14-18th | To be determined | To be determined |