3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206112r02

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung, Huawei

Title:	Conclusion for FS_GMEC KI#4

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Conclusion for KI#4.

# 1. Introduction/Discussion

It is proposed to update the conclusion for KI#4 based on evaluations of solutions for KI#4.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

## 8.4	Key Issue #4: Multiple SMFs for VN group communication

The following are way forward principles.

- The multiple SMFs serving the same 5G VN group can belong to a single SMF Set or different SMF Sets.

- .

- In ETSUN case, local PSA UPF can support local switching as instructed by A-SMF via the I-SMF

- N6 interface between UPFs controlled by different SMFs can also be used for traffic forwarding using existing/implementation mechanism.

# * * * * End of changes * * * *