3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206112r07

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung, Huawei, CATT, ZTE, CAICT, China Mobile, China Southern Power Grid

Title:	Conclusion for FS_GMEC KI#4

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Conclusion for KI#4.

1. Introduction/Discussion

It is proposed to update the conclusion for KI#4 based on evaluations of solutions for KI#4.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

8.4	Key Issue #4: Multiple SMFs for VN group communication

The following are way forward principles.

-	The multiple SMFs serving the same 5G VN group can belong to a single SMF Set or different SMF Sets

-	AF/NEF can set the Single-SMF indicator for a 5G VN group to indicate the SMF whether the procedures used for multiple SMFs is applicable for this 5G VN group and this Single-SMF indicator is stored as part of the group data in group subscription data, or

-	Single-SMF indicator can be pre-configured for the VN group in 5GC according operator’s configuration.

-	If N19-based forwarding is used between UPFs controlled by different SMFs

-	GSMF (NRF) is used for discovery and registration of SMFs serving a 5G VN group

-	N19 tunnels between UPFs controlled by different SMFs are set up via SMF interactions. The SMF interactions can occur via Nx interface (between SMFs) or via GSMF (NRF).

-	The topology of UPFs controlled by different SMFs is determined based on network policies, e.g. Relay-SMF/UPF, where the network policies are determined by GSMF and indicate to the involved SMFs.

NOTE:	Whether to use a standalone NF or enhance existing NF (e.g. NRF) to support the above functions will be determined during normative phase.

-	If N6-based forwarding is used between UPFs controlled by different SMFs

-	N6 interface between UPFs controlled by different SMFs can also be used for traffic forwarding using existing/implementation mechanism

-	If I-UPF and local PSA UPF is inserted for a PDU Session targeting to a 5G VN group serving by a single SMF Set

-	local PSA UPF can support local switching as instructed by A-SMF via the I-SMF

# * * * * End of changes * * * *