3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206113

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Evaluation for FS_5TRS_URLLC KI#5

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing evaluations of solutions for KI#5.

# 1. Introduction/Discussion

There are 4 solutions (Sol#9, Sol#10, Sol#11, Sol#20) for KI#5.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change * * * *

### 7.x	Key Issue #5: Interworking with TSN network deployed in the transport network

There are 4 solutions (Sol#9, Sol#10, Sol#11, Sol#20) for KI#5. The solution #10 has been merged into Solution #9, therefore it is not considered here. The following technical issues are studied:

a) The architecture enhancement to support the interworking between 5GS and TSN networks deployed in the transport network.

-	NG-RAN/NG-TT and UPF/TNW-TT support LLDP to report the topology to CNC in TSN Transport Network. It supports the container to communicate with TSNCF. TSNCF collects the Talker/Listener stream requirement as specified in IEEE Std 802.1Qcc [6] from NG-TT and TNW-TT via PMIC/UMIC. It provides the Talker/Listener status to CNC and receives the status of stream configuration from CNC. It provides the Talker/Listener configuration status to NG-TT and TNW-TT via PMIC/UMIC. (Sol#11)

-	SMF allows information access with the collocated TN CUC to support UNI as described in IEEE P802.1Qdj [10]. SMF determines the traffic requirements for a QoS Flow and initiates that CUC translates them to merged stream requirements which are then passed to the CNC in TSN Transport Network. (Sol#9)

-	NG-RAN and UPF may support Listener/Talker and Stream Transformation as described in IEEE P802.1Qdj [10]. If the Stream Transformation in NG-RAN and UPF is not supported, two options for identifying the traffic on QoS Flow basis in the TN: 1) SMF instructs UPF and RAN to assign a separate CN tunnel end point address for each QFI or 2) the interface between the CUC and CNC in the TN allows the SMF and CUC to indicate the TEID and QFI of the given QoS Flow to the CNC in the TN (Sol#9).

- 	SMF may transfer received merged end station communication-configuration from the TN CNC to the Talker/Listener accordingly. (Sol#9)

-	NG-RAN and UPF may support the functionality of Listener/Talker as described above. (Sol#9)

-	NG-RAN provides stream requirements to transport network CNC, and allocates separate tunnel end-point destination addresses per QoS Flow (for QoS flows with TSCAI) used for the N3 tunnels (existing functionality). (Sol#20)

-	SMF instructs the NG-RAN and UPF to allocate separate tunnel end-point destination addresses per QoS Flow (for QoS flows with TSCAI) used for the N3 tunnels using existing NGAP and existing N4 signalling. (Sol#20)

-	NG-RAN provides stream identification to transport network CNC. (Sol#20)

b) What information are needed and how to collect the information from 5GS (e.g. NG-RAN, 5GC NF), so that the 5GS can interact with TSN network. Also, determine which 5GS entity is responsible to provide it to the TSN network deployed in the transport network.

-	TSNCF collects the Talker/Listener stream requirement as specified in IEEE Std 802.1Qcc [6] from NG-TT and TNW-TT via PMIC/UMIC. It provides the Talker/Listener status to CNC and receives the status of stream configuration from CNC. It provides the Talker/Listener configuration status to NG-TT and TNW-TT via PMIC/UMIC. (Sol#11)

-	SMF allows information access with the collocated CUC to support UNI as described in IEEE P802.1Qdj [10]. SMF determines the traffic requirements for a QoS Flow and initiates that CUC translates them to merged stream requirements which are then passed to the CNC in Transport Network. (Sol#9)

-	NG-RAN provides stream requirements to transport network CNC, and allocates separate tunnel end-point destination addresses per QoS Flow (for QoS flows with TSCAI) used for the N3 tunnels (existing functionality). (Sol#20)

-	NG-RAN may determine a dynamic value of 5G-AN PDB in UL direction at the gNB egress. The NG-RAN provides the dynamic value of 5G-AN PDB value to the SMF in a response to the QoS Flow establishment or modification request. This is used as more accurate value for AN-PDB when the SMF determines the traffic requirements for a QoS Flow towards the TN CNC (Sol#9).

# * * * * End of changes * * * *