3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206124r02

# Elbonia, August 17 – 26, 2022	(revision of S2- 220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0540 | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Nadrf_DataManagement_RetrievalRequest service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the Question 1 of the LS IN from CT3 (i.e. S2-2205410/C3-223295), the current procedures of Analytics Exposure via DCCF and Data Collection via DCCF only use the Nadrf_DataManagement_RetrievalSubscribe service operation but not use the Nadrf_DataManagement_RetrievalRequest service operation. The advantage of using DCCF is to avoid duplicate data collection, especially in a Pub/Sub manner. Since Nadrf_DataManagement_RetrievalRequest service is used for one time data retrieval, there is no advantage to use DCCF here. Furthermore, Nadrf_DataManagement_RetrievalSubscribe can also achieve the task of one time data retrieval In addition, it is unclear what’s the "unsolicited" scenario means in clause 10.2.5 for the description of the Nadrf_DataManagement_RetrievalRequest service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the procedures in clause 5B.1, 6.2.6.3.3, 10.2.5 to clarify Nadrf_DataManagement_RetrievalRequest service operation is for one time data retrieval from the ADRF which could be related to a Nadrf_DataManagement_StorageRequest operation or a Fetch Instructions from ADRF. 2. Clarify the "unsolicited" scenario for the usage of the Nadrf_DataManagement_RetrievalRequest service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The differences between Nadrf_DataManagement_RetrievalSubscribe service and Nadrf_DataManagement_RetrievalRequest service are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5B.1, 6.2.6.3.3, 10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5B.1	General

The ADRF offers services that enable a consumer to store and retrieve data and analytics. The analytics are produced by the NWDAF as described in clause 6.1, and data are collected as described in clause 6.2.

Data may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics, providing ADRF Notification Address (+Notification Correlation ID). Analytics or Data are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_DataManagement Nnwdaf_DataManagement or Nmfaf_3caDataManagement service).

Data may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a Storage Transaction Identifier or a a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_DataManagement_RetrievalSubscribe request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. If the time window includes the future and the ADRF has subscribed to receive the data or analytics, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the data or analytics and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_DataManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the data or analytics or provide instructions to the endpoint to fetch the data or analytics using an Nadrf_DataManagement_RetrievalRequest.

Data may be deleted from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_Delete request.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.2.3. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6.

# * * * * Next change * * * *

##### 6.2.6.3.3	Historical Data Collection via DCCF

The procedure depicted in figure 6.2.6.3.3-1 is used by data consumers (e.g. NWDAF) to obtain historical data, i.e. data related to past time period. The data consumer requests data using Ndccf_DataManagement_Subscribe service operation. Whether the data consumer uses this procedure or directly contacts the ADRF or NWDAF is based on configuration.

Figure 6.2.6.3.3-1: Historical Data Collection via DCCF

1.	The data consumer requests data via DCCF by invoking the Ndccf_DataManagement_Subscribe (Service_Operation, Data Specification, Time Window, Formatting Instructions, Processing Instructions, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) service operation as specified in clause 8.2.2. The data consumer may specify one or more notification endpoints to receive the data. If data to be collected is subject to user consent: if the data consumer checked user consent, the data consumer shall provide user consent check information (i.e. an indication that it has checked user consent), otherwise, the data consumer shall provide a purpose for the data collection.

"Service_Operation" is the service operation used to acquire the data from a data source. "Data Specification" provides Service_Operation-specific parameters (e.g. event IDs, UE-ID(s)) used to retrieve the data. "Time Window" specifies a past time period and comprises a start and stop time. "Formatting and Processing Instructions" are as defined in clause 5A4. The data consumer may optionally include the ADRF or NWDAF instance (or ADRF Set or NWDAF Set) ID where the stored data resides.

2.	The DCCF checks if data is to be collected for a user, i.e. SUPI or GPSI, then, depending on local policy and regulations, the DCCF checks the user consent by retrieving the user consent information from UDM using Nudm_SDM_Get including data type "User consent" and taking into account purpose for data collection as provided in step 1. If user consent is not granted, DCCF does not subscribe to event exposure for events related to this user, the data collection for this SUPI or GPSI stops here and DCCF sends a response to the data consumer indicating that user consent for data collection was not granted. If the user consent is granted, the DCCF subscribes to UDM to notifications of changes on subscription data type "User consent" for this user using Nudm_SDM_Subscribe.

If the data consumer is NWDAF and it provides user consent check information (i.e. an indication that it has checked user consent) in Ndccf_DataManagement_Subscribe in step 1, which has been obtained by the NWDAF from UDM before, then the DCCF can do data collection for a user based on the user consent information from the NWDAF and skip retrieving it from UDM.

3.	If an ADRF or NWDAF instance or ADRF Set ID or NWDAF Set ID is not provided by the data consumer, the DCCF determines if any ADRF or NWDAF instances might provide the data as described in clause 5B and 5A.2.

NOTE 1:	An ADRF or NWDAF might have previously registered data it is collecting with the DCCF.

4.	(conditional) If the DCCF determines that an ADRF instance might provide the data, or an ADRF instance or Set was supplied by the data consumer, the DCCF sends a request to the ADRF, using Nadrf_DataManagement_RetrievalSubscribe (Data Specification, Notification Target Address=DCCF) service operation as specified in clause 10.2. The ADRF responds to the DCCF with an Nadrf_DataManagement_RetrievalSubscribe response indicating if the ADRF can supply the data. If the data can be provided, the procedure continues with step 5.

5.	(conditional) If the DCCF determines that an NWDAF instance might provide the data or an NWDAF instance or Set was supplied by the data consumer, the DCCF sends a request to the NWDAF using Nnwdaf_DataManagement_Subscribe (Data Specification, Notification Target Address=DCCF) as specified in clause 7.4.2.

6.	The ADRF uses Nadrf_DataManagement_RetrievalNotify or the NWDAF uses Nnwdaf_DataManagement_Notify to send the requested data (e.g. one or more stored notifications archived from a data source) to the DCCF. The data may be sent in one or more notification messages.

7.	The DCCF uses Ndccf_DataManagement_Notify to send data to all notification endpoints indicated in step 1. Notifications are sent to the Notification Target Address(es) using the data consumer Notification Correlation ID(s) received in step 1. Data sent to notification endpoints may be processed and formatted by the DCCF, so they conform to delivery requirements specified by the data consumer.

NOTE 2:	According to Formatting Instructions provided by the data consumer, multiple notifications from an ADRF or NWDAF can be combined in a single Ndccf_DataManagement_Notify so many notifications from the ADRF or NWDAF results in fewer notifications (or one notification) to the data consumer. Alternatively, a Ndccf_DataManagement_Notify can instruct the data notification endpoint to fetch the data from the DCCF before an expiry time.

8.	If a notification contains a fetch instruction, the notification endpoint sends a Ndccf_DataManagement_Fetch request to fetch the data from the DCCF.

9.	The DCCF delivers the data to the notification endpoint.

10.	The UDM may notify the DCCF on changes of user consent at any time after step 2. If user consent is no longer granted for a user for which data has been collected and if there are no other consumers for the data, the DCCF shall unsubscribe to any data collection for that SUPI or GPSI. The DCCF shall further update or terminate affected subscriptions of the Data Consumer. The DCCF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which user consent has been revoked.

11.	When the data consumer no longer wants data to be collected or has received all the data it needs, it invokes Ndccf_DataManagement_Unsubscribe (Subscription Correlation ID) as specified in clause 8.2.3, using the Subscription Correlation Id received in response to its subscription in step 1.

12.	If the data are being provided by an ADRF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the ADRF using Nadrf_DataManagement_RetrievalUnsubscribe as specified in clause 10.2.7.

13.	If the data are being provided by an NWDAF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the NWDAF using Nnwdaf_DataManagement_Unsubscribe as specified in clause 7.4.3.

# * * * * Next change * * * *

### 10.2.5	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics. The Nadrf_DataManagement_RetrievalRequest may be unsolicited (e.g. when the consumer itself has known "Storage Transaction Identifier") or sent in response to a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Inputs, Required: one of the following:

-	Storage Transaction Identifier;

or

-	Fetch Correlation ID(s) if the RetrievalRequest is in response to a Fetch Instruction.

Inputs, Optional: None.

Outputs Required: Result Indication.

Outputs, Optional: Data or Analytics.

# * * * * End of changes * * * *