3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206144r02

Elbonia, Aug 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion on KI#4

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDG_ph2 / Rel-18

Abstract: Proposes evaluation and conclusion of KI#4.

1. Introduction/Discussion

So far, there have been 10 solutions for key issue #4 (solution #14, #15, #16, #17, #18, #19, #34, #35, #36 and #37). The solutions address one or more of the following aspect: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI. This contribution evaluates these solutions from the three aspects.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48 v0.3.0.

# * * * * First change, all new * * * *

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.x	Evaluation for Key Issue #4

Solutions of KI#4 address one or more of the following aspects: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI.

| Solution | UE collection definition | 5GC selects common EAS/DNAI | AF selects common EAS/DNAI |
| --- | --- | --- | --- |
| #14: Group Management | X |  |  |
| #15: Selection of common DNAI |  | X | X |
| #16: Selecting the same EAS/DNAI for collection of UEs | X | X | X |
| #17: Application layer EAS selection for collections of UEs |  |  | X |
| #18: Discovery of the same EAS for collections of UEs | X | X |  |
| #19: Influencing UPF and EAS (re)location for collections of UEs | X |  |  |
| #34: Selecting the same EAS/DNAI for collection of UEs | X | X |  |
| #35: Providing dedicated (re)location information as traffic routing information | X |  | X |
| #36: Providing dedicated (re)location information as EAS Deployment information | X |  | X |
| #37: (Re)location of same EAS and coordination across UEs |  | X |  |

The solutions are analysed with focus on the above three aspects.

Aspect 1: UE collection definition.

Solution #14 performs UE collection definition by group management. It utilizes Nnef_ParameterProvision (or new defined) service for AF to send GPSI list and external group ID. UDM aligns the group data and UE subscription data. UDR generates internal group ID, and stores SUPI list, internal group ID and external group ID.

If Nnef_ParameterProvision service is used, it shall be enhanced to support dynamic UE group management, which only supports 5G VN group or MBS group management in current specification.

Solution #19 performs UE collection definition by group management. AF uses Nnef_ParameterProvision (or new defined) service to send group management information, including group member management parameters and group data:

-  Group member management parameters: GPSI list, external group ID.

-  Group data: DNN/S-NSSAI, Group Type (i.e. ad-hoc group for gaming/platooning/…), EAS ID, Group Attributes (e.g. same EAS/same PSA is needed/…).

NEF translates external group ID into internal group ID, and stores these information in UDM.

Solution #16, #18 and #34 use UE list, group ID or any UE, and Spatial Validity Condition to define the UE collection of specific application, which is identified by FQDN or Application ID.

Solution #35 and #36, the collection of UEs can be identified using explicit group management extending the 5GVN group management which is similar as that described for Solution #14 and #19. The collection of UEs can also be identified with target UEs with service information. The PCF or SMF associates the UE with the collection based on the group ID or the service information via traffic routing information or EDI, and the specific service information for each PDU session received via the procedure of Service specific parameter provisioning.

Aspect 2: 5GC selection of common EAS/DNAI.

Solution #16 proposes that the SMF determines that UE belongs to the dynamic UE group when steps 1-9 of clause 6.2.3.2.2 of TS 23.548 [3] are performed, and the SMF obtains common EAS/DNAI of the UE collection, locally or retrieving from UDR. If the common EAS/DNAI has not been determined for UE collection, the SMF determines the common EAS/DNAI via the Rel-17 EAS discovery procedure, and synchronizes to UDR with the common EAS/DNAI.

Solution #34 EDI is used for maintaining the common EAS/DNAI. SMF synchronizes common EAS/DNAI by updating EDI to NEF/UDR.

Solution #37 UDM/UDR is used for maintaining the common DNAI/EAS as part of group information. The difference is, when common EAS/DNAI is synchronized, the UDM/UDR shall send notification to the SMF(s) that subscribed the group data (see solution #19), including the common EAS/DNAI. Solution #37 only applies to the case of pre-defined group.

Solution #15 introduces a new network function SCMF to manage the common DNAI. SMF may obtain the common DNAI either when PDU Session is established (pre-invoke of SCMF) or when reported DNS Query by EASDF (dynamic invoke of SCMF). The SMF sends EASDF information to build ECS option or L-DNS server address according to the common DNAI. Then rest of procedure in clause 6.2.3.2.2 of TS 23.548 [3] are performed. Solution #15 only deals with common DNAI case.

Solution #18 assumes that the UE group is served by a single SMF. When reported DNS Query, the SMF respond to the EASDF with either the common EAS, or the same information to build ECS option or L-DNS server address as other member(s) of the dynamic UE group.

Aspect 3: AF selection of common EAS/DNAI.

Solution #15 proposes both 5GC and AF selection ways, and only AF selection procedure is evaluated in this aspect.

SMF selects UPF according to common DNAI obtained from SCMF, and notifies the common DNAI to AF as target DNAI via UP path change event. Then AF sends “group FQDN” corresponding to the common DNAI to UE via application layer. UE solves common EAS via DNS. If common DNAI is updated, SMF gets notified by SCMF, and above is repeated.

Solution #17 proposes that AF determines members of the dynamic UE group. For each group member UE, AF sends request to the SMF for candidate DNAI(s) via PCF. SMF notifies AF the candidate DNAI(s). AF selects common DNAI and common EAS according to the candidate DNAI(s) from all the SMF(s). Then AF sends the common DNAI to each SMF via AF influence traffic procedure, and sends the address of the common EAS to each UE via application layer.

Solution #35 and #36 propose solution to support the case that AF determines common EAS/DNAI for certain application for dynamic UE group. The common EAS/DNAI is sent to the SMF via pcc rule or EAS Deployment Information, and the SMF use the common DNAI/EAS for the routing decisions for traffic of PDU Session and the DNS handling rule decision. In Solution #35, the common EAS/DNAI for the collection of UEs is received via traffic routing information, and the PCF associates the UE accessing the application with the collection of UEs using the same DNAI/EAS, and the PCF send the pcc rule with the common EAS/DNAI to the SMF. In Solution #36, the common EAS/DNAI for the collection of UEs is received via EAS Deployment Information, and the SMF associates the UE accessing the application with the collection of UEs using the same DNAI/EAS.

# * * * * Second change, all new * * * *

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.x	Conclusions of Key Issue #4

KI#4 shall be concluded by the three aspects: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI.

UE collection definition.

UE list, or group ID or any UE and Spatial Validity Condition, or Target UEs with service information could be used for UE collection definition of specific application, which is identified by FQDN or Application ID.

For group ID case, the dynamic group management could be used. For group management, all the related solutions need enhancement to existing NEF service or definition of new service. Nnef_ParameterProvision service is used for 5G VN/MBS UE group management, and enhanced for dynamic UE group management is reasonable.

For this aspect, it is proposed that:

-	Group management could be used for UE collection definition.

-	UDR performs management of group ID and corresponding UE ID list is the base mechanism for group management.

-	UE list, or group ID or any UE with Spatial Validity Condition could be used for UE collection definition of specific application, which is identified by FQDN or Application ID.

-	Targeting UEs information with service information can be used to dynamically identify the collection of UEs.

5GC selection of common EAS/DNAI.

All the solutions above are based on Rel-17 EAS discovery procedure, and use a 5GC NF to manage the common EAS/DNAI. This is the consensus of the solutions for 5GC selection of common EAS/DNAI.

For this aspect, it is proposed that:

-	5GC selection of the common EAS/DNAI for UE collection shall be supported.

-	UDM/UDR is used for maintaining the common EAS/DNAI for coordination in case of multiple SMFs.

-	Add the parameter indication of EAS/DNAI correlation in Traffic Influence (defined in clause 5.6.7 of TS 23.501 [x]) to indicate the UE collection assessing the same EAS/DNAI.

-	The SMF identifies the collection of UEs and decides the common DNAI/EAS for them.

-	The SMF determines to configure the same DNS handling rules (for example, the same DNS server) to resolve the FQDN in DNS query from collection of UEs or directly responses the common EAS IP address to DNS query from collection of UEs, and configures the DNS message handling rules to EASDF correspondingly EAS/DNAI for collection of UEs.

AF selection of common EAS/DNAI.

As an option, AF could determine the common EAS/DNAI and provide the common EAS/DNAI to 5GS.

For this aspect, it is proposed that:

-	AF selection of the common EAS/DNAI for UE collection shall be supported.

-	AF determined DNAI/EAS for UE collection shall be sent to the 5GC for the routing decisions for traffic of PDU Session and the DNS handling rule decision.

# * * * * End of changes * * * *