3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206231r15

Elbonia, August 17 –26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, vivo?, InterDigital Inc?, Nokia?, Tecent, Lenovo, MediaTek Inc?, Xiaomi, ZTE?, Meta USA, Google Inc., KDDI

Title:	Update of Sol#52 on EN Resolving and solution supplement

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract: Update of Sol#52 on including EN resolvents and newly agreed solutions/updates from last meeting.

1. Introduction

As a consolidated solution for PDU Set based QoS framework, the Sol#52 has been agreed in SA2#151e. This paper intends to update Sol#52 to capture more newly agreed solutions/updates in last meeting and resolve the ENs.

Editor's note:	PDU Set identification for peer-to-peer scenarios (i.e. between two UEs routed via the 5G CN) is FFS.

Observation#1: The peer-to-peer communication scenarios can be split into the uplink communication from UE to the UPF and the downlink communication from the UPF to the UE. The downlink case has been covered by most of the current solutions And for uplink case, there is an Editor’s Note that the PDU Set QoS handling for Uplink media services is FFS.

Proposal#1: It’s proposed to remove this Editor’s Note as the UL cases have been already covered by another existing EN.

Editor's note:	Whether PDU Set Sequence number can also convey Start/End PDU of a PDU Set instance is FFS.

Observation#2: The PDU Set Sequence Number (PSSN) may be unique for each PDU Set (at least at a specific time period). The UPF/RAN may use this unique PSSN to detect whether the arrived PDUs belongs to the same PDU Set. When a PDU with a new PSSN arrives, it means this is the start of a new PDU Set. However, a receiver (e.g. UPF) can only realize a PDU is the last one in the PDU Set until the next PDU with a new PSSN arrives, which will be too late since the last PDU of previous PDU Set has already been transmitted. Therefore, the PSSN cannot also convey the Start/End PDU of a PDU Set.

Proposal#2: It’s proposed to remove this Editor’s Note.

Editor's note:	The definition of the PDU Set Error Rate is FFS.

Observation#3: The definitions of PDU Set Delay Budget and PDU Set Error Rate are proposed as below:

PDU Set Delay Budget: The PDU Set Delay Budget defines an upper bound of the time that a PDU Set may be delayed between UE and the N6 termination point at the UPF. For DL, the PSDB indicates the upper bound of time from the moment when the first PDU of a PDU Set arrives at UPF to the moment when the last PDU of the PDU Set is transmitted to UE. Similarly, for UL, the PSDB shows the upper bound of the time from the moment when the first PDU of the PDU Set is sent by the UE to the moment when last PDU of the PDU Set is delivered to the UPF.

PDU Set Error Rate: The PDU Set Error Rate defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access).

Proposal#3: It’s proposed to remove this Editor’s Note and add definitions of PSDB and PSER.

Editor's note:	Whether the "PDU Set Priority" will be the same for all PDU Sets (i.e. same as existing QoS Flow Priority) or it will be different for each PDU Set (i.e. same as "PDU Set importance") is FFS.

Observation#4: Whether the “PDU Set Priority” will be the same for all PDU Sets or it will be different for each PDU Set depends on whether the PDU Sets with different importance are delivered in one single QoS flow. If PDU Sets with same importance are within a single QoS flow, the PDU Set Priority can be seemed as the newly-introduced PDU Set granularity QoS parameter, which corresponds to the ARP priority level. If the PDU Sets with different importance are delivered over a single QoS flow, the PDU Set Priority is same as the PDU Set importance.

Proposal#4: It’s proposed to remove the Editor’s Note.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

6.52	Solution #52: Consolidated Solution for PDU Set based QoS framework

6.52.1	Key Issue mapping

This solution aims to resolve Key Issue #4, "PDU Set integrated packet handling" and Key Issue # 5, "Differentiated PDU Set Handling".

6.52.2	Description

This solution provides a framework for the PDU Set based QoS handling. The following principles apply:

PDU Set QoS policies/rules/profiles and PDU Set QoS Flow establishment

-	Dynamic PCC rules and non-dynamic PCC rules are supported.

-	PDU Set QoS requirements provisioning by AF supported (for dynamic PCC).

-	PDU Set QoS policies are provided to SMF by PCF for dynamic PCC rules.

-	RAN receives and enforces PDU Set QoS profiles from SMF.

-  UPF receives and enforces N4 rules related with PDU Set from SMF.

PDU Set identification and marking over the user plane

-	PDU Sets are detected in the UPF.

-	Between AS and UPF, multiple PDU Set marking techniques can be supported (see bullet 2 below).

-	PDU Set identification between AS and UPF can be negotiated over Control Plane (via NEF for non-trusted AF), or based on pre-configuration.

Editor's note:	PDU Set identification for peer-to-peer scenarios (i.e. between two UEs routed via the 5G CN) is FFS.

-	UPF marks PDU Sets in GTP-U layer for DL PDU via GTP-U header extension. GTP-U marking is independent from and common to different PDU Set markings between AS and UPF.

The following aspects are included:

In the following list, a set of baseline parameters are identified as part of PDU Set QoS framework. Additionally, a set of potential parameters for further study (as proposed by other solutions) is described.

NOTE 1:	The baseline set of parameters does not exclude additional parameters to be agreed in the future.

1.	UPF identifies the PDUs belong to a PDU Set and the following information for each PDU Set:

Info for intra-PDU Set handling (i.e. KI#4, PDU Set integrated handling).

Baseline parameters:

-	PDU Set Sequence number (SN) (solution 7, 8, 9, 11, 12, 14, 19, 20, 21, 22, 23, 50, 53, 55, 56).

-	Start/End PDU of the PDU Set (solution 11, 12, 15, 18, 21, 22, 55, 56).

-	PDU SN within a PDU Set (solution 11, 20, 22, 55, 56).

Parameters for further study:

-	Number of PDUs within a PDU Set (solution 9, 20, 50).

Editor's note: Among above parameters, which one is mandatory or optional need further discussion.

The QoS Flow is identified using QoS Flow ID and each PDU Set within the QoS Flow is identified using PDU Set SN. Each QoS Flow can be used to deliver one or more PDU Set.

Info for inter-PDU Set handling (i.e. KI#5, PDU Set differentiated handling)

Baseline parameters:

-	PDU Set importance (solution 7, 11, 14, 15, 18, 19, 22, 24)

Parameters for further study:

-	PDU Set dependency (solution 11, 14, 19, 22, 23, 24, 56)

2.	UPF identifies the above information (listed in bullet #1) based on the following mechanism(s):

Options for further study:

Option 1: by matching RTP/SRTP header and payload (RFC 3550/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking).

(solution 7, 12, 14, 15, 16, 17, 18, 23, 24, 54).

Option 2: new 3GPP specific RTP extension header (solution 8).

NOTE: Option 1 and 2 can only work if these headers are not encrypted (e.g., not carried above TLS/QUIC/DTLS layer).

NOTE: Option 2 can be supported after SA4 confirmation. The information carried via new 3GPP specific RTP extension header will be confirmed based on SA4 outputs.

Option 3: by information provided by the AS in N6 header e.g. encapsulation GTP-U header, IP packet fields (solution 7, 9, 17, 18, 22).

Editor’s Note: Option 3 solutions 7, 17 and 18 needs further discussion.

Option 4: by non-standardized UPF implementation mechanism, e.g., detection based on traffic characteristics (solution 7, 12, 17, 18, 20, 24, 25).

Editor's note:	Which above options will be supported is FFS.

Editor's note:	What information can be achieved by each option and more options need further discussion.

3.	UPF provides the above PDU Set related information (listed in bullet #1) to the RAN.

For PDU Set importance:

Options for further study:

Option 1: UPF classifies the DL traffics into different QoS Flows based on PDU Set importance (solution 10, 14, 24, 26).

Option 2: UPF classifies the DL traffics into different sub-QoS Flows based on PDU Set importance (solution 17, 18).

Option 3: UPF adds PDU Set importance into GTP-U header (solution 7, 11, 14, 15,17, 18, 19, 22, 25, 56).

For other PDU Set related info (listed in bullet #1):

UPF adds them into GTP-U Header (solution 7, 11, 14, 15, 19, 22, 23, 53, 54, 56).

4.	New QoS parameters for PDU Set based QoS handling in 5GS:

Baseline parameters:

-	PDU Set Delay Budget (PSDB) (solution 8, 9, 12, 14, 25, 26). The PDU Set Delay Budget defines an upper bound of the time  taken to transmit all the PDUs within a PDU Set. This is determined by the sum of the time taken by the 5GS to deliver all the PDUs within the PDU Set. Time taken to deliver an individual PDU is the amount of time taken between UE and N6 termination point at the UPF.

-	PDU Set Error Rate (PSER) (solution 8, 12, 25, 26). The PDU Set Error Rate defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in UE of a 3GPP access). The PDU Set Error Rate applies to PDU set which needs PDU set integrated handling.

NOTE: A PDU Set is considered as error in case all or partial PDUs of the PDU Set are not successfully delivered. The other PDU Set error cases depends on the consensus of the other PDU Set integrity handing.

Editor's note: PSDB and PSER definitions need further discussion.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25, 49, 51).

Parameters for further study:

-	Whether to drop a  PDU Set in case PSDB is exceeded (solution 8, 22, 24, 25) .

NOTE 2:	The PDU Set Discard Time can be determined by RAN implementation.

-	PDU Set Priority (14, 56).

-	The other cases for PDU Set integrity handing including: 1)the PDU Set is useful only if X% of bits of the PDU Set are delivered correctly as defined in solution# 25; 2) the PDU Set is useful only up to the first bit in error as defined in solution 25; 3) the PDU Set is not useful if the first packet of the PDU Set is lost in LS(S2-2203658).

NOTE 3:	LS(S2-2203658) lists the following examples: 1) In some implementations, all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information applies. 2) In other implementations, as an example if the first packet of the PDU Set is lost, all other packets of the fragmentation units are useless, whereas of the last packet is lost, the decoder can use all packets except the last one. 3) In yet another example, a PDU Set may be mapped to all source and repair packets of an Application Layer FEC source block.

5.	Information from AF:

Baseline QoS parameters for each PDU Set within the QoS Flow:

-	PDU Set handling indication (solution 8, 15, 19, 25, 26, 49) (indicating if PDU Set based handling should be activated to a flow).

NOTE 3:	This indication may be implicitly indicated via other PDU Set related information provided by the AF.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25, 49, 51).

Editor's note:	Whether more detailed indicators are needed for different PDU Set QoS handling features (e.g. PDU Set integrated handling, QoS handling based on PDU Set importance, PDU Set dropping due to delivery failure of dependent PDU Sets) is FFS.

-	PDU Set Delay Budget (solution 8,12, 25, 26).

-	PDU Set Error Rate (solution 8,12, 25, 26).

Baseline parameters on frame identification:

-	Burst periodicity (solution 8, 12, 14, 24, 26, 56).

NOTE 4:	The details of RAN behaviour are up to RAN WG.

NOTE 5:	Whether and how to support the PDU Set handling for Uplink will be studied and led by RAN WG.  SA2 will align with RAN’s progress and decision for Uplink, if any.

NOTE 6:	In this release, the charging offset issue of DL PDU set eligible dropping by the NG-RAN will not be addressed.

6.52.3	Procedures

6.52.3.1	PDU Set based QoS handling

Figure 6.52.3.1-1: High-level Procedure of PDU Set based QoS handling

The PDU Set based QoS control may happens during PDU Session establishment or modification procedures. The process includes the following steps:

1a.	Steps 1-7a of PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502 [3]) are performed. A network slice type for XR service can be used for such a PDU Session.

1b.	The AF may send the information described in above bullet#5 of clause 6.52.2 to PCF via Nnef_AFsessionWithQoS_Create request as defined in clause 4.15.6.6 of TS 23.502 [3]. AF may also provide these information to the 5GS prior to PDU Session Establishment.

2.	The PCF generates appropriate PCC rules, which may include PDU Set related QoS parameters listed in bullet #4. The PCF sends the PCC rules to SMF.

NOTE 1:	This step is done in step 7b in PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502 [3]) or step 1b in PDU Session Modification procedure (defined in clause 4.3.3.2 of TS 23.502 [3]).

If this step is triggered by step 1b, the PCF generates the PCC rules considering the information provided by the AF.

3.	The SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF. SMF sends the N4 rules to UPF and sends the QoS profiles to the RAN node via AMF.

NOTE 2:	This step is done via steps 8-15 in PDU Session Establishment procedure (defined in clause 4.3.2.2.1 of TS 23.502 [3]) or steps 2-7 in PDU Session Modification procedure (defined in clause 4.3.3.2 of TS 23.502 [3]).

NOTE 9: The extensions of N4 rule and QoS profile to support PDU Set related handling are mainly listed in bullet#1,2,3,4. Further N4 rule details to realize the above UPF behaviour will be determined in normative phase.

4. The leftover steps of PDU Session Establishment or Modification procedures are performed.

5.	Based on received N4 rules or locally configuration on the UPF, the UPF identifies the info in above bullet #1 via the methods described in bullet#2 of clause 6.52.2, and performs PDU Set based QoS handling according to N4 rule instruction.

6.	UPF sends the PDU Set info to RAN according to above bullet #3.

7.	Based on the PDU Set related info in GTP-U header, RAN performs PDU Set based QoS handling.

NOTE 3:	The details of RAN behaviour is up to RAN WGs.

6.52.4	Impacts on services, entities and interfaces

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

# * * * * End of changes * * * *