SA WG2 Meeting #152E e-meeting	S2-2206237r01

Elbonia, 17 – 26 Aug, 2022	(revision of S2-22xxxxx)

Source:	China Telecom, Huawei

Title:	23.700-89 Conclusion for Key Issue 1

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract: Conclusion for Key Issue #1 for FS_AMP.

- Introduction

This paper integrates the principles from solutions proposed for KI#1 for FS_AMP in TR23.700-89 for N26-based interworking and gives out a general solution.

2.	Proposal

It is proposed to adopt the following conclusion into 23.700-89.

# * * * * ALL NEW TEXT * * * *

8.1.X	Conclusion Key Issue #1: RFSP Index consistency when UE moves from 5GC to EPC for N26-based interworking

If UE's subscription data indicates the Subscribed RFSP Index as "5G prioritized", the ping-pong issue may be triggered when the UE is directed to EPC by the Authorized RFSP Index as "4G prioritized" from the PCF for a UE at 5GC in without N26-interface interworking, because the serving MME cannot receive an RFSP Index other from HSS, i.e., the subscribed RFSP Index.

The following conclusion are agreed:

- When a UE moves from 5GC to EPC, the MME sets the "RFSP Index in use" value the

- same as the value in the UE context from AMF via N26 for a validity period

- When the validity period expires, the MME re-evaluate the RFSP Index value according to current specification.

- The validity period, i.e., "RFSPinUseExpiryTime" may be set to:

  - A preconfigured value at MME locally, or

  - A value selected by PCF for a UE. When the PCF for a UE provides the Authorized RFSP Index indicating the UE to move to 4G, it may also provide the value of "RFSPinUseExpiryTime". If the AMF selects the RFSP Index in use identical to the authorized RFSP Index, it should send the received "RFSPinUseExpiryTime" to MME in UE context along with the "RFSP Index in use" when N26 interface applied. Otherwise, the "RFSPinUseExpiryTime" value should leave empty in the UE context. If the "RFSPinUseExpiryTime" in the UE context from N26 is not empty, it has the higher precedence than the one locally set in MME.

- When UE handover happens between MMEs before the "RFSPinUseExpiryTime" expires, the remaining value of the "RFSPinUseExpiryTime" should also be handover and the new MME should use the received "RFSPinUseExpiryTime" if not empty in the UE context.

# * * * * End of change * * *