3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206314r01

Elbonia, August 17th – 26th, 2022	(revision of S2-2206314)

Source:	Huawei, HiSilicon, vivo

Title:	KI#2, Sol#9 Update Supporting AR Communication

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes to update solution #9 AR communication in FS_NG_RTC TR.

- Introduction

This paper updates solutions 9 of TR 23.700-87.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.9	Solution #9: Supporting AR communication

6.9.1	Description

According to the definition of TR 26.998 [14], different AR devices have different media processing capabilities. IMS needs to support media transmission and media rendering related to AR communication. This solution defines how IMS can be enhanced to support AR communication.

6.9.1.1	Assumption

This solution has the following assumptions:

-	Due to the flexibility (constant iteration, frequent upgrade required) and diversity (various industries, different experiences) of AR applications, built-in or pre-installed AR applications in AR devices will lead to software version fragmentation and difficult integration of different terminals. Therefore, this solution only considers the unified use of IMS data channel downloading mode for AR applications.

-	For the security of national laws and regulations, 3D models used for AR communication should be provided to the IMS when the AR application connects to IMS and saved in the persistent database. IMS needs to be notified when the 3D model is changed by AR application.

6.9.1.2	Definitions

The following definitions apply to this solution:

3D Model: 3D Model include 3D virtual object model and 3D virtual human avatar model.

AR metadata: Data used for AR media rendering, including user pose information and camera information and so on.

6.9.1.3	Architecture

Figure 6.9.1.3-1: Enhanced IMS architecture to support AR communication

Editor's note:	The DCMF could be an enhanced MRF that support DC, the DC related architecture and interfaces will align with the solution to KI#1.

AR communication architecture is enhanced based on data channel architecture, including the following network functions:

1.	Add a new network function AR Application Server, responsible for AR service control related to AR communication, including AR session media controlling, AR media rendering negotiation and AR service handling.

NOTE:	AR Application Server can be deployed in operator's network or belongs to third party.

2.	Add the AR media processing network function ARMF, responsible for AR communication media transmission and media rendering function, including the following functions:

-	AR Rendering Logic: controls the application-based rendering logic of AR communication.

-	AR Media Processing Function: including Vision Engine, 3D Rendering Engine. Vision Engine and 3D Rendering Engine will establish spatial map, and render the scenes, virtual human models and 3D object models according to the field of view, posture, position, etc. which are transmitted from UE using data channel.

NOTE: 	ARMF is a logic network function element, it can be deployed together with other media functions such as DCMF, it can also be collocated in other functional entities, e.g. UMF (Unified Media Function). The details of UMF is described in solution #15.

The following reference points are used for the IMS AR communication architecture. It is suggested that SBI is applied to these reference points.

-	DC4: Service based reference point between the AR Application Server and the DCSF for AR service handling and AR session media control.

-	DC5: Service based reference point between the MMTEL AS and the ARMF for AR resource management.

Note: The DC4 interface will be defined in solution #X.

The following media interface are used for the IMS AR communication architecture.

-	MDC2: Reference point of data channel media between the AR Application Server and DCMF for AR media rendering negotiation.

-	MDC4: Reference point of AR media between the AR Application Server and ARMF for AR media transmisison.

-	Mb (RTP&DC): The media interface between UE and IMS-AGW, including RTP channel used for audio/video media stream transmission and data channel used for data media stream transmission.

-	Mb (RTP): The media interface between IMS-AGW and ARMF, using RTP channel for audio/video media stream transmission.

-	Mb (HTTP): The media interface between DCMF and ARMF, used by DCMF to transmit data in HTTP channel after transforming data media stream received in data channel to HTTP format.

-	Mb (DC): the media interface between IMS-AGW and DCS-M, using data channel for data media stream transmission.

6.9.1.4 Interface and Operations

DC5 interface should be defined in this solution. DC1, DC2 and DC4 will be defined in solution #3.

AR media resource operations are provided by ARMF shown as follows:

| Service Name | Service Operations | Operation Semantics | IMS consumers |
| --- | --- | --- | --- |
| Numf_ARMF | NFQueryXRResourcePoint | Request/Response | MMTEL AS |
|  | NFAllocateXRResourcePoint | Request/Response | MMTEL AS |
|  | NFReleaseXRResourcePoint | Request/Response | MMTEL AS |

Numf_ARMF_NFQueryXRResourcePoint service operation is used by the MMTEL AS with following parameters:

-	Required Inputs: None

-	Optional Inputs: None

-	Required Outputs: Available Computing Power, Available Parallel Thread

-	Optional Outputs: None

Numf_ARMF_NFAllocateXRResourcePoint service operation is used by the MMTEL AS with following parameters:

-	Required Inputs: Remote Connection Address1, Remote Connection Address2, XR Computing Power

-	Optional Inputs: Remote SCTP Port, SCTP stream ID, Remote SCTP maximum message size, Remote Dcmap, Remote certificate fingerprint

-	Required Outputs: Context, Bearer Termination, Local Connection Address1, Local Connection Address2

-	Optional Outputs: Local URL address

Numf_ARMF_NFReleaseXRResourcePoint service operation is used by the MMTEL AS with following parameters:

-	Required Inputs: Context, Bearer Termination

-	Optional Inputs: None

-  Required Outputs: None

-	Optional Outputs: None

6.9.2	Procedures

According to AR communication scenarios defined by TR 26.998 [14] and TR 26.928 [15], media processing capabilities are different for different AR devices (STAR, EDGAR, and WLAR). The AR communication call flows can be summarized as follows:

1.	Terminal rendering process: When the media processing capability of AR device meets the requirements of AR communication (such as AR mark and simple AR effect) according to its status such as power status, signal status, computing power status, internal storage status, etc, the AR device can independently implement AR media rendering by itself. The either side of IMS provides AR application to the calling and the called users at the same time. The terminal performs local rendering based on the media obtained locally or sent by the peer.

2.	Network rendering process: When the media processing capability of AR device cannot meet AR communication requirements (such as complex scene or virtual human rendering requirements) according to its status, the AR device can decide to request IMS for AR media rendering. The either side of IMS provides AR application to the calling and the called users at the same time, IMS then performs AR media rendering based on AR media received from the calling or the called users, finally IMS sends rendered AR media using normal audio/video streams through RTP channel to the calling or called user.

6.9.2.1	Terminal Rendering Process

Figure 6.9.2.1-1: Terminal rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B.

2.	The bootstrap and application data channel connections are established for the UE-A and UE-B according to AR application requirements.

Scenario 1: The UE performs AR media rendering based on the AR media get from local application.

3.	The UE-A get AR media (such as 3D model) from AR application locally and

performs AR media rendering itself.

4.	The UE-A sends the rendered AR audio/video media to the peer through the RTP channel.

Scenario 2: The UE performs AR media rendering based on the AR media get from remote application.

5. 	The UE-B sends AR media through application data channel to UE-A.

6.	The UE-A performs AR media rendering itself based on the AR media received, and displays the rendered audio/video media on its screen.

6.9.2.2	Network Rendering Process

Figure 6.9.2.2-1: Network rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B. Then the bootstrap data channels are established for the UE-A and UE-B.

2.	The UE-A decide to request network media rendering based on its status such as power status, signal status, computing power status, internal storage status, etc.

Editor's note: 	It is FFS how to provide such information from the UE-A to the AR application.

3.	If the network rendering is needed, the UE-A will establish an application data channel with the DCMF. Meanwhile, if the network rendering should be done based on the UE-B’s AR media, another application data channel between the UE-B and DCMF is also established.

AR Media Rendering Negotiation Procedure:

4. 	The UE sends a Media Rendering Negotiation Request including XR-Session-Attribute parameter to the DCMF through application data channel. The XR-Session-Attribute is defined as follows:

XR-Session-Attribute: network_rendering_content=X,Y,Z…; rendering_type=partial/all; video_resolution=X,Y;video_frame_rate=X

-  network_rendering_content: Represents which AR objects that need to be rendered in IMS for this conversation.

-  rendering_type: Represents the rendering contents which need to be rendered in IMS are partial or all, “partial” means some AR objects will be rendered on the UE, the other will be rendered in IMS, “all” means all the AR objects will be rendered in IMS.

-  video_resolution: Represents the video resolution in this conversation.

-  video_frame_rate: Represents the video frame rate in this conversation.

5. 	The DCMF transfers the Media Rendering Negotiation Request to HTTP POST and sends it to the AR AS.

6. 	The AR AS decides whether to provide media rendering function and what network_rendering_content could be done based on the request from UE in step 4 and its available resource and XR-Session-Attribute.

7.	The AR AS will send a XR Resource Allocation Request to DCSF to confirm whether ARMF has enough resource to finish AR media rendering, carrying AR service type and computing power value.

8. 	The DCSF transfers the request to MMTEL AS.

9. 	The MMTEL AS sends a XR Resource allocation request to ARMF, carrying the UE-A’s application data channel address information, the IMS-AGW(for UE-A) audio/video address information, and the remote UE-B’s address information.

10. When the ARMF resource applied successfully, it returns a successful response carrying the termination addresses information allocated on ARMF to the MMTEL AS.

11-12. The MMTEL AS sends a successful response to AR AS through DCSF.

13. The AR AS stores the media point information of the ARMF.

14. The AR AS sends 200 for HTTP POST to indicate the media rendering negotiation is successful.

15. The DCMF transfer the 200 POST and sends the Media Rendering Negotiation Response to UE-A through application data channel.

The network_rendering_content determines by AR AS is sent to UE-A. And UE-A can request to change the network_rendering_content if the status changed in the UE-A, such as low power, low computing power etc.

AR Session Media re-Negotiation Procedure For Network Rendering:

16. If the XR resource are allocated successfully in step11, the MMTEL AS sends a DC modification request to the DCMF, carrying ARMF address information, to connect DCMF and ARMF termination.

17. The MMTEL AS sends a REINVITE message carrying ARMF address information, to anchor UE-A’s audio/video media stream to ARMF through IMS-AGW.

18. The UE-A sends 200 response carrying UE-A’s audio/video media address information.

19. The MMTEL AS sends a REINVITE message carrying ARMF address information, to connect ARMF to UE-B.

20. The terminating IMS side sends 200 for REINVITE.

Networking Rendering Procedure:

21. The UE-A starts AR media rendering.

22-23. The UE-A sends AR media through application data channel to DCMF and DCMF transfers the AR media to ARMF for AR media rendering.

24. The ARMF performs AR media rendering.

25. The UE-A performs part of the AR media rendering itself.

26-27. The UE-A sends the rendered audio/video media over RTP to ARMF through IM-AGW. If UE-A does not perform the rendering, the UE-A sends the original audio/video media over RTP to ARMF.

28-29. The ARMF combines the AR media rendered by ARMF and the rendered AR media received from UE-A, and sends the complete audio/video media to UE-B. If UE-A does not perform the rendering itself, step 25 can be omitted, and the ARMF directly sends the rendered media to the UE-B.

30. The ARMF sends the rendered video RTP media stream to the UE-A when it needs to send video back to the UE-A through the video back channel.

6.9.3	Impacts on existing nodes and functionality

MMTel AS:

-	Support data channel resource modification, to connect DCMF to a new network node.

-	Support ARMF resource management, and connect ARMF to IMS-AGW and DCMF.

DCMF:

-  Support transforming AR media in data channel to HTTP and send to a new network node.

AR AS:

-  Support AR media rendering negotiation between UE and IMS.

-  Support AR application service handling.

ARMF:

-	Support AR media rendering function for AR communication based on the received AR metadata and audio/video media stream.

UE:

-	Support data channel capability.

-	Decides whether to use the network rendering method and change the split ratio of the rendering tasks based on its status.

# * * * * End of changes * * * *