3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206317r03

Elbonia, August 17th – 26th, 2022	(revision of S2-2206317)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes the conclusion of KI#2 in FS_NG_RTC TR.

- Introduction

This paper propose the conclusion of KI#2

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

8.2 Conclusions Related to Key Issue #2

The following conclusions are agreed for supporting AR telephony communication:

-	Data channel architecture can be used to support AR telephony communication. If the UE needs the network support for media rendering, the IMS will use the architecture and procedures specified in the solution #9 to finish the media rendering. Otherwise, the procedures as specified in Solution #8 are taken as baseline for terminal rendering process.

-	The media rendering of AR telephony communication can be completed by the UE independently, or with the support of the IMS using the rendering negotiation procedure.

Editor Note:	The media negotiation for network rendering between the UE and the network is FFS. SA4 has been asked to include procedures to support media rendering negotiation.

# * * * * End of changes * * * *