3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206317r06

Elbonia, August 17th – 26th, 2022	(revision of S2-2206317)

Source:	Huawei, HiSilicon, ZTE

Title:	KI#2: Conclusion

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes the conclusion of KI#2 in FS_NG_RTC TR.

- Introduction

This paper propose the conclusion of KI#2

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

8.2 Conclusions Related to Key Issue #2

The following conclusions are agreed for supporting AR telephony communication:

-	The data channel architecture is used as baseline to support AR telephony communication. If the UE needs network support for media rendering, the architecture and procedures specified in Solution #9 are used. Otherwise, if the UE can perform the media rendering without network support, the procedures as specified in Solution #8 are taken as baseline for terminal rendering process.

NOTE:	The media negotiation for network supported rendering between the UE and the network will be specified in SA4.

# * * * * End of changes * * * *