SA WG2 Meeting #SA2-152E 	S2-2206342r03

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	Conclusion for KI#9

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes conclusion for KI9.

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

*** Start of 1st change ***

7.X Evaluation on solutions of KI #9

Solution #34, Solution #35, , Solution #61, Solution#x, Solution#y are for Key Issue #9. These solutions propose a mechanism that can be used to trade-off throughput/latency/reliability and power consumption. Each solution relies on the network changing a configuration (i.e. CDRX or QoS) based on the state of the UE (i.e. battery or memory status)

All solutions rely on battery related information being provided to the network. Where the solutions differ is how this information is used. In Solution 34, this information is used at the RAN to select a new CDRX configuration which has already been configured in the UE. In solution 35,61,x,y, this information is used to change the QoS policy level.

In Solution 34 the AF provides XR Communication Pattern(s) to NG-RAN. NG-RAN uses the XR Communication Pattern(s) to derive multiple CDRX configurations. The UE may request to move from one CDRX configuration to another by indicating the selected CDRX configuration to NG-RAN.

In Solution 35 the AF provides service information as well as conditional service requirements to NG-RAN. The conditional service requirements contain one or more sets of QoS reference parameters, and an event associated with each set of QoS reference parameter.  The event, for example, could be "time interval [start, end]", "duration of the service=x", "UE power/memory status", etc. When the event is detected by RAN, the QoS parameters are changed accordingly.

In Solution 61, when UE battery level change event is received, the PCF determines the policy based on the current UE battery level, e.g. a QoS level corresponding to a low or ultra-low battery level is selected to conduct the service.

In Solution X(S2-2206007), UE or AF provides the UE’s power mode to PCF. AF may additionally provide media codec info to PCF. PCF uses the received UE’s power mode/ media codec info for QoS decision.

In Solution Y(S2-2206519), SMF provides alternative QoS profiles to NG-RAN. A UE overheating status associated with each alternative QoS profile, RAN change the QoS parameters according to the overheating status received from UE.

*** End of 1st change ***

*** Start of 2nd change ***

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.X Key Issue #9 Conclusion

The following points are interim Conclusions for KI#9：

1. PCF considers UE power mode , media codec type provided by AF, and generates new policy e.g. new QoS parameters, for the XR/media traffic.

Editor’s Note: Whether and how UE power mode information can be provided to 5GC to help PCF adjust the policy will be decided in future meeting.

# * * * *end of changes * * * *