SA WG2 Meeting # 152e	S2-2206354r03

17th August – 26th August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	MediaTek Inc., Nokia

Title:	KI#4 KI#5: Sol#56: Evaluation and Conclusion (PDU Set packet handling with QoS information)

Agenda Item: 	9.19

Document for:	Discussion and Decision

Work Item / Release:	FS_XRM / Rel-18

Abstract: 	This contribution provides an evaluation and conclusion for Solution #56 “PDU Set integrated packet handling with QoS differentiation”.

1 Introduction

Solution #56 was introduced to address:

-	KI#4: PDU Set integrated packet handling and

-	KI#5: Differentiated PDU Set Handling

This solution describes PDU Set packet handling where the dynamic per-PDU Set QoS policy provided by the SMF is used in combination with a new GTP-U header received in-band to determine the priority of a PDU Set.

This contribution discusses the above.

2 Discussion

This solution recommends using the "Frame Marking RTP Header Extension", an IETF paper that is nearing of approval. The document describes the enhanced RTP header required to transmit video information in-band to middleboxes, switches, or network nodes.

Observation 1: The RTP header extension is used to carry encrypted or unencrypted video data for the standard VP8/9 and H264/265 codec families.

Observation 2: Deep packet inspection of the NAL-U header that is specific to a codec is no longer necessary.

The RTP header extension can describe either scalable or non-scalable (i.e., base layer field is 0) media streams, with both sharing the following fields:

-	1 bit to signal first packet in a frame

-	1 bit to signal last packet in a frame

-	1 bit to signal an independent frame which can be decoded independent of prior frames (e.g., I-frame)

- 1 bit to signal if a frame/slice can be discarded and still a provide decodable media stream (e.g., P-frame)

- 1 bit to signal base layer (0 for non-scalable and 1 for scalable)

Observation 3: The RTP header extensions includes information useful to identify and mark PDU Sets sent over the user-plane.

3 Proposal

It is proposed to adopt the following proposal into TS23.700-60.

pCR 23.700-60

**** FIRST CHANGE (all new text) ****

7.X	KI#4/5: Evaluation

Solution#56 uses the RTP header extension, as described in the IETF "Frame Marking RTP Header Extension" paper, to extract codec-agnostic information. The RTP header extension is used to carry encrypted or unencrypted video data for the standard VP8/9 and H264/265 codec families.

This solution is introduced to support scalable streams, where the RAN using the dynamic per-PDU Set QoS policy handed down from SMF in combination with the data received in-band in a new GTP-U header on N3, containing multiple spatial and temporal layers, is able to determine the relative priority of the PDU Set of downlink.

NOTE: The details of RAN behavior is up to RAN WG.

**** END OF CHANGES ****