SA WG2 Meeting #S2-152E	S2-2206358r126

22 – 26 August 2022, Electronic	(revision of S2-220xxxx)

Source: 	MediaTek Inc.

Title: 	General: Clarification on Location Server UE and peer-to-peer operation

Document for: 	Approval

Agenda item: 	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract of the contribution: This contribution clarifies that a Location Server UE only applies in peer-to-peer operation. It also defines peer-to-peer operation.

1	Proposal

Solution #26 introduced, among others, the notion of a Location Server UE to cater for the case when a network is not available (e.g. out of coverage) thus enabling a UE to host functionality of a Location Server (see §6.26.4) and other UEs to seek assistance from / support a Location Server UE as they would from / with an LMF. The term peer-to-peer operation was introduced to cater for the case of not having network involvement.

The definition of Location Server UE is updated to restrict its use (at least in this Release) to peer-to-peer operation and peer-to-peer operation is defined, as shown in the pCR below.

It should be noted that the term "peer UE" is already used in TR23.700-86.

2	pCR 23.700-86

**** FIRST CHANGE ****

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Ranging: refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

Reference UE: A UE who determines a reference plane and reference direction in the Ranging based service and Sidelink positioning.

NOTE 1:	Reference UE is the same as Observer UE used in TR 22.855 [2].

Target UE: A UE whose distance, direction and/or position is measured comparing to the reference plane, reference direction and/or location of a Reference UE in the Ranging based service and Sidelink positioning.

NOTE 2:	Any UE participating in the Ranging/Sidelink Positioning can be both a Target UE and a Reference UE and can switch roles in the same Ranging/Sidelink Positioning session.

Editor's note:	Concerning Reference UE and Target UE, whether new name will be defined to differentiate with the term Reference UE and Target UE used in eLCS is FFS.

Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement/result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE.

Located UE: A UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

Editor's note:	Change term Network assisted UE and Second UE to Located UE throughout the document.

SL Positioning Server UE: A UE offering location calculation for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported.

Editor's note:	Change term Location Server UE to SL Positioning Server UE throughout the document.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

NOTE X: The  SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the  SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

Editor's note:	Change term Third Party UE to SL Positioning Client UE throughout the document.

Sidelink Positioning: Positioning UE using PC5.

Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal).

Network-assisted Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation.

UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE.

NOTE X:	For UE-only Operation, the communication among UEs are over PC5.

Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

Editor's note:	Definition on the terminology of Ranging and Sidelink positioning will be aligned with RAN WGs, and will be revisited when there's any conclusion in RAN WGs.

**** END OF CHANGES ****