3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206367r01

Elbonia, August 17 – 26, 2022		(revision of S2-220xxxx)

Source:	vivo

Title:	KI#7,Sol#45,Update: EN removal

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This paper proposes to update Sol#45 for removing ENs.

1. Introduction/Discussion

This paper mainly resolves the following Editor’s notes:

| #1 | Editor's note: How to handle emergency service for 5G ProSe Layer-2 UE-to-Network Relay in RRC_CONNECTED state is FFS. |
| --- | --- |
| #2 | Editor's note: Whether a 5G ProSe enabled UEs in limited service state can act as 5G ProSe UE-to-Network Relay is FFS. |
| #3 | Editor's note: It is FFS whether the 5G ProSe layer-2 Relay needs to notify the Relay network of that it is performing emergency services for relaying if it has been in CM-CONNECTED. |
| #4 | Editor's note: Based on what condition that the Remote UE's AMF rejects the remoted UE's request is FFS. |

For Editor's note#1, when the relay UE has been in the RRC_CONNECTED state, it may informs its network for emergency service for relaying to avoid congestion control.

For Editor's note#2, following instructions refereed to TS23.501 for emergency service are still valuable to relay UE:

Per SA1 LS: In case of emergency session via a relay UE, it is assumed that a remote UE is connected to a relay UE that is registered to a PLMN, i.e. the relay UE has a SIM/USIM/ISIM.

So the relay UE in limited service state can not act as 5G ProSe UE-to-Network Relay, and can remove this EN and corresponding descriptions directly.

For Editor's note#3, this EN is similar to EN#1, when the relay UE has been in the connected stated, NG-RAN has no need to provide additional treatment about emergency service to relay UE, so this EN also can be removed directly.

For Editor's note#4, following descriptions state the condition that the Remote UE's AMF rejects the remoted UE's request, which is that the request type of the NAS connection setup and PDU session establishment from remote UE is not emergency type. So this EN can be removed directly.

-	Remote AMF needs to determine the request type of the NAS connection setup and PDU session establishment, if it is not for emergency, the Remote AMF shall reject the Remote UE's request.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

6.45	Solution #45: Solution for support of emergency service for U2N relay

6.45.1	Description

This solution is related to the Key Issue #7 on Support of Emergency for 5G ProSe UE-to-Network Relay.

In this solution, for the UE-to-Network Relay and remote UE that how to identify the emergency service, it is assumed that a list of dedicated Relay service code is to be used to reflect the emergency for UE to Network Relaying which can be called Emergency RSC. The Emergency RSC has same format as the RSC configured in clause 5.1.4 of TS 23.304 [3], it can be pre-configured in the USIM or ME, or provided by the network as the UE ProSe policy.

6.45.2	Principles for support of emergency service for U2N relay

When the Emergency RSC is configured by the network as the ProSe policy, it means the network of the 5G ProSe UE-to-Network Relay or the 5G ProSe Remote UE supports the emergency service.

When the Emergency RSC is included in the UE-to-Network Relay Discovery message including the Model A and Model B discovery, it means the 5G ProSe UE-to-Network Relay provides emergency service for relaying or the 5G ProSe Remote UE requests for emergency service for relaying. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE can recognise the discovery and the subsequent procedure (e.g. relay connection setup or management) which are for emergency service based on the Emergency RSC.

When the Emergency RSC is included in the UE-to-Network Relay Discovery Announcement message (model A discovery) or in the UE-to-Network Relay Discovery Response message (model B discovery), it means the 5G ProSe UE-to-Network Relay would like to provide the emergency service to remote UE and its network supports emergency service from the 5G ProSe Remote UE perspective.

When to perform the 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay as specified in clause 6.5.1 of TS 23.304 [3] for emergency service, the 5G ProSe Layer-3 UE-to-Network Relay sets the PDU session Request Type to "Emergency Request" for relaying.

When to perform the Connection Establishment for 5G ProSe Layer-2 UE-to-Network Relay as specified in clause 6.5.2 of TS 23.304 [3] for emergency service, the 5G ProSe Layer-2 UE-to-Network Relay sets the service request type to "Emergency service" to enter the CM-CONNECTED state. If the 5G ProSe Layer-2 UE-to-Network Relay is already in CM-CONNECTED, it may inform its network of the emergency service for relaying to avoid congestion control.

When the 5G ProSe UE-to-Network Relay determines to relay emergency service for 5G ProSe Remote UE, it is allowed to act as the role of relay and perform the Relay operation (e.g. relay discovery and connection setup) when it is under congestion control as specified in clause 5.12 of TS 23.304 [3] and mobility restriction as specified in clause 5.4.3 of TS 23.304 [3].

The 5G ProSe Layer-2 Remote UE follows the principles of a normal UE for emergency service handling.

It shall ensure that the traffic and the signalling of the 5G ProSe Layer-2 Remote UE are related to the emergency service when the Connection Establishment for 5G ProSe Layer-2 UE-to-Network Relay in clause 6.5.2 of TS 23.304 [3] is used to emergency service, especially when the 5G ProSe Layer-2 UE-to-Network Relay is under congestion control as specified in clause 5.12 of TS 23.304 [3], mobility restriction as specified in clause 5.4.3. Clause 6.45.3 in this solution provides a way to ensure that the traffic and the signalling of the 5G ProSe Layer-2 Remote UE are related to the emergency service, which proposes that the RAN notify the Remote AMF of that the Remote UE is only allowed to perform the emergency service.

6.45.3	Procedure to ensure emergency service via Layer-2 U2N relay

Figure 6.45.2.2-1: Connection Establishment for 5G ProSe Layer-2 UE-to-Network Relay with ensuring emergency service

0~5. Step 0~5 are basically similar to the step 0~5 of clause 6.5.2.2 in TS 23.304 [3] for Connection Establishment of 5G ProSe Layer-2 UE-to-Network Relay. Following may be some differences:

For step 2~3, the Relay discovery, selection and PC5 connection establishment are for emergency services which may associated with an Emergency RSC.

For step 4, the 5G ProSe layer-2 Relay uses the service request with emergency type to enter to the CM-CONNECTED.

6.	NG-RAN informs Remote AMF that the 5G ProSe Remote UE is using the Relay communication and is only allowed to use emergency service.

Editor's note:	The above notification from NG-RAN to AMF needs coordination with RAN groups.

7~9.	Step 7~9 are basically similar to the step 6~8 of clause 6.5.2.2 in TS 23.304 [3] for Connection Establishment of 5G ProSe Layer-2 UE-to-Network Relay. Following may be some differences:

-	Remote AMF needs to determine the request type of the NAS connection setup and PDU session establishment, if it is not for emergency, the Remote AMF shall reject the Remote UE's request.

6.45.4	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

UE(s):

-	Enhancement on the emergency service handling, e.g. the 5G ProSe Relay is allowed to act the role of relay under mobility restriction and congestion control.

RAN:

-	notify Remote AMF of that the Remote UE is performing relay communication with only emergency service allowed.

Remote AMF:

-	determine whether the Remote UE's request is for emergency service, reject the request if it is not.

# * * * * End of changes * * * *