3GPP SA WG2 Meeting #152E	S2-2206432r03

E-meeting, 17-26 Aug 2022

Source:	CATT, Samsung

Title:	KI #5, Sol #39: Update to clarify EAS relocation between operators

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This paper proposes supporting EAS relocation between operators via controlled IP network.

1. Discussion

It is proposed to update the solution to support EAS relocation between operators via controlled IP network.

Since this solution is to address KI#1 and KI#5, it should clarify that EAS relocation from VPLMN to HPLMN or from HPLMN to VPLMN (clause 6.39.3.1 and 6.39.3.2) belong to KI#1, and then the EAS relocation between operators via controlled IP network (6.39.3.3) belong to KI#5.

2. Proposal

It is proposed to agree the following texts in clause 6.39 of TR 23.700-48.

>>>> Start of Changes <<<<

6.39	Solution 39 (KI#1, KI#5): Support EAS relocation of inter-PLMN

6.39.1	Description

The following solution corresponds to the key issue #1 on Accessing EHE in a VPLMN when roaming as specified in clause 5.1 "how to support the edge relocation in roaming scenarios" and key issue #5 on GSMA OPG impacts and improvements for EHE operated by separate party as specified in clause 5.5 "how the 5GS facilitates edge relocation between an EAS deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers".

This solution makes the following assumptions:

-	the EAS relocation is triggered by AF (e.g. due to the load balance between EAS instances in the EHE);

-	an OP to deploy EAS on another OP, and these OPs are different PLMNs;

-	the EAS is relocated from VPLMN/HPLMN to HPLMN/VPLMN.

The solution is to address EAS relocation of different PLMNs, and it is illustrated as the scenario of figure 6.39.2.1-1. The EHE1 is operated by OP1 in PLMN1. In the EHE1, there is a EAS1 deployed by OP1 and a EAS4 deployed by OP2. The EHE2 is operated by OP2 in PLMN2. In the EHE2, there is a EAS2 deployed by OP1 and a EAS3 deployed by OP2.

Figure 6.39.1-1: Scenario of EAS relocation from VPLMN to HPLMN

Figure 6.39.1-2: Scenario of EAS relocation from HPLMN to VPLMN

Figure 6.39.1-3: Scenario of EAS relocation between operators via controlled IP network

6.39.2	Procedures

6.39.2.1	EAS relocation from VPLMN to HPLMN (KI #1)

Figure 6.39.2.1-1: EAS relocation from VPLMN to HPLMN

0.	When UE is roaming to VPLMN, it establishes a LBO PDU Session to access the source EAS (EAS2), which is deployed by OP1 in VPLMN (EHE of OP2), e.g. OP1 and OP2 have a federation agreement.

1.	When the EAS relocation is triggered, AF determines the source EAS should be relocated to the target EAS (e.g. EAS 1) deployed by OP1 in HPLMN (e.g. based on a federation agreement). The AF invokes Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update service operation to send the request message to NEF, or invokes Npcf_PolicyAuthorization_Create or Npcf_PolicyAuthorization_Update to send the request message to the PCF directly. The request message includes the PLMN ID, which indicates the EAS should be relocated to the target PLMN (identified by PLMN ID).

If 5GC supports EAS IP replacement mechanism, the EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is sent to the V-SMF as specified in clause 6.3.3 of TS 23.548 [3]. Alternatively, the AF sends the EAS information (e.g. EAS IP address) to network and to UE via application layer.

2.	The PCF creates and sends the PCC rule to the V-SMF by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule includes the PLMN ID.

3.	V-SMF discovers the H-SMF.

If the V-SMF has knowledge of EAS deployment information in HPLMN, the V-SMF determines the target DNAI based on the EAS deployment information and IP address of target EAS. V-SMF sends the target DNAI to the V-AMF by invoking Nsmf_PDUSession_SMContextStatusNotify service operation. Then, V-AMF selects H-SMF based on the target DNAI. Then, H-SMF sends Nsmf_PDUSession_Context Request to V-SMF.

Alternatively, the V-SMF sends discovery request to vNRF by invoking Nnrf_NFDiscovery_Request (serving PLMN ID, home PLMN ID), and discovers the H-SMF as specified in clause 4.17.5 in TS 23.502 [9].

4.	The V-SMF inserts or changes the UL-CL/BP V-UPF. The V-SMF sends an inter-PLMN relocation indicator to H-SMF by Nsmf_PDUSession_Update Request. The inter-PLMN relocation is to indicate the EAS is relocated from VPLMN to HPLMN.

5.	The H-SMF selects the H-PSA UPF based on the EAS IP replacement information/EAS information. H-SMF configures the H-PSA UPF for traffic routing towards target EAS. If the EAS IP replacement information is received, the H-SMF configures H-PSA UPF with EAS IP replacement information.

6.	The H-SMF sends the acknowledgement message to the V-SMF, which includes the tunnel information towards H-PSA UPF.

7.	When V-SMF receives the acknowledgement, V-SMF configures the UL-CL/BP V-UPF for traffic routing towards H-PSA UPF if the destination address is the IP address of source EAS or target EAS.

6.39.2.2	EAS relocation from HPLMN to VPLMN (KI #1)

Figure 6.39.2.2-1: EAS relocation from HPLMN to VPLMN

0.	The UE establishes PDU Session to access the source EAS (EAS1).

1.	When the EAS relocation is triggered, AF determines the source EAS should be relocated to the target EAS (e.g. EAS 2) deployed by OP1 in VPLMN (e.g. based on a federation agreement). The AF invokes Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update service operation to send the request message to NEF, or invokes Npcf_PolicyAuthorization_Create or Npcf_PolicyAuthorization_Update to send the request message to the PCF directly. The request message includes the PLMN ID, which indicates the EAS should be relocated to the target PLMN (identified by PLMN ID).

If 5GC supports EAS IP replacement mechanism, the EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is sent to the V-SMF as specified in clause 6.3.3 of TS 23.548 [3]. Alternatively, the AF sends the EAS information (e.g. EAS IP address) to network and to UE via application layer.

2.	If the PCF authorizes the traffic routing in the VPLMN is allowed, the PCF creates and sends the PCC rule to the V-SMF by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule includes the PLMN ID.

3.	H-SMF discovers the V-SMF.

If the H-SMF has knowledge of EAS deployment information in VPLMN, the H-SMF determines the target DNAI based on the EAS deployment information and IP address of target EAS. H-SMF sends the target DNAI to the H-AMF by invoking Nsmf_PDUSession_SMContextStatusNotify service operation. Then, H-AMF selects V-SMF based on the target DNAI. Then, V-SMF sends Nsmf_PDUSession_Context Request to H-SMF.

Alternatively, the H-SMF sends discovery request to hNRF by invoking Nnrf_NFDiscovery_Request (home PLMN ID, serving PLMN ID), and discovers the V-SMF as specified in clause 4.17.5 in TS 23.502 [9].

4.	The H-SMF inserts or changes the UL-CL/BP H-UPF. The H-SMF sends an inter-PLMN relocation indicator to V-SMF by Nsmf_PDUSession_Update Request. The inter-PLMN relocation indicator is to indicate the EAS is relocated from HPLMN to VPLMN. Optionally, the H-SMF may check with H-UDM whether UE is authorized to access VPLMN.

5.	The V-SMF selects the V-PSA UPF based on the EAS IP replacement information/EAS information. V-SMF configures the V-PSA UPF for traffic routing towards target EAS. If the EAS IP replacement information is received, the V-SMF configures V-PSA UPF with EAS IP replacement information.

6.	The V-SMF sends the acknowledgement message to the H-SMF, which includes the tunnel information towards V-PSA UPF.

7.	When H-SMF receives the acknowledgement, H-SMF configures the UL-CL/BP H-UPF for traffic routing towards V-PSA UPF if the destination address is the IP address of source EAS or target EAS

6.39.2.3	EAS relocation between operators via controlled IP network (KI #5)

Figure 6.39.2.3-1: EAS relocation between operators via controlled IP network

0.	The UE establishes PDU Session to access the source EAS (EAS1) in PLMN1.

1.	When the EAS relocation is triggered, AF determines the source EAS should be relocated to the target EAS (e.g. EAS 2 as shown in figure 6.39.1-3) deployed by OP2 in target PLMN2 (e.g. based on a federation agreement). The AF invokes Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update service operation to send the request message to NEF, or invokes Npcf_PolicyAuthorization_Create or Npcf_PolicyAuthorization_Update to send the request message to the PCF directly. The request message includes the PLMN ID, which can be used as an inter-PLMN relocation indicator and indicates the EAS should be relocated to the target PLMN (identified by PLMN ID).

If 5GC supports EAS IP replacement mechanism, the EAS IP replacement information (i.e. source EAS IP address and port number, target EAS IP address and port number) is sent to the V-SMF as specified in clause 6.3.3 of TS 23.548 [3]. Alternatively, the AF sends the EAS information (e.g. EAS IP address) to network and to UE via application layer.

2.	If the PCF authorizes the traffic routing in the PLMN1 (serving PLMN) is allowed, the PCF creates and sends the PCC rule to the SMF by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule includes the PLMN ID.

3.	Based on the received PLMN ID, the SMF can differentiate the EAS deployment information of different PLMNs and determine the target DNAI, and then SMF inserts or changes the PSA UPF that supports target DNAI to access target EAS via controlled IP network. The SMF configures the PSA UPF for traffic routing towards target EAS. If the EAS IP replacement information is received, the SMF configures UPF with EAS IP replacement information.

4.	UE accesses the EAS deployed in PLMN2 via PSA UPF using controlled IP network.

6.39.3	Impacts on services, entities and interfaces

1. EAS relocation between VPLMN and HPLMN (clause 6.39.2.1 and 6.39.2.2):

AF:

-	determines the target EAS and sends corresponding PLMN ID.

V-SMF:

-	determines target DNAI based on EAS information and EAS deployment information;

-	sends an inter-PLMN relocation indicator to H-SMF;

-	configures the UL-CL/BP V-UPF for traffic routing towards H-PSA UPF if the destination address is the IP address of source EAS or target EAS.

H-SMF:

-	determines target DNAI based on EAS information and EAS deployment information;

-	sends an inter-PLMN relocation indicator to V-SMF;

-	configures the UL-CL/BP H-UPF for traffic routing towards V-PSA UPF if the destination address is the IP address of source EAS or target EAS.

PCF:

-	authorizes the traffic routing in the target PLMN is allowed;

-	creates the PCC rule, which includes the PLMN ID.

H-AMF:

- H-AMF selects V-SMF based on the target DNAI received from H-SMF.

2. EAS relocation between operators via controlled IP network (clause 6.39.2.3):

AF:

-	determines the target EAS and sends corresponding PLMN ID.

PCF:

-	authorizes the traffic routing in the target PLMN is allowed;

-	creates the PCC rule, which includes the PLMN ID.

>>>> Next Changes <<<<

6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 22 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 26 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 29 | X |  |  |  |  |  |  |
| 04: Support EAS (re-)discovery in VPLMN via HR PDU Session | 32 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU Session | 38 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 40 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 42 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 43 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 45 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 47 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 49 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 52 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 56 |  | X |  |  |  |  |  |
| 14: Group Management | 58 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 62 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 70 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 73 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 75 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 79 |  |  |  | X |  |  |  |
| 20: Global EASDF | 81 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 83 |  |  |  |  | X |  |  |
| 22: EAS discovery for federated OPs | 84 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 90 |  |  |  |  | X |  |  |
| 24: Reuse Option D after UL-CL insertion | 92 | X |  |  |  |  |  |  |
| 25: EAS discovery in VPLMN via V-EASDF for a HR PDU Session | 94 | X |  |  |  |  |  |  |
| 26: SM Policy for HR Session Breakout in VPLMN | 97 | X |  |  |  |  |  |  |
| 27: EAS discovery with dynamic setup of a LBO PDU Session | 100 | X |  |  |  |  |  |  |
| 28: Support edge computing in Roaming | 102 | X |  |  |  |  |  |  |
| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |  |  |  |  |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |  |  |  |  |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |  |  |  |  |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |  |  |  |  |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |  |  |  |  |
| 34: Selecting the same EAS/DNAI for collection of UEs | 116 |  |  |  | X |  |  |  |
| 35: Providing dedicated (re)location information as traffic routing information | 119 |  |  |  | X |  |  |  |
| 36: Providing dedicated (re)location information as EAS Deployment information | 123 |  |  |  | X |  |  |  |
| 37: (Re)location of same EAS and coordination across UEs | 126 |  |  |  | X |  |  |  |
| 38: EAS Discovery for EHE shared with other PLMN | 128 |  |  |  |  | X |  |  |
| 39: Support EAS relocation of inter-PLMN | 129 | X |  |  |  | X |  |  |
| 40: EAS discovery for shared EHE | 133 |  |  |  |  | X |  |  |
| 41: Controlling non-3GPP access of EC traffic via URSP and ATSSS | 136 |  |  |  |  |  | X |  |
| 42: Network-guided EC traffic switching | 139 |  |  |  |  |  | X |  |
| 43: Network-based solution for keeping EC traffic on 3GPP Access | 141 |  |  |  |  |  | X |  |
| 44: EAS traffic switching avoidance | 144 |  |  |  |  |  | X |  |
| 45: Application selected PDU Session | 146 | X |  |  |  |  | X |  |
| 46: Avoid UE switching on-going EC traffic away from 3GPP access | 147 |  |  |  |  |  | X |  |
| 47: Avoiding Switch Away Based on an SMF Indication | 149 |  |  |  |  |  | X |  |
| 48: Avoiding Switch Away Based on an Indication in the URSP | 150 |  |  |  |  |  | X |  |
| 49: URSP based solution to avoid UE to switch away from Edge PDU Session | 151 |  |  |  |  |  | X |  |
| 50: Obtain and maintain mapping table between IP address/IP range with DNAI | 152 |  |  |  |  |  |  | X |
| 51: EDI holding the IP address to DNAI mapping | 154 |  |  |  |  |  |  | X |
| 52: AF obtaining target DNAI provided by NEF | 155 |  |  |  |  |  |  | X |

>>>> End of Changes <<<<