3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206486r02

Elbonia, August 17th – 26th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	Interim conclusions on KI#6

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes evaluation update and interim conclusions on KI#6.

1. Introduction

This paper proposes evaluation update and interim conclusions on KI#6.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

## 8.6	Conclusion on Key Issue #6

The following principles are concluded for Key Issue #6 "Policy and parameters provisioning for PIN":

- The PIN policy and parameter(s) are configured in the PEGC via application layer. The parameters include:

a) 	IP address allocation information for allocating IP address to PINE,

b)	PIN connection parameters for a PIN, e.g., SSID, BT ID, password,

c)	PIN discovery parameters for a PIN.

2)	 void

a)	void

b)	void

c)	void

d)	void

e)	void

f)	void

3)	The policy and parameters provisioned to PEGC by 5GC for PIN communication include the following information:

a)	URSP policy where the application and traffic are mapped to DNN, Slice, etc. The traffic related to PIN may be identified by Application ID or traffic filters.

b)	void

c)	void

d)	void

e)	The QoS flow mapping for PINE’s traffic relay is received via existing procedure from PCF,.

f) Non-3GPP QoS assistance information.

4)	The existing procedure used by the SMF to provide the UPF with, PDR, FAR, etc are applicable without modification.

a) void

b)	void .

c)	Framed Route support will be further considered during normative work.

5)	void

7) 	void

Editor’s note: It is FFS whether the parameters will originate in the UDM/UDR, PCF, or a new network/application function.

Editor’s note: What procedure (i.e., Registration, UE Configuration Update, or a new procedure) is used to send the parameters to the UE is FFS.

Editor’s note: whether needs AF or 5GC NF for PIN is dependent on final conclusion of KI#1

# * * * * End of changes * * * *