SA WG2 Meeting #152E	S2-2206521r01

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	China Telecom

Title:	KI#6, Sol#16: Update to remove EN

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes to update solution #16 by removing an EN.

# Discussion

In solution #16: NWDAF assisted URSP determination, there is an editor’s note as following:

Editor's note:	Whether and how to collect data from 4G access and non-3GPP access is FFS.

For 5GC non-3GPP access, as the architecture described in clause 4.2.8.2 in TS 23.501[2], NWDAF can collect Access Type from SMF and other Network Data from 5GC NF under non-3GPP access, e.g. S-NSSAI, Location, DNN, etc. For 5GC 3GPP access, in addition to AMF, Access Type can also be collected from SMF.

This pCR is proposed to collect Access Type from SMF and resolve the editor’s note by clarifying only 5G access is considered and how to collect data non-3GPP access.

# Proposal

It is proposed to update the following solution in TR 23.700-81.

# * * * * First change * * * *

#### 6.16.1.1	Input Data

Based on input data of Observed Service Experience related network data Analytics as described in clause 6.4.2 of TS 23.288 [5], table 6.16.1.1-1 provides enhanced input data by adding PDU Session related information and table 6.16.1.1-2 provides enhanced input data access type.

Table 6.16.1.1-1: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). If UE IDs are not provided as Target of Analytics Reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| PDU Session type | SMF | Type of the PDU Session. |
| SSC Mode | SMF | SSC Mode selected for the PDU Session. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

Table 6.16.1.1-1: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). |
| Access Type | SMF | The Access type the UE camps on.(NOTE) |
| RAT Type | SMF | The RAT type the UE camps on. |
| NOTE : Only 5G access is considered. NWDAF collects input data when UE is under non-3GPP access same as it does when UE is under 3GPP access. |  |  |

# * * * * End of changes * * * *