3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-226535r01

Elbonia, 17 - 26 August, 2022

Source:	Huawei, HiSilicon

Title:	KI#1, Evaluation and conclusion of KI#1

Document for:	Discussion/Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This document provide the evaluation and conclusion for KI#1.

# 1. Introduction

There are 12 candidate solutions proposed to address key issue#1, i.e. solution#1/#3/#4/#5/#6/#18/#19/#20/#21/ #22/#23/#26. This document provides the evaluation and conclusion for KI#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# * * * * Next change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	MBS session reception in RRC Inactive

Editor's note:	The details will be coordinated/checked with RAN WGs.

8.1.1	Interim conclusions

The following conclusions is proposed for KI#1:

-	NG-RAN nodes take the responsibility to determine the UEs within an MBS multicast session is able to receive MBS session data while in CM-CONNECTED with RRC Inactive state. The 5GC shall provide sufficient information to the NG-RAN nodes (i.e., assistance information for RRC Inactive multicast MBS data reception).

- It is possible to keep some UEs within the same MBS session in RRC_CONNECTED and some in RRC_INACTIVE based on different UE priority information, associated MBS session to be established.

- The SMF supports to provide the UE session priority information on whether the UE is suitable to receive the MBS session data in RRC inactive state as the "MBS assistance information for RRC Inactive" to the NG-RAN node.

- The MB-SMF supports to provide the MBS session information on whether it is suitable to receive the MBS session data in RRC inactive state and the session priority information to NG-RAN node.

- What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and is also used for MBS session assistance information for RRC Inactive by the RAN.

- The used delivery mode for an MBS session and the RRC states RRC_inactive or RRC_connected of UEs remains transparent to the CN.

-	When the MBS session is activated, the UE in RRC Inactive state should be able to keep in RRC Inactive state for receiving the MBS session data.

- For CN initiated group paging, the paging information includes which MBS session is to be activated and whether the MBS session is allowed to be received in RRC-inactive state.

- The NG-RAN node shall not release MBS context for an MBS session, if there is any RRC Inactive UE has previously joined and use this NG-RAN node as anchor NG-RAN node.

- When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but RRC resume failure and not receive the MBS data at the new cell, the UE activates the associated PDU session via the registration procedure, i.e. include the associated PDU session information activated status in Registration Request. So that the shared tunnel can be established (if not already established).

- During the hand over procedure, the SMF includes the "MBS assistance information for RRC Inactive" in N2 SM Info and sent to NG-RAN via AMF.

The following requirement, which need be supported by RAN WG, is concluded:

- RAN WG2 define UE radio capability for MBS reception in RRC_INACTIVE state.

- Based on the assistance information provided by 5GC, NG-RAN nodes decide which MBS sessions and/or UEs to keep in RRC_CONNECTED vs. RRC_INACTIVE. How NG-RAN decides which UEs to keep to RRC_INACTIVE or RRC_CONNECTED is up to NG-RAN implementation.

- When the MBS session is activated, the UE in RRC Inactive state should be able to keep in RRC Inactive state for receiving the MBS session data.

- For RAN initiated group paging, the RRC-inactive state UE should be able to determine whether it can keep in RRC inactive state to receive the multicast data reception.

- When the UE is in RRC Inactive state and moves within the RNA, it should be able to continue receiving DL multicast MBS data.

# * * * * End of changes * * * *