3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-226535r01

Elbonia, 17 - 26 August, 2022

Source:	Huawei, HiSilicon

Title:	KI#1, Evaluation and conclusion of KI#1

Document for:	Discussion/Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This document provide the evaluation and conclusion for KI#1.

# 1. Introduction

There are 12 candidate solutions proposed to address key issue#1, i.e. solution#1/#3/#4/#5/#6/#18/#19/#20/#21/ #22/#23/#26. This document provides the evaluation and conclusion for KI#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# * * * * Next change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	MBS session reception in RRC Inactive

Editor's note:	The details will be coordinated/checked with RAN WGs.

8.1.1	Interim conclusions

The following conclusions is proposed for KI#1:

-	The 5GC provides the following assistance information to NG-RAN to assist the NG-RAN on deciding multicast MBS session delivery to the UE(s) in RRC Inactive state:

-	Whether the transmission to UEs in RRC Inactive state is allowed for specific multicast MBS service(s).

-	UE session priority.

-	MBS session priority.

-	UE capability and preference for multicast reception in RRC Inactive state.

-	To provide the above assistance information:

- The SMF supports to provide the UE session priority information on whether the UE is suitable to receive the MBS session data in RRC inactive state to the NG-RAN node.The UE session priority is stored in the UDM a part of MBS subscription data.

- The MB-SMF supports to provide the MBS session information on whether it is suitable to receive the MBS session data in RRC inactive state and the session priority information to NG-RAN node.

- The AF may decide to enable or disable the transmission to UEs in RRC Inactive state for specific multicast MBS service(s) based on the analytics information from the NWDAF.

- During the hand over procedure, the SMF includes the "MBS assistance information for RRC Inactive" in N2 SM Info and sent to NG-RAN via AMF.

- What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and is also used for MBS session assistance information for RRC Inactive by the RAN.

-	When the MBS session is activated, the UE in RRC Inactive state should be able to stay in RRC Inactive state for receiving the MBS session data.

- For CN initiated group paging, the paging information includes which MBS session is to be activated.

- When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but RRC resume failure and not receive the MBS data at the new cell, the UE includes the information of the associated PDU session and multicast MBS session via the registration procedure, i.e. include the associated PDU session  status (i.e. established/active) and multicast MBS session status (join or leave) in Registration Request, so that the shared tunnel can be established (if not already established).

- The UE can initiate MBS session join or leave during Mobility Registration Update procedure by including an MBS session container in the Registration Request message.

- During the hand over procedure, the SMF includes the "MBS assistance information for RRC Inactive" in N2 SM Info and sent to NG-RAN via AMF.

The following requirements need confirmation or decision by RAN WG:

- NG-RAN nodes take the responsibility to determine whether to switch the UE(s) in an multicast MBS session from receiving multicast data in RRC Connected state to receiving multicast data in CM-CONNECTED with RRC Inactive state, or vice versa.

- It is possible to keep some UEs within the same MBS session in RRC_CONNECTED and some in RRC_INACTIVE for multicast data reception.

- UE radio capability for MBS reception in RRC_INACTIVE state is to be defined.

- Based on the assistance information provided by 5GC, NG-RAN nodes decide which multicast MBS session(s) to be delivered to UE(s) in RRC Inactive state, and/or which UEs to receive the multicast MBS session(s) in RRC_CONNECTED or RRC_INACTIVE state. How NG-RAN makes these decisions is up to NG-RAN implementation.

- When the MBS session is activated, the UE in RRC Inactive state should be able to keep in RRC Inactive state for receiving the MBS session data.

- For RAN initiated group paging, paging cause (e.g. for MBS session activation or release) can be included in the paging message and sent to the UE. The UE in RRC Inactive state should be able to stay in RRC inactive state to receive the multicast data reception.

- When the UE is in RRC Inactive state and moves within the RNA, it should be able to continue receiving DL multicast MBS data.

# * * * * End of changes * * * *