3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-226535r01

Elbonia, 17 - 26 August, 2022

Source:	Huawei, HiSilicon

Title:	KI#1, Evaluation and conclusion of KI#1

Document for:	Discussion/Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This document provide the evaluation and conclusion for KI#1.

# 1. Introduction

There are 12 candidate solutions proposed to address key issue#1, i.e. solution#1/#3/#4/#5/#6/#18/#19/#20/#21/ #22/#23/#26. This document provides the evaluation and conclusion for KI#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# * * * * Next change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	MBS session reception in RRC Inactive

Editor's note:	The details will be coordinated/checked with RAN WGs.

8.1.1	Interim conclusions

The following conclusions is proposed for KI#1:

-	NG-RAN nodes take the responsibility to determine the UEs within an MBS multicast session is able to receive MBS session data while in CM-CONNECTED with RRC Inactive state. The 5GC shall provide sufficient information to the NG-RAN nodes (i.e., assistance information for RRC Inactive multicast MBS data reception) if the UE is deemed to be able to receive MBS session data in CM-CONNECTED with RRC Inactive state.

NOTE 1:	NG-RAN nodes may keep UE in CM-CONNECTED state as Rel-17 if none assistance information for RRC Inactive multicast MBS data reception provided.

- Depending on RAN WG feedback, it is possible to keep some UEs within the same MBS session in RRC_CONNECTED and some in RRC_INACTIVE based on different UE session priority information, associated MBS session information to be established.

- NG-RAN use the MBS session information and UE session priority information to decide which MBS sessions and/or UEs to keep in RRC_CONNECTED vs. RRC_INACTIVE. How NG-RAN decides which UEs to keep to RRC_INACTIVE or RRC_CONNECTED is up to NG-RAN implementation.

Editor’s NOTE: it is FFS whether the UE session priority information is the QoS parameter of the associated QoS Flow (e.g. PER, ARP, etc).

NOTE 2:	It is not expected the UE with ongoing non-MBS data transmission can be switch to CM-CONNECTED with RRC Inactive state

- What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and is also used for MBS session assistance information for RRC Inactive by the RAN.

- The used delivery mode for an MBS session and the RRC states RRC_inactive or RRC_connected of UEs remains transparent to the CN.

-	When the MBS session is activated, the UE in RRC Inactive state should be able to keep in RRC Inactive state for receiving the MBS session data.

- For CN initiated group paging, the network notifies which MBS session is to be activated and whether the MBS session is allowed to be received in RRC-inactive state

- The method NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs.

- The NG-RAN node shall not release MBS context for an MBS session, if there is any RRC Inactive UE has previously joined and use this NG-RAN node as anchor NG-RAN node.

- When the UE moves outside the RNA, the UE performs RRC Resume to the target RAN node as per existing procedures in TS 23.501 [2].

- When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but RRC resume fails , it follows existing procedures and transition to CM-IDLE. When the UE transition to CM-IDLE since is not able to receive the MBS multicast data at the new cell, the UE initiate the registration procedure including  the associated PDU session activated status. So that the shared tunnel can be established (if not already established).

- During the hand over procedure, the SMF includes the "MBS assistance information for RRC Inactive" in N2 SM Info and sent to NG-RAN via AMF.

The following requirement, which need be supported by RAN WG, is concluded:

- RAN WG2 define UE radio capability for MBS reception in RRC_INACTIVE state.

- Based on the assistance information provided by 5GC, NG-RAN nodes decide which MBS sessions and/or UEs to keep in RRC_CONNECTED vs. RRC_INACTIVE. How NG-RAN decides which UEs to keep to RRC_INACTIVE or RRC_CONNECTED is up to NG-RAN implementation.

- What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and is also used for MBS session assistance information for RRC Inactive by the RAN.

- When the MBS session is activated, the UE in RRC Inactive state should be able to keep in RRC Inactive state for receiving the MBS session data.

- For RAN initiated group paging, the RRC-inactive state UE should be able to determine whether it can keep in RRC inactive state to receive the multicast data reception per network notification.

- When the UE is in RRC Inactive state and moves within the RNA, it should be able to continue receiving DL multicast MBS data.

# * * * * End of changes * * * *