3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-226535r05

Elbonia, 17 - 26 August, 2022

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell

Title:	KI#1, Interim conclusion of KI#1

Document for:	Discussion/Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This document provide the evaluation and conclusion for KI#1.

# 1. Introduction

There are 12 candidate solutions proposed to address key issue#1, i.e. solution#1/#3/#4/#5/#6/#18/#19/#20/#21/ #22/#23/#26. This document provides the conclusion for KI#1. Evaluation is provided in the revision of S2-2205928.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# * * * * Next change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	MBS session reception in RRC Inactive

Editor's note:	Feedback from RAN WGs is required to conclude KI#1 for the issue which nee RAN input.

8.1.1	Interim conclusions

The following conclusions is proposed for KI#1:

-	Depending on RAN WG feedback, it may be possible to keep some UEs within the same MBS session in RRC_CONNECTED and some in RRC_INACTIVE state. NG-RAN nodes take the responsibility to determine which UE(s) within an MBS multicast session will be moved from CM-CONNECTED with RRC CONNECTED to CM-CONNECTED with RRC Inactive state and still receive MBS session data.

-	The 5GC provides information about the MBS session as specified for Rel-17 and may provide additional assistance information to help NG-RAN to determine whether to apply delivery enabling reception by UEs in RRC_Inactive state for an MBS session and which UE(s) to be moved to RRC Inactive state.

-	The assistance information may include recommendations whether to enable delivery for reception in RRC_Inactive state for an MBS session and information about UEs that should preferably be kept in RRC_Connected state, i.e. the MBS session information and UE session assistance information, and may be provided by the AF to 5GC and then NG-RAN.

Editor’s note: it is FFS how to encode information about UEs that should preferably be kept in RRC_Connected state. What kind of assistance information is needed in the RAN will be confirmed by RAN WGs.

- NG-RAN use the MBS session information and UE session assistance information to help decision on which MBS sessions and/or UEs to keep in RRC_CONNECTED vs. RRC_INACTIVE. How NG-RAN performs those decisions is up to NG-RAN implementation.

NOTE 1:	How the NG-RAN handles the situation without assistance information for RRC Inactive multicast MBS data reception is to be determined by RAN WGs.

NOTE 3: What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and is also used for MBS session assistance information for RRC Inactive by the RAN.

-	When the MBS session is activated, the UEs in RRC Inactive state in cells, where the MBS session is delivered in the delivery mode for RRC-inactive reception, should be able to remain in RRC Inactive state for receiving the MBS session data.

- For group paging, the network notifies which MBS session is to be activated and whether the MBS session is allowed to be received in RRC-inactive state.

- The method how NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs.

- When the UE moves outside the RNA, the UE performs RRC Resume to the target RAN node as per existing procedures in TS 23.501 [2].

- When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but RRC resume fails, it follows existing procedures and transition to CM-IDLE. When the UE transition to CM-IDLE since is not able to receive the MBS multicast data at the new cell, the UE initiate the re-registration procedure and activates the associated PDU session, so that the shared tunnel can be established (if not already established).

- During the handover procedure, the SMF includes the "MBS assistance information for RRC Inactive" if any in N2 SM Info and sent to NG-RAN via AMF.

The following requirements, which need be supported by RAN WG, are concluded:

- Backward compatibility with Rel-17 UEs not supporting the RRC_Inactive reception of MBS multicast data needs to be provided

- RAN WG2 define UE radio capability for MBS reception in RRC_INACTIVE state.

- NG-RAN nodes decide for which MBS sessions to apply delivery enabling reception in RRC_Inactive state. The NG RAN nodes handling RRC-Connected UE in an MBS multicast session also decides whether the UEs can transition to RRC_Inactive state and may consider assistance information from the 5G core network for that decision. How NG-RAN performs those decisions is up to NG-RAN implementation.

- When the MBS session is activated, the UE in RRC Inactive state in cells where the MBS session is delivered in the delivery mode for RRC-inactive reception should be able to remain in RRC Inactive state for receiving the MBS session data.

- If RAN determines to use RAN initiated group paging for that scenario, the RRC-inactive state UE should be able to determine whether it can keep in RRC inactive state to receive the multicast data reception.

- When the UE is in RRC Inactive state and moves within the RNA, it shall be able to continue receiving DL multicast MBS data unless it leaves the MBS service area.

# * * * * End of changes * * * *