3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206548r02

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Samsung, Ericsson?

Title:	KI#1 evaluations update and conclusions

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Principles for solution evaluation and conclusion proposal in terms of KI#1

# 1. Introduction/Discussion

It is proposed for solution evaluation and conclusion proposal for KI#1.

There are two aspects for study in key issue #1: service area or QoS that is applicable to each UE within the group.

For service area applicable to each UE within the group, since 6 out of 7 uses the parameter provisioning service used for 5G VN group management to support provisioning of service area for a 5G VN group, so it is proposed to enhance the parameter provisioning service used for 5G VN group management to support provisioning of service area for a 5G VN group.

4 out of 6 uses the LADN mechanism for enforcement of the service area for a group, and pure 5GC handling of UE's PDU Session targeting to the 5G VN group based on UE location or UE presence in service area of the group doesn’t support UE restricting its behavors to comply with the 5G VN group service area since the UE has no knowledge of information for 5G VN group service area. Hence, it is better to support enforcement at both the 5GC side and UE side.

Considering that a group is uniquely linked to a (DNN, S-NSSAI) combination and the UE can simultaneously belong to multiple groups associated with different (DNN, S-NSSAI) combinations, so if the UE supports LADN per DNN and S-NSSAI, it is proposed that UE applies LADN mechanism for enforcement of the service area for a group, otherwise the UE does not. In both cases, the AMF informs SMF of whether the UE is IN/OUT the 5G VN group service area and the SMF handles PDU Session accordingly.

For QoS applicable to each UE within the group,

-	Sol#8 and sol#21 propose to enhance the UDM/NEF parameter provisioning service to configure QoS information applicable to each UE group member within the group, such QoS information is stored at as part of the 5G VN group data (subscription data type). The PCF can obtain the QoS information as part of the 5G VN group data from UDR and apply it when making policy decisions. Further, sol#21 extends Group-MBR as part of the QoS applicable for a group, the Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group.

-	Sol#9 assumes the group is general group not specific to 5G VN group, and the group is already defined at 5GC. The AF uses the NEF Service Parameter service to configure the QoS information for this group. The NEF stores the authorized QoS information for the group at UDR (application data type). The PCF can obtain the QoS information for a group from UDR and take it into account in policy decisions.

Since the QoS applicable to each UE within the group is managed as group attribute, and to make the management of group data as "subscription data" in an uniformed manner, it is proposed to enhance the parameter provisioning service used for 5G VN group management to support provisioning of QoS for a group (at least for 5G VN group), and the PCF can obtain such QoS from UDR to consider it in policy decisions.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.3.0.

# * * * * First change * * * *

## 7.1	Key Issue #1: Enhance group attribute management

There are two aspects for study in key issue #1: service area or QoS that is applicable to each UE within the group. There are 7 solutions (sol#1, sol#8, sol#9, sol#10, sol#12, sol#13 and sol#18) for service area applicable to each UE within the group, there are 3 solutions (sol#8, sol#9 and sol#21) for QoS applicable to each UE within the group.

The evaluation of key issue #1 separates into two aspects and uses the principles as below table:

Table 7.1-1: Evaluation of KI#1 related principles

| Objectives | Principles | Impacts | Pros/Cons | Solution |
| --- | --- | --- | --- | --- |
| Provisioning of the service area for a group | Enhance the parameter provisioning service used for 5G VN group management to support provisioning of service area for a group. Service area for a group can be stored as part of the group subscription data or each group member's AM subscription data. | NEF, UDM, UDR | Pros: Cons: | sol#1, sol#8, sol#10, sol#12, sol #13, sol #18 |
|  | Enhance the parameter provisioning service used for provisioning of UE related information to support provisioning of service area for a group. Service area for a group can be stored as part of shared data related to the group or each group member's AM subscription data. | NEF, UDM, UDR | Pros: Cons: | sol#9 |
| Enforcement of the service area for a group | AMF obtains service area for a group from UDM/UDR. Apply LADN mechanism with service area for a group. | AMF, UDM | Pros: Cons: | sol#1, sol#8, sol#9, sol#10 |
|  | SMF obtains service area for a group from UDM/UDR, SMF obtains UE location or UE presence in service area of the group and determines how to handle UE's PDU Session. | SMF, UDM | Pros: Cons: | sol#12, sol #18 |
| Provisioning of the QoS for a group | Enhance the parameter provisioning service used for 5G VN group management to support provisioning of QoS for a group. QoS for a group can be stored as part of the group subscription data. | NEF, UDM, UDR | Pros: Cons: | sol#8 |
|  | Enhance the general Parameter Provisioning service to support provisioning of QoS for a group. QoS for a group can be stored at UDR as part of shared data related to the group or each group member's SM subscription data.. | NEF, UDR | Pros: Cons: | sol#9 |
|  | The 5G VN group data is extended to include Group-MBR. Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group. | NEF, UDM, UDR | Pros: Cons: | sol#21 |
| Enforcement of the QoS for a group | The PCF can obtain the QoS information as part of the group subscription data from UDR and apply it when making policy decisions. | PCF. | Pros: Cons: | sol#8 |
|  | The SMF can obtain the QoS information as application data type from UDM as per existing specifications. SMF provides it to PCF as per existing specifications, allowing PCF to apply it when making policy decisions. | SMF/PCF. | Pros: Cons: | sol#9 |
|  | GSMF can obtain Group-MBR from UDM. GSMF updates and stores the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group. PCF can query GSMF to retrieve the group information and make corresponding decisions. | SMF, GSMF, PCF | Pros: Cons: | sol#21 |

Provisioning of QoS and service area:

As described in the table above, there are two basic approaches proposed for how to provision service area and QoS:

1. 	Extend 5GVN API to include service area and QoS (Sol#1, 8)

2. 	Extend the general Parameter Provisioning API to include service area and QoS information (Sol#9)

Either Alternative 1 or 2 can support provisioning of QoS and service area for a group, since Rel-16 stage 2 specifications use the general parameter provisioning service to support management of 5G VN group, but Rel-16 stage 3 specifies the separate PP service API for management 5G VN group. So SA2 can simply conclude that the parameter provisioning service can be used to support provisioning of QoS and service area for a group (including 5G VN group or non-5GVN group), then it is up to stage 3 to decide whether to enhance 5G VN API to support management of different types of group or use separate PP service API or reformulate the 5G VN API in Rel-16 for a unified PP service API for any type of group management in Rel-18.

Provisioning of QoS:

Alternative 1 enhances 5GVN Parameter Provisioning with QoS information. The QoS information will be stored as 5GVN group data in UDR and be provided to PCF. This alternative is thus overlapping with solutions for KI#3 where API solutions for providing QoS for a group of UEs to the PCF are also discussed.

Alternative 2 enhances the general Parameter Provisioning with QoS information. This information will be stored in UDM/UDR as subscribed (default) QoS and provided to SMF. This solution thus complements the solutions discussed as part of KI#3.

Enforcement of service area:

There are three basic solutions proposed for enforcement of service area

a.	Re-use LADN. Limited to one LADN area per DNN, independent of S-NSSAI (Sol#9)

b.	Extend LADN to support a LADN area per DNN/S-NSSAI combination (Sol#1, 8)

c.	Introduce a SMF-based enforcement of Service Area (Sol#12, 18)

Alt b is more flexible than Alt a but can possibly be considered as minor enhancements when re-using existing service area enforcement mechanisms. Alt b also has UE impacts. Alt a has no impact to the UE and can thus work with legacy UEs. It may be possible to support both a Alt a and b to have improved flexibility as well as support legacy UEs in case the UE indicates its support LADN area per DNN/S-NSSAI combination.

Alt c has an issue that the UE is not aware of the Service Area and it needs to be studied how it can ensured that the UE does not repeatedly try to access the DNN/S-NSSAI when it is outside the Service Area. It may also lead to a confusing user experience if the UE is rejected but unaware of when it can retry. The solution is also less efficient than Alt a/b since the SMF needs to subscribe to AMF to be notified when the UE moves in/out of the Service Area using Namf_EventExposure which leads to more signaling than Alt a/b that re-uses existing Nsmf signalling. Alt c has also higher impacts to the 5GS in general.

Enforcement of QoS

No changes to the 5G QoS model are proposed.

# * * * * Second change * * * *

## 8.1	Key Issue #1: Enhance group attribute management

The following principle is proposed for normative work:

-	This key issue shall at least apply for 5G (VN) group.

-	For provisioning of service area applicable to each UE within the group, the UDM/NEF parameter provisioning service is adopted as baseline

-	The service area is provisioned as part of the group subscription data.

-	For enforcement of service area applicable to each UE within the group

-	If the UE supports LADN per DNN and S-NSSAI, the AMF can configure UE with the Group Service Area, otherwise, the AMF does not. If the UE receives group service area per DNN and S-NSSAI, the UE behaves same as defined for LADN as defined in clause 5.6.5 of TS 23.501 [2].

-	When the AMF receives the NAS SM request with the DNN, S-NSSAI associated with the group, the AMF determines UE presence in Group Service Area and forwards it to the SMF. When the SMF receives NAS SM request with the DNN, S-NSSAI associated with the group, the SMF determines UE presence in Group Service Area based on the indication received from the AMF. The SMF shall also subscribe to "UE mobility event notification" for reporting UE presence in Area of Interest by providing DNN, S-NSSAI associated with the group to the AMF as described in clauses 5.6.11 and 5.3.4.4 of TS 23.501 [2].

-	For provisioning of QoS applicable to each UE within the group, the UDM/NEF parameter provisioning service is adopted as baseline

-	The QoS is provisioned as part of the group subscription data.

-	The Group-MBR can be part of the QoS, and the Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group.

-	For enforcement of QoS applicable to each UE within the group,

-	The SMF can obtain such QoS from UDM and send to PCF to consider it in policy decisions.

-	GSMF can obtain Group-MBR from UDM. GSMF updates and stores the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the 5G VN Group. PCF can query GSMF to retrieve such information and make corresponding decisions.

# * * * * End of changes * * * *